| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0113-v2 |
| --- | --- |

| Title: | AHG11: neural network based in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang, Wei Jiang, Xiaozhong Xu, Shan Liu | Tel: Email: | {liqiangwang, vwjiang, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a neural network based in-loop filter is exploited to replace the Deblocking and SAO modules. Compared with EE1 anchor (VTM11+V0056), the proposed method reports {4.73%, 10.73, 11.51%} and {3.54%, 10.99%, 10.24%} BD-rate savings with AI and RA configurations, respectively.

# Introduction

In order to improve the quality of reconstructed image, neural network models are trained for I slice and B slice, separately. In the in-loop filter process, Deblock and SAO are replaced by the proposed NN filter.

# Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively.

Fig. 1 the pre-processing and post-processing units

## Network architecture

Fig. 2 shows the network structure of the proposed NN filter. The first and last layers are regular convolutional layers with kernel size 3. The skip layer is a regular convolutional layer with kernel size 5. The structure of residual block (ResBlock) consists of 3 regular convolutional layers, where N denotes the number of ResBlock. N is set to 8 in this contribution. In the ResBlock, the first layer is a 1x1 convolutional layer followed by a ReLU activation function, the second layer is a 1x1 convolutional layer and the third layer is 3x3 convolutional layer. As for the internal convolutional layers, the number of feature maps is set as 64.

Fig. 2: Network architecture

## Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla V100 x 8 x 16GB) |
|  | Framework: | PyTorch v1.5 |
|  | Number of GPUs per Task |  |
|  | Epoch: | 40 |
|  | Batch size: | 128Kx64 |
|  | Loss function: | L1 30 epochs then L2 10 epochs |
|  | Training time: | 72h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Disable Deblock and SAO |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: |  |
|  | Optimizer: |  |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | PyTorch v1.5 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 596031 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 2.28M/model, 10 model in all |
|  | Multiplay Accumulate (MAC) | 610335744.0 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

## Implementation

- Deblock and SAO are disabled, and the proposed filter is placed before ALF.

- The proposed filter can be turned on/off at the CTU level and slice level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component.

# Results

The anchor is VTM11+V0056[][], and the corresponding CTC results are provided in Table 3 and Table 4.

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT |  |
| Class A1 | -3.02% | -6.43% |  | -7.01% |  | -3.95% |  | -9.49% |  | -8.13% |  | 175% |  | 114554% |  |
| Class A2 | -3.70% | -12.16% |  | -8.83% |  | -3.47% |  | -12.00% |  | -6.48% |  | 170% |  | 97254% |  |
| Class B | -3.44% | -10.95% |  | -10.54% |  | -3.05% |  | -10.52% |  | -9.61% |  | 171% |  | 94209% |  |
| Class C | -3.94% | -13.61% |  | -13.33% |  | -3.03% |  | -10.54% |  | -9.76% |  | 143% |  | 89578% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.54% | -10.99% |  | -10.24% |  | -3.31% |  | -10.61% |  | -8.73% |  | 164% |  | 97275% |  |
| Class D | -5.83% | -15.41% |  | -16.15% |  | -3.27% |  | -11.78% |  | -11.27% |  | 138% |  | 62376% |  |
| Class F | -0.77% | -6.74% |  | -6.74% |  | -0.52% |  | -6.70% |  | -7.19% |  | 227% |  | 28503% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |

Table 4. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -4.15% | -7.35% | -8.85% | -4.78% | -8.84% | -8.97% | 230% | 60980% |
| Class A2 | -4.10% | -10.57% | -9.15% | -4.09% | -10.54% | -7.23% | 168% | 47767% |
| Class B | -4.12% | -10.89% | -11.75% | -3.70% | -11.42% | -11.60% | 162% | 42477% |
| Class C | -5.13% | -14.07% | -14.99% | -4.36% | -13.21% | -12.92% | 128% | 40496% |
| Class E | -6.39% | -9.56% | -11.47% | -5.76% | -8.77% | -11.32% | 166% | 60336% |
| Overall | -4.73% | -10.73% | -11.51% | -4.43% | -10.80% | -10.68% | 165% | 48263% |
| Class D | -5.77% | -14.48% | -16.90% | -4.43% | -14.05% | -15.21% | 126% | 36850% |
| Class F | -2.07% | -8.05% | -8.55% | -2.01% | -8.62% | -8.93% | 130% | 38788% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

The neural network based in-loop filter is presented in this contribution, where Deblock and SAO are both replaced with the NN based filter. Good coding performance can be achieved by applying the proposed method.

# References

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-V2016, 22nd Meeting, by teleconference, 20–28 Apr. 2021.