| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0114-v5 |
| --- | --- |

| Title: | CE-related: High throughput mode for high bit-depth coding – harmonization of CE-3.2 and CE-1.1 – |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

In JVET-W0045 (CE-3.2), CABAC high throughput mode is proposed, where all the syntax elements in residual coding, except for last significant coefficient position in RRC, are fully bypass-coded and CABAC bypass alignment is carried out once after the last significant coefficient position in RRC and at the beginning of a TB in TSRC. It is asserted that for SVT 16-bit sequences this technique improves weighted Delta Binrate by -37.53% for AI, -40.3% for RA and -40.40% for LB with minor coding losses of {0.06%, 0.09%, 0.07%} for AI, {0.32%, 0.36%, 0.35%} for RA and {0.35%, 0.41%, 0.41%}, respectively. However, there is room to further improve CABAC throughput by bypass-coding last significant coefficient position in RRC.

In JVET-W0046 (CE-1.1), a modification to last significant coefficient position is proposed, which codes the relative coordinates of last significant coefficient with reference to the bottom-right corner of a zero-out transform block. It is asserted that for SVT 16-bit sequences this technique improves weighted Delta Binrate by -4.02% for AI, -2.17% for RA and -2.15% for LB with coding gain of {-0.29%, -0.28%, -0.28%} for AI, {-0.28%, -0.26%, -0.26%} for RA and {-0.27%, -0.25%, -0.25%} for LB, respectively. It implies that CE-1.1 is also useful to reduce the number of bins of last significant coefficient position as well as improve coding efficiency.

In order to fully bypass-code all the syntax elements in residual coding, this contribution proposes a harmonization of CE-3.2 and CE-1.1 with bypass-coding of last significant coefficient position, where all the syntax elements in residual coding, including last significant coefficient position in RRC, are fully bypass-coded and CABAC bypass alignment is carried out once at the beginning of TB in both RRC and TSRC. It is suggested to apply the proposed method only in 16-bit high throughput profile.

The simulation results for SVT 16-bit sequences are shown below. The full simulation results are shown in section 3.

BD-bitrate compared to VTM-13.0 (low QP):

- AI:  0.26% (G), 0.17% (B), 0.15% (R), 81% (EncT), 71% (DecT)

- LDB: 0.31% (G), 0.31% (B), 0.31% (R), 90% (EncT), 79% (DecT)

- RA:  0.35% (G), 0.35% (B), 0.35% (R), 92% (EncT), 78% (DecT)

Delta Binrate and Bin-to-Bit ratio compared to VTM-13.0 (low QP):

- AI:  -43.79% (weighted Delta Binrate), -12.91% (peak unweighted Bin2Bit), -43.76 % (peak weighted Bin2Bit)

- LDB: -44.73% (weighted Delta Binrate), -12.84% (peak unweighted Bin2Bit), -43.37 % (peak weighted Bin2Bit)

- RA:  -45.11% (weighted Delta Binrate), -12.55% (peak unweighted Bin2Bit), -42.40% (peak weighted Bin2Bit)

BD-bitrate compared to CE-3.2 (low QP):

- AI:  0.19% (G), 0.08% (B), 0.07% (R), 95% (EncT), 90% (DecT)

- LDB: -0.01% (G), -0.05% (B), -0.04% (R), 99% (EncT), 94% (DecT)

- RA:  0.00% (G), -0.06% (B), -0.06% (R), 100% (EncT), 94% (DecT)

Delta Binrate and Bin-to-Bit ratio compared to CE-3.2 (low QP):

- AI:  -10.02% (weighted Delta Binrate), -2.61% (peak unweighted Bin2Bit), -10.08% (peak weighted Bin2Bit)

- LDB: -7.81% (weighted Delta Binrate), -2.55% (peak unweighted Bin2Bit), -9.88% (peak weighted Bin2Bit)

- RA:  -7.86% (weighted Delta Binrate), -2.46% (peak unweighted Bin2Bit), -9.54% (peak weighted Bin2Bit)

In v2, the results for low QPs were updated.

In v3, estimated throughput performance is included.

In v4, typos are fixed in estimated throughput performance.

In v5, presentation deck is attached.

Introduction

In JVET-W0045 (CE-3.2) [1] , CABAC high throughput mode is proposed, where all the syntax elements in residual coding, except for last significant coefficient position in RRC, are fully bypass-coded and CABAC bypass alignment is carried out once after the last significant coefficient position in RRC and at the beginning of a TB in TSRC. It is asserted that for SVT 16-bit sequences this technique improves weighted Delta Binrate by -37.53% for AI, -40.3% for RA and -40.40% for LB with minor coding losses of {0.06%, 0.09%, 0.07%} for AI, {0.32%, 0.36%, 0.35%} for RA and {0.35%, 0.41%, 0.41%}, respectively. However, there is room to further improve CABAC throughput by bypass-coding last significant coefficient position in RRC.

In JVET-W0046 (CE-1.1) [2] , a modification to last significant coefficient position is proposed, which codes the relative coordinates of last significant coefficient with reference to the bottom-right corner of a zero-out transform block. According to Table 1 below, it is asserted that for SVT 16-bit sequences this technique improves weighted Delta Binrate by -4.02% for AI, -2.17% for RA and -2.15% for LB with coding gain of {-0.29%, -0.28%, -0.28%} for AI, {-0.28%, -0.26%, -0.26%} for RA and {-0.27%, -0.25%, -0.25%} for LB, respectively. It implies that CE-1.1 is also useful to reduce the number of bins of last significant coefficient position as well as improve coding efficiency.

Binrate and Bin-to-Bit ratio of CE-1.1 over VTM-13.0 at low QP

The following measurements for the peak case, for the average case and for the transform coefficients coding part are calculated based on the method in JVET-N0049 [4] and JVET-V0150 [5] :

- Weighted BD-binrate

- {Weighted, Unweighted} bin to bit ratio

- Weighted Delta Binrate, which is measured by using Bjøntegaard Delta-rate on bitrate vs binrate

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1).

Table 1 shows BD-bitrate, BD-binrate, Delta Binrate and Bin-to-Bit ratio of CE-1.1 compared to VTM-13.0 at low QP for SVT 16-bit sequences. Please refer to the attached spreadsheet for PQ/HLG/SVT 12-bit sequences.

Table 1: Coding performance of CE-1.1 over VTM-13.0 at low QP for SVT 16-bit sequences

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.29% | -0.28% | -0.28% | -4.19% | -4.19% | -4.19% | -4.02% | 1.16 | 1.14 | -1.59% | 0.47 | 0.45 | -4.04% | 99% | 97% |
| SVT12 | -0.33% | -0.29% | -0.29% | -4.33% | -4.31% | -4.31% | -4.15% | 1.17 | 1.14 | -2.13% | 0.58 | 0.55 | -4.12% | 96% | 99% |
| Overall | -0.31% | -0.28% | -0.29% | -4.26% | -4.25% | -4.25% | -4.09% | 1.16 | 1.14 | -1.86% | 0.52 | 0.50 | -4.08% | 97% | 98% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.28% | -0.26% | -0.26% | -2.34% | -2.32% | -2.32% | -2.17% | 1.16 | 1.14 | -1.54% | 0.47 | 0.45 | -3.94% | 107% | 101% |
| SVT12 | -0.29% | -0.26% | -0.26% | -2.82% | -2.79% | -2.80% | -2.68% | 1.17 | 1.14 | -2.11% | 0.57 | 0.55 | -4.15% | 105% | 101% |
| Overall | -0.28% | -0.26% | -0.26% | -2.58% | -2.56% | -2.56% | -2.42% | 1.16 | 1.14 | -1.82% | 0.52 | 0.50 | -4.06% | 106% | 101% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.27% | -0.25% | -0.25% | -2.31% | -2.30% | -2.30% | -2.15% | 1.15 | 1.14 | -1.47% | 0.46 | 0.44 | -3.84% | 111% | 101% |
| SVT12 | -0.26% | -0.23% | -0.23% | -2.46% | -2.43% | -2.43% | -2.33% | 1.17 | 1.14 | -2.08% | 0.56 | 0.53 | -4.24% | 107% | 100% |
| Overall | -0.27% | -0.24% | -0.24% | -2.38% | -2.36% | -2.36% | -2.24% | 1.16 | 1.14 | -1.78% | 0.51 | 0.49 | -4.06% | 109% | 100% |

Proposal

In order to fully bypass-code all the syntax elements in residual coding, this contribution proposes a harmonization of JVET-W0045 (CE-3.2) and JVET-W0046 (CE-1.1) with bypass-coding of last significant coefficient position (last_sig_coeff_{x,y}_prefix), where all the syntax elements in residual coding, including last significant coefficient position in RRC, are fully bypass-coded and CABAC bypass alignment is carried out once at the beginning of TB in both RRC and TSRC.

In the same as CE-3.2, an SPS level syntax element sps_high_throughput_flag is introduced to enable/disable CABAC high throughput mode. If sps_high_throughput_flag is equal to 0, decoding process of RRC and TSRC in VVC are kept. If sps_high_throughput_flag is equal to 1, the following adjustments are applied:

- The maximum number of context coded bins is set to 0 for both RRC and TSRC, which results in the coefficient or residual levels at all positions are coded as bypass bins.

- last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are coded in the bypass mode.

- The CABAC bypass alignment is carried out at the beginning of TB in RRC, i.e., before the coding last signficant coefficeint position, and, and at the beginning of TB in TSRC.

- All the syntax elements of sb_coded_flag are always coded as bypass bins.

The pseudo-code below illustrates the proposed changes to the residual coding process.

For regular residual coding:

- For remBinsPass1 and alignment // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - else

    - remBinsPass1 = 0

    - CABAC bypass alignment (i.e., set ivlCurrRange to 256)

- For last significant coefficient position

  - if (sps_high_throughput_flag == 0)

    - context_coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix

  - else

    - bypass-coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix

  - bypass-coding of last_sig_coeff_x_suffix and last_sig_coeff_y_suffix

- For remBinsPass1

  - remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1) // no change

    - if (remBinsPass1 >= 4)  // context-coded bins

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

        - set firstPosMode1

    - Pass 2:  // bypass-coded bins

      - if (n > firstPosMode1 && abs_level_gtx_flag[ ][ 1 ] == 1) abs_remainder

      - if (n <= firstPosMode1) dec_abs_level

    - Pass 3: coeff_sign_flag  // bypass-coded bins

For transform-skip residual coding (same as CE-3.2):

- For RemCcbs and alignment // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - else

    - RemCcbs = 0

    - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

- For each coefficient group // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1) // no change

    - if (RemCcbs >= 4) // context-coded bins

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4) //context-coded bins

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - Pass 3: abs_remainder, coeff_sign_flag // bypass-coded bins

Furthermore, the same modification to the derivation method of last significant coefficient position for RRC proposed in CE-1.1 is applied, which codes the relative coordinates of last significance coefficient with reference to the bottom-right corner of a zero-out transform block. Please refer to JVET-W0046 (CE-1.1) on the modification to the derivation method of last significant coefficient position in RRC.

Figure 1 shows a comparison of the bitstream format between CE-3.2 and the proposed method in RRC. It shows that in proposed method all the syntax element in residual coding are coded in only bypass-mode, and CABAC bypass alignment is necessary only once before coding last significant coefficient position.

- CE-3.2                                        (b) Proposal

Figure 1: Comparison of bitstream format between CE-3.1 and the proposed method in RRC.

Simulation results

The proposed method was integrated on top of CE-3.2 and CE-1.1 code base and simulations were conducted based on the CTC for high bit depth and high bit rate video coding [3] .

Comparison with VTM-13.0

Table 2 and Table 3 shows the results of the proposed method over VTM-13.0 under low QP and lossless condition, respectively.

Low QP

Table 2: Coding performance of the proposed method over VTM-13.0 under low QP condition

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.26% | 0.17% | 0.15% | 81% | 71% |
| SVT12 | 1.76% | 1.76% | 1.75% | 83% | 76% |
| Overall | 1.01% | 0.96% | 0.95% | 82% | 73% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.31% | 0.31% | 0.31% | 90% | 79% |
| SVT12 | 3.08% | 3.36% | 3.35% | 82% | 80% |
| Overall | 1.70% | 1.84% | 1.83% | 86% | 79% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.35% | 0.35% | 0.35% | 92% | 78% |
| SVT12 | 3.61% | 3.88% | 3.86% | 80% | 80% |
| Overall | 1.98% | 2.12% | 2.11% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 7.48% | 8.88% | 9.48% | 89% | 78% |
| HLG422 | 6.79% | 7.96% | 8.40% | 85% | 79% |
| Overall | 7.14% | 8.42% | 8.94% | 87% | 78% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 17.51% | 18.41% | 19.33% | 72% | 81% |
| HLG422 | 18.57% | 19.37% | 19.71% | 69% | 80% |
| Overall | 18.04% | 18.89% | 19.52% | 70% | 81% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 17.87% | 18.43% | 19.18% | 70% | 81% |
| HLG422 | 18.82% | 19.01% | 19.35% | 67% | 80% |
| Overall | 18.35% | 18.72% | 19.26% | 68% | 81% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 13.42% | 15.70% | 16.18% | 13.11% | 15.43% | 16.01% | 95% | 87% |
| PQ422 | 11.07% | 12.95% | 13.06% | 11.00% | 12.82% | 13.01% | 90% | 86% |
| Overall | 12.24% | 14.32% | 14.62% | 12.06% | 14.13% | 14.51% | 92% | 86% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 13.15% | 14.57% | 14.83% | 12.83% | 14.33% | 14.68% | 93% | 86% |
| PQ422 | 20.80% | 24.35% | 24.72% | 20.21% | 23.85% | 24.55% | 89% | 89% |
| Overall | 16.97% | 19.46% | 19.77% | 16.52% | 19.09% | 19.61% | 91% | 88% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 19.58% | 22.34% | 22.63% | 19.04% | 21.86% | 22.49% | 84% | 88% |
| PQ422 | 20.51% | 23.52% | 24.59% | 19.98% | 23.12% | 24.51% | 88% | 90% |
| Overall | 20.05% | 22.93% | 23.61% | 19.51% | 22.49% | 23.50% | 86% | 89% |

Lossless

Table 3: Coding performance of the proposed method over VTM-13.0 under lossless condition

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13 | Test |  | VTM13 | Test |  | VTM13 | Test |  |
| PQ444 | 2.6 | 2.5 | 3.34% | 3.1 | 2.9 | 6.41% | 3.1 | 2.9 | 6.38% |
| PQ422 | 2.4 | 2.3 | 2.86% | 2.9 | 2.7 | 5.92% | 2.9 | 2.7 | 5.86% |
| Overall | 2.5 | 2.4 | 3.10% | 3.0 | 2.8 | 6.17% | 3.0 | 2.8 | 6.12% |
| Enc Time[%] | 101% |  |  | 89% |  |  | 90% |  |  |
| Dec Time[%] | 62% |  |  | 74% |  |  | 74% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13 | Test |  | VTM13 | Test |  | VTM13 | Test |  |
| HLG444 | 1.8 | 1.7 | 1.28% | 2.0 | 1.9 | 1.34% | 2.0 | 1.9 | 1.34% |
| HLG422 | 1.7 | 1.7 | 1.11% | 1.9 | 1.9 | 1.32% | 1.9 | 1.9 | 1.31% |
| Overall | 1.7 | 1.7 | 1.19% | 1.9 | 1.9 | 1.33% | 1.9 | 1.9 | 1.32% |
| Enc Time[%] | 103% |  |  | 88% |  |  | 90% |  |  |
| Dec Time[%] | 62% |  |  | 64% |  |  | 68% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13 | Test |  | VTM13 | Test |  | VTM13 | Test |  |
| SVT16 | 1.2 | 1.2 | -0.11% | 1.2 | 1.2 | -0.29% | 1.2 | 1.2 | -0.26% |
| SVT12 | 1.3 | 1.3 | 0.66% | 1.3 | 1.3 | 0.39% | 1.3 | 1.3 | 0.41% |
| Overall | 1.2 | 1.2 | 0.28% | 1.3 | 1.3 | 0.05% | 1.3 | 1.3 | 0.07% |
| Enc Time[%] | 87% |  |  | 90% |  |  | 87% |  |  |
| Dec Time[%] | 61% |  |  | 67% |  |  | 66% |  |  |

Binrate and Bin-to-Bit ratio at low QP

To evaluate bin-rate and bin-to-bit ratio, the relevant tests were conducted based on the method in JVET-N0049 [4] , and JVET-V0150 [5] .

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bitrate of a coded sequence

- The average context coded bin rate, bypass bin rate and bitrate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bitrate consumed for the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements for the peak case, for the average case and for the transform coefficients coding part are calculated based on the collected data:

- Weighted BD-bin rate

- {Weighted, Unweighted} bin to bit ratio

- Weighted Delta Binrate

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1).

The summary results of proposed method compared with VTM-13.0 are provided in Table 4 as below. For detailed results, please refer to the attached spreadsheets.

Table 4: BD-bitrate, BD-binrate, Delta-Binrate and Bin-to-Bit ratio of the proposed method over VTM-13.0 under low QP condition

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.26% | 0.17% | 0.15% | -43.76% | -43.78% | -43.79% | -43.79% | 1.16 | 1.01 | -12.91% | 0.47 | 0.26 | -43.76% | 81% | 71% |
| SVT12 | 1.76% | 1.76% | 1.75% | -52.25% | -52.24% | -52.24% | -52.45% | 1.17 | 1.01 | -13.40% | 0.58 | 0.27 | -53.20% | 83% | 76% |
| Overall | 1.01% | 0.96% | 0.95% | -48.00% | -48.01% | -48.02% | -48.12% | 1.16 | 1.01 | -13.16% | 0.52 | 0.27 | -48.96% | 82% | 73% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.31% | 0.31% | 0.31% | -44.73% | -44.69% | -44.69% | -44.73% | 1.16 | 1.01 | -12.84% | 0.47 | 0.26 | -43.37% | 90% | 79% |
| SVT12 | 3.08% | 3.36% | 3.35% | -55.29% | -55.24% | -55.23% | -55.47% | 1.17 | 1.01 | -13.41% | 0.57 | 0.27 | -52.59% | 82% | 80% |
| Overall | 1.70% | 1.84% | 1.83% | -50.01% | -49.96% | -49.96% | -50.10% | 1.16 | 1.01 | -13.13% | 0.52 | 0.27 | -48.44% | 86% | 79% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.35% | 0.35% | 0.35% | -45.10% | -45.04% | -45.04% | -45.11% | 1.15 | 1.01 | -12.55% | 0.46 | 0.26 | -42.40% | 92% | 78% |
| SVT12 | 3.61% | 3.88% | 3.86% | -56.03% | -55.96% | -55.95% | -56.35% | 1.17 | 1.01 | -13.30% | 0.56 | 0.27 | -51.29% | 80% | 80% |
| Overall | 1.98% | 2.12% | 2.11% | -50.56% | -50.50% | -50.50% | -50.73% | 1.16 | 1.01 | -12.92% | 0.51 | 0.27 | -47.26% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 7.48% | 8.88% | 9.48% | -60.70% | -60.24% | -59.84% | -62.78% | 1.24 | 1.02 | -17.80% | 0.80 | 0.29 | -63.18% | 89% | 78% |
| HLG422 | 6.79% | 7.96% | 8.40% | -58.75% | -58.30% | -57.93% | -60.63% | 1.23 | 1.02 | -16.86% | 0.77 | 0.30 | -61.39% | 85% | 79% |
| Overall | 7.14% | 8.42% | 8.94% | -59.72% | -59.27% | -58.89% | -61.70% | 1.24 | 1.02 | -17.33% | 0.78 | 0.29 | -62.30% | 87% | 78% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 17.51% | 18.41% | 19.33% | -66.50% | -66.13% | -65.70% | -70.62% | 1.25 | 1.02 | -17.99% | 0.81 | 0.30 | -63.67% | 72% | 81% |
| HLG422 | 18.57% | 19.37% | 19.71% | -64.78% | -64.49% | -64.11% | -69.31% | 1.23 | 1.03 | -16.80% | 0.78 | 0.30 | -61.32% | 69% | 80% |
| Overall | 18.04% | 18.89% | 19.52% | -65.64% | -65.31% | -64.90% | -69.97% | 1.24 | 1.02 | -17.40% | 0.80 | 0.30 | -62.52% | 70% | 81% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 17.87% | 18.43% | 19.18% | -66.44% | -66.21% | -65.88% | -70.72% | 1.24 | 1.02 | -17.17% | 0.79 | 0.29 | -62.53% | 70% | 81% |
| HLG422 | 18.82% | 19.01% | 19.35% | -64.78% | -64.72% | -64.36% | -69.44% | 1.22 | 1.03 | -16.14% | 0.75 | 0.30 | -60.20% | 67% | 80% |
| Overall | 18.35% | 18.72% | 19.26% | -65.61% | -65.46% | -65.12% | -70.08% | 1.23 | 1.02 | -16.66% | 0.77 | 0.30 | -61.39% | 68% | 81% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 13.37% | 15.63% | 16.23% | -60.28% | -59.76% | -59.44% | -96.68% | 1.27 | 1.04 | -18.55% | 0.90 | 0.32 | -64.36% | 94% | 87% |
| PQ422 | 11.05% | 12.70% | 12.85% | -59.59% | -58.98% | -58.65% | -62.53% | 1.26 | 1.04 | -18.10% | 0.87 | 0.32 | -63.09% | 92% | 86% |
| Overall | 12.21% | 14.17% | 14.54% | -59.94% | -59.37% | -59.04% | -79.61% | 1.27 | 1.04 | -18.32% | 0.88 | 0.32 | -63.73% | 93% | 87% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 20.21% | 23.85% | 24.55% | -61.65% | -60.67% | -60.53% | -66.79% | 1.26 | 1.04 | -17.57% | 0.89 | 0.32 | -64.09% | 89% | 89% |
| PQ422 | 19.04% | 21.86% | 22.49% | -61.14% | -60.46% | -60.05% | -66.23% | 1.24 | 1.04 | -16.20% | 0.83 | 0.32 | -61.34% | 84% | 88% |
| Overall | 19.63% | 22.85% | 23.52% | -61.40% | -60.57% | -60.29% | -66.51% | 1.25 | 1.04 | -16.89% | 0.86 | 0.32 | -62.77% | 86% | 89% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 19.98% | 23.12% | 24.51% | -61.85% | -61.00% | -60.60% | -67.07% | 1.23 | 1.04 | -15.82% | 0.84 | 0.32 | -61.80% | 88% | 90% |
| PQ422 | 18.67% | 21.41% | 22.01% | -61.34% | -60.59% | -60.19% | -66.41% | 1.22 | 1.04 | -15.39% | 0.80 | 0.32 | -60.30% | 83% | 88% |
| Overall | 19.32% | 22.27% | 23.26% | -61.59% | -60.79% | -60.39% | -66.74% | 1.23 | 1.04 | -15.61% | 0.82 | 0.32 | -61.06% | 85% | 89% |

Comparison with CE-3.2

Table 5 and Table 6 shows the results of the proposed method over CE-3.2 under low QP and lossless condition, respectively.

Low QP

Table 5: Coding performance of the proposed method over CE-3.2 under low QP condition

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | 0.19% | 0.08% | 0.07% | 95% | 90% |
| SVT12 | 0.24% | 0.06% | 0.06% | 100% | 91% |
| Overall | 0.22% | 0.07% | 0.07% | 98% | 91% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -0.01% | -0.05% | -0.04% | 99% | 94% |
| SVT12 | 0.08% | 0.06% | 0.07% | 100% | 95% |
| Overall | 0.03% | 0.01% | 0.01% | 100% | 94% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | 0.00% | -0.06% | -0.06% | 100% | 94% |
| SVT12 | 0.20% | 0.18% | 0.18% | 97% | 95% |
| Overall | 0.10% | 0.06% | 0.06% | 99% | 95% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.80% | 0.32% | 0.33% | 113% | 95% |
| HLG422 | 0.69% | 0.30% | 0.31% | 107% | 94% |
| Overall | 0.74% | 0.31% | 0.32% | 110% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.00% | 0.59% | 0.61% | 100% | 99% |
| HLG422 | 0.97% | 0.45% | 0.61% | 103% | 98% |
| Overall | 0.98% | 0.52% | 0.61% | 101% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.99% | 0.55% | 0.56% | 100% | 98% |
| HLG422 | 0.97% | 0.39% | 0.56% | 98% | 99% |
| Overall | 0.98% | 0.47% | 0.56% | 99% | 98% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 2.27% | 1.94% | 2.11% | 2.27% | 1.95% | 2.12% | 105% | 97% |
| PQ422 | 1.81% | 1.49% | 1.63% | 1.80% | 1.48% | 1.63% | 104% | 97% |
| Overall | 2.04% | 1.71% | 1.87% | 2.03% | 1.72% | 1.88% | 105% | 97% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.23% | 1.05% | 1.16% | 1.24% | 0.99% | 1.09% | 108% | 98% |
| PQ422 | 2.47% | 2.57% | 2.79% | 2.45% | 2.55% | 2.78% | 110% | 98% |
| Overall | 1.85% | 1.81% | 1.97% | 1.84% | 1.77% | 1.94% | 109% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 2.20% | 2.41% | 2.67% | 2.20% | 2.38% | 2.64% | 104% | 97% |
| PQ422 | 2.42% | 2.43% | 2.64% | 2.40% | 2.42% | 2.62% | 107% | 99% |
| Overall | 2.31% | 2.42% | 2.65% | 2.30% | 2.40% | 2.63% | 106% | 98% |

Lossless

Table 6: Coding performance of the proposed method over CE-3.2 under lossless condition

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE-3.2 | Test |  | CE-3.2 | Test |  | CE-3.2 | Test |  |
| PQ444 | 2.5 | 2.5 | 0.18% | 2.9 | 2.9 | 0.03% | 2.9 | 2.9 | 0.00% |
| PQ422 | 2.3 | 2.3 | 0.19% | 2.7 | 2.7 | 0.08% | 2.7 | 2.7 | 0.04% |
| Overall | 2.4 | 2.4 | 0.19% | 2.8 | 2.8 | 0.06% | 2.8 | 2.8 | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 86% |  |  | 95% |  |  | 95% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE-3.2 | Test |  | CE-3.2 | Test |  | CE-3.2 | Test |  |
| HLG444 | 1.8 | 1.7 | 0.29% | 1.9 | 1.9 | 0.02% | 1.9 | 1.9 | 0.01% |
| HLG422 | 1.7 | 1.7 | 0.23% | 1.9 | 1.9 | 0.06% | 1.9 | 1.9 | 0.04% |
| Overall | 1.7 | 1.7 | 0.26% | 1.9 | 1.9 | 0.04% | 1.9 | 1.9 | 0.02% |
| Enc Time[%] | 104% |  |  | 98% |  |  | 101% |  |  |
| Dec Time[%] | 85% |  |  | 91% |  |  | 94% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE-3.2 | Test |  | CE-3.2 | Test |  | CE-3.2 | Test |  |
| SVT16 | 1.2 | 1.2 | 0.18% | 1.2 | 1.2 | -0.01% | 1.2 | 1.2 | 0.00% |
| SVT12 | 1.3 | 1.3 | 0.24% | 1.3 | 1.3 | -0.01% | 1.3 | 1.3 | -0.01% |
| Overall | 1.2 | 1.2 | 0.21% | 1.3 | 1.3 | -0.01% | 1.3 | 1.3 | -0.01% |
| Enc Time[%] | 97% |  |  | 102% |  |  | 98% |  |  |
| Dec Time[%] | 90% |  |  | 95% |  |  | 94% |  |  |

Binrate and Bin-to-Bit ratio at low QP

To evaluate bin-rate and bin-to-bit ratio, the relevant tests were conducted based on the method in JVET-N0049 [4] and JVET-V0150 [5] described above. Table 7 shows the summary results of proposed method compared with CE-3.2.

Table 7: BD-bitrate, BD-binrate, Delta-Binrate and Bin-to-Bit ratio of the proposed method over CE-3.2 under low QP condition

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.19% | 0.08% | 0.07% | -9.85% | -9.95% | -9.95% | -10.02% | 1.04 | 1.01 | -2.61% | 0.29 | 0.26 | -10.08% | 95% | 90% |
| SVT12 | 0.24% | 0.06% | 0.06% | -12.47% | -12.63% | -12.63% | -12.69% | 1.05 | 1.01 | -3.35% | 0.31 | 0.27 | -12.58% | 100% | 91% |
| Overall | 0.22% | 0.07% | 0.07% | -11.16% | -11.29% | -11.29% | -11.35% | 1.04 | 1.01 | -2.98% | 0.30 | 0.27 | -11.36% | 98% | 91% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.01% | -0.05% | -0.04% | -7.84% | -7.86% | -7.86% | -7.81% | 1.03 | 1.01 | -2.55% | 0.29 | 0.26 | -9.88% | 99% | 94% |
| SVT12 | 0.08% | 0.06% | 0.07% | -9.17% | -9.17% | -9.17% | -9.31% | 1.05 | 1.01 | -3.38% | 0.31 | 0.27 | -12.66% | 100% | 95% |
| Overall | 0.03% | 0.01% | 0.01% | -8.51% | -8.52% | -8.52% | -8.56% | 1.04 | 1.01 | -2.97% | 0.30 | 0.27 | -11.31% | 100% | 94% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.00% | -0.06% | -0.06% | -7.87% | -7.92% | -7.92% | -7.86% | 1.03 | 1.01 | -2.46% | 0.29 | 0.26 | -9.54% | 100% | 94% |
| SVT12 | 0.20% | 0.18% | 0.18% | -8.96% | -8.98% | -8.98% | -9.21% | 1.05 | 1.01 | -3.39% | 0.31 | 0.27 | -12.66% | 97% | 95% |
| Overall | 0.10% | 0.06% | 0.06% | -8.42% | -8.45% | -8.45% | -8.54% | 1.04 | 1.01 | -2.93% | 0.30 | 0.27 | -11.14% | 99% | 95% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.80% | 0.32% | 0.33% | -12.51% | -12.78% | -12.72% | -13.02% | 1.05 | 1.02 | -3.15% | 0.34 | 0.29 | -13.33% | 113% | 95% |
| HLG422 | 0.69% | 0.30% | 0.31% | -13.03% | -13.27% | -13.24% | -13.52% | 1.06 | 1.02 | -3.36% | 0.34 | 0.30 | -13.70% | 107% | 94% |
| Overall | 0.74% | 0.31% | 0.32% | -12.77% | -13.02% | -12.98% | -13.27% | 1.06 | 1.02 | -3.26% | 0.34 | 0.29 | -13.52% | 110% | 94% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 1.00% | 0.59% | 0.61% | -9.61% | -9.37% | -8.95% | -9.87% | 1.06 | 1.02 | -3.43% | 0.35 | 0.30 | -15.26% | 100% | 99% |
| HLG422 | 0.97% | 0.45% | 0.61% | -10.19% | -10.19% | -9.88% | -10.67% | 1.06 | 1.03 | -3.56% | 0.35 | 0.30 | -14.97% | 103% | 98% |
| Overall | 0.98% | 0.52% | 0.61% | -9.90% | -9.78% | -9.42% | -10.27% | 1.06 | 1.02 | -3.50% | 0.35 | 0.30 | -15.11% | 101% | 98% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.99% | 0.55% | 0.56% | -9.30% | -9.11% | -8.68% | -9.61% | 1.06 | 1.02 | -3.41% | 0.34 | 0.29 | -14.06% | 100% | 98% |
| HLG422 | 0.97% | 0.39% | 0.56% | -9.77% | -9.82% | -9.47% | -10.30% | 1.06 | 1.03 | -3.62% | 0.35 | 0.30 | -14.33% | 98% | 99% |
| Overall | 0.98% | 0.47% | 0.56% | -9.54% | -9.47% | -9.08% | -9.96% | 1.06 | 1.02 | -3.51% | 0.35 | 0.30 | -14.20% | 99% | 98% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 2.09% | 1.76% | 1.92% | -13.21% | -13.43% | -13.29% | -15.72% | 1.07 | 1.04 | -2.87% | 0.37 | 0.32 | -14.81% | 105% | 97% |
| PQ422 | 1.66% | 1.39% | 1.52% | -13.35% | -13.52% | -13.44% | -14.56% | 1.07 | 1.04 | -3.01% | 0.37 | 0.32 | -14.56% | 106% | 97% |
| Overall | 1.87% | 1.57% | 1.72% | -13.28% | -13.47% | -13.36% | -15.14% | 1.07 | 1.04 | -2.94% | 0.37 | 0.32 | -14.69% | 105% | 97% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 2.45% | 2.55% | 2.78% | -14.41% | -14.35% | -14.22% | -16.33% | 1.07 | 1.04 | -3.22% | 0.39 | 0.32 | -17.08% | 110% | 98% |
| PQ422 | 2.20% | 2.38% | 2.64% | -14.22% | -14.28% | -14.21% | -16.22% | 1.07 | 1.04 | -3.25% | 0.38 | 0.32 | -16.26% | 104% | 97% |
| Overall | 2.32% | 2.46% | 2.71% | -14.31% | -14.31% | -14.22% | -16.27% | 1.07 | 1.04 | -3.24% | 0.38 | 0.32 | -16.67% | 107% | 98% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 2.40% | 2.42% | 2.62% | -14.17% | -14.17% | -14.07% | -16.07% | 1.07 | 1.04 | -3.08% | 0.38 | 0.32 | -15.85% | 107% | 99% |
| PQ422 | 2.14% | 2.23% | 2.47% | -13.88% | -14.00% | -13.95% | -15.81% | 1.07 | 1.04 | -3.35% | 0.38 | 0.32 | -15.97% | 107% | 97% |
| Overall | 2.27% | 2.33% | 2.55% | -14.03% | -14.08% | -14.01% | -15.94% | 1.07 | 1.04 | -3.21% | 0.38 | 0.32 | -15.91% | 107% | 98% |

Estimated clock cycle

To measure the throughput performance of the proposed method, the clock cycle is also estimated for VTM-13.0 and the proposed method based on the method described in JVET-W0044 [6] .

The clock cycle is estimated as follows:

Clk Cycle in MHz = (((totalCtxBins + totalbypassBin/bypassWeight)* numCodedFrame)/framerate)/1000000

Where,

- totalCtxBins = Total number of context coded bins in the sequence

- totalbypassBin = Total number of bypass coded bins

- bypassWeight is the number of bypass coded bins can be parsed per cycle. For anchor, bypassWeight is equal to 4. For proposed method, the bypassWeight is equal to 12.

- numCodedFrame is the total number of encoded frames

Table 8 shows the estimated saving of clock cycle over VTM-13.0 at low QPs. Table 9 shows the estimated max clock cycle per sequence at low QPs.

In the clock cycle estimation, it is assumed that the proposed method can process 12 bypass-coded bin per cycle while VTM can process 4 bypass-coded bin per cycle. As shown in Table 8, the proposed method can save roughly 80% of clock cycle for SVT-16 sequences.

As shown in Table 9, the max clock cycle of VTM-13.0 reaches up to about 1450 MHz but that of the proposed method is roughly 300 MHz for SVT-16 sequences.

Table 8: Estimated saving of clock cycle over VTM-13.0

|  | HDR PQ | HDR HLG | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- |
| AI | 33% | 12% | 15% | 18% |

Table 9: Estimated max clock cycle per sequence at low QPs

|  |  | VTM-13.0 | JVET-W0114 |
| --- | --- | --- | --- |
|  |  | clk cycle TRF bin in MHz | clk cycle TRF bin in MHz |
| SVT-AI16 | OldTownCross | 1435.4 | 287.7 |
|  | CrowdRun | 1423.5 | 282.8 |
|  | DucksTakeOff | 1441.5 | 289.3 |
|  | InToTree | 1404.8 | 277.5 |
|  | ParkJoy | 1420.4 | 281.6 |
| SVT-AI12 | OldTownCross | 1157.4 | 201.5 |
|  | CrowdRun | 1136.5 | 196.3 |
|  | DucksTakeOff | 1163.4 | 203.0 |
|  | InToTree | 1119.0 | 191.2 |
|  | ParkJoy | 1132.5 | 195.1 |
| HLG-AI444 | DayStreet | 1056.1 | 150.5 |
|  | PeopleInShoppingCenter | 1063.6 | 153.1 |
|  | NightStreet | 1155.4 | 176.8 |
|  | StainedGlass | 1103.7 | 165.5 |
| HLG-AI422 | DayStreet | 727.7 | 106.7 |
|  | PeopleInShoppingCenter | 729.3 | 107.7 |
|  | NightStreet | 781.6 | 120.8 |
|  | StainedGlass | 757.6 | 115.7 |
| PQ-AI444 | BalloonFestival | 179.1 | 70.8 |
|  | Hurdles | 681.4 | 288.8 |
|  | Market3 | 817.9 | 330.2 |
|  | Starting | 615.8 | 265.7 |
|  | FireEater | 299.7 | 96.6 |
| PQ-AI422 | BalloonFestival | 136.5 | 55.3 |
|  | Hurdles | 524.1 | 223.5 |
|  | Market3 | 576.4 | 244.7 |
|  | Starting | 474.5 | 202.5 |
|  | FireEater | 226.7 | 74.3 |

Conclusion

This contribution proposes a harmonization of JVET-W0045 (CE-3.2) and JVET-W0046 (CE-3.1) with bypass-coding of last significant coefficient position, where all the syntax elements in residual coding, including last significant coefficient position in RRC, are fully bypass-coded and CABAC bypass alignment is carried out once at the beginning of TB in both RRC and TSRC.

According to the above results, the proposed method provides large gain in BD-binrate, Delta Binrate and bin-to-bit ratio over VTM-13.0 with small coding loss if bitrate is enough high; For the SVT 16-bit sequences, 0.26% loss for AI, 0.31% loss for LB and 0.35% loss for RA. Compared to CE-3.2, it provides additionally about 8~10% gain in BD-binrate or Delta Binrate and about 10% gain in Weighted Bin-to-Bit ratio with negligible loss, 0.19% for AI, -0.01% for LB and 0.00% for RA, for SVT 16-bit sequences. It is asserted that the proposed method is beneficial to improve the CABAC high throughput for high bit-depth coding and high bitrate coding like 16-bit high throughput profile.

It is recommended to study the proposed method in CE.

Reference

- F. Wang, “CE-3.2: a high throughput mode for high bit depth and high bit rate extensions”, JVET-W0045.

- F. Wang, “CE-1.1: coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-W0046.

- A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018.

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049.

- T. Tsukuba, M. Ikeda, T. Suzuki, “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding”, JVET-V0150.

- M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao, “CE3.1: CABAC-bypass alignment for high bit-depth coding”, JVET-W0044

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).