Changes for CE-3.2 are highlighted in yellow.

Changes for CE-1.1 are highlighted in cyan.

Changes for bypass-coding last significant coefficient position is highlighted in green.

# Changes to the specification text:

# Change clause 7.3.2.22 (Sequence parameter set range extension syntax ) as follows (additions are yellow-highlighted):

- 7.3.2.22	Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| sps_reverse_last_sig_coeff_enabled_flag | u(1) |
| sps_high_throughput_mode_enabled_flag | u(1) |
| } |  |

- 7.3.7 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_reverse_last_sig_coeff_enabled_flag ) |  |
| sh_ reverse_last_sig_coeff_flag | u(1) |
| … |  |

# Change clause 7.3.11.11 (Residual coding syntax) as follows (additions are yellow-highlighted):

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( log2TbWidth > 0 ) |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = sps_high_throughput_mode_enabled_flag ? 0 : ( ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 ) |  |
| ... |  |
| if( sb_coded_flag[ xS ][ yS ] ) { |  |
| dec_abs_level[ n ] | ae(v) |
| if( updateHist && dec_abs_level[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( dec_abs_level[ n ] ) ) + sps_high_throughput_mode_enabled_flag ? 2 : 0) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| } |  |

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| inferSbCbf = 1 |  |
| RemCcbs = sps_high_throughput_mode_enabled_flag ? 0 : ( ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 ) |  |

# In clause 7.4.3.22, add the following after the semantics of sps_persistent_rice_adaptation_enabled_flag :

- 7.4.3.22 Sequence parameter set range extension semantics

…

sps_reverse_last_sig_coeff_enabled_flag equal to 1 specifies that sh_reverse_last_sig_coeff_flag is present in slice_header( ) syntax structure referring to the SPS. sps_reverse_last_sig_coeff_enabled_flag equal to 0 specifies that sh_reverse_last_sig_coeff_flag is not present in slice_header( ) syntax structure referring to the SPS. When not present, the value of sps_reverse_last_sig_coeff_enabled_flag is inferred to be equal to 0.

sps_high_throughput_mode_enabled_flag equal to 1 specifies that the high throughput mode is enabled.

sps_high_throughput_mode_enabled_flag equal to 0 specifies that the high throughput mode is not enabled. When not present, the value of sps_high_throughput_mode_enabled_flag is inferred to be equal to 0.

- 7.4.8 Slice header semantics

…

sh_reverse_last_sig_coeff_flag equal to 1 specifies that the last significant coefficient is measured and coded in reference to the bottom-right corner of zero-out transform block for corresponding slice. sh_reverse_last_sig_coeff_flag equal to 0 specifies that the last significant coefficient is measured and coded in reference to the top-left corner of zero-out transform block for corresponding slice. When sh_reverse_last_sig_coeff_flag is not present, its value is inferred to be equal to 0.

…

- 7.4.12.11 Residual coding semantics

…

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred to be 0.

last_sig_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows:

- If last_sig_coeff_x_suffix is not present, the following applies:

LastSignificantCoeffX = last_sig_coeff_x_prefix		(192)

- Otherwise (last_sig_coeff_x_suffix is present), the following applies:

LastSignificantCoeffX = ( 1  <<  ( (last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) *	(193)
			( 2 + (last_sig_coeff_x_prefix & 1 ) ) + last_sig_coeff_x_suffix

If sh_reverse_last_sig_coeff_flag is equal to 1, the following applies:

LastSignificantCoeffX =  ( 1 << log2ZoTbWidth ) - 1- LastSignificantCoeffX

last_sig_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows:

- If last_sig_coeff_y_suffix is not present, the following applies:

LastSignificantCoeffY = last_sig_coeff_y_prefix		(194)

- Otherwise (last_sig_coeff_y_suffix is present), the following applies:

LastSignificantCoeffY = ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) *	(195)
			( 2 + ( last_sig_coeff_y_prefix & 1 ) ) + last_sig_coeff_y_suffix

If sh_reverse_last_sig_coeff_flag is equal to 1, the following applies:

LastSignificantCoeffY =  ( 1 << log2ZoTbHeight ) - 1- LastSignificantCoeffY

# In clause 9.3.1, make the following changes (additions are highlighted in yellow):

- 9.3 CABAC parsing process for slice data

- 9.3.1General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.11.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

When sps_high_throughput_mode_enabled_flag is equal to 1, the request for a value of a syntax element is for the first bypass decoding syntax element in last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix, last_sig_coeff_y_suffix, sb_coded_flag[][], abs_remainder[ ] and dec_abs_level[ n ] in current transform block, the alignment process prior to bypass decoding as specified in subclause 9.3.4.3.6 is invoked.

In clause 9.3.4.2.1, make the following changes (additions are highlighted in yellow):

- 9.3.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

- 9.3.4.2.1 General

Table 132 – Assignment of ctxInc to syntax elements with context coded bins

| last_sig_coeff_x_prefix sps_high_throughput_mode_enabled_flag == 0 | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| last_sig_coeff_x_prefix sps_high_throughput_mode_enabled_flag == 1 | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_prefix sps_high_throughput_mode_enabled_flag == 0 | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| last_sig_coeff_y_prefix sps_high_throughput_mode_enabled_flag == 1 | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| sb_coded_flag[ ][ ] sps_high_throughput_mode_enabled_flag == 0 | 0..6 (clause 9.3.4.2.6) | na | na | na | na | na |
| sb_coded_flag[ ][ ] sps_high_throughput_mode_enabled_flag == 1 | bypass | na | na | na | na | na |

Add new clause 9.3.4.3.6 as follows:

- 9.3.4.3.6 Alignment process prior to aligned bypass decoding

Input to this process is the variable ivlCurrRange.

Output of this process is the updated variable ivlCurrRange.

ivlCurrRange is set equal to 256.