| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0115-v2 |
| --- | --- |

| Title: | AHG8: A study on Bin-to-Bit ratio of VTM-13.0 for high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Steve Keating Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com steve.keating@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Group Corporation, Sony Europe BV |  |  |

_____________________________

- Abstract

This contribution reports the coding performance (BD-bitrate, BD-binrate) and bin-to-bit ratio of VTM-13.0 relative to HM-16.23 for high bit depth coding under low QP setting (QP=-13,…,12 for 12-bit, QP=-33,…,-8 for 16-bit) and normal QP setting (QP=22, 27, 32, 37). The average results are summarized as follows:

In the normal QPs,

- BD-bitrate changes are -26.21% for AI and -17.17% for RA, respectively.

- BD-binrate changes are -18.01% for AI and -2.43% for RA, respectively.

- The unweighted bin to bit ratios of VTM are roughly 13~15% higher than HM’s. The weighted ones are roughly 20% higher than HM’s.

In the low QPs,

- BD-bitrate changes for AI/LDB/RA is {-8.61%, -7.34%, -7.54%} for PQ, {-4.95%, -5.74%, -6.24%} for HLG, and {-3.31%, -3.37%, -3.56%} for SVT, respectively.

- BD-binrate changes for AI/LDB/RA is {7.71%, 6.41%, 6.51%} for PQ, {3.75%, 3.32%, 3.03%} for HLG, and {-1.77%, -2.39%, -2.07%} for SVT, respectively.

- The unweighted bin to bit ratio changes for AI/LDB/RA are {6.35%, 5.36%, 3.66%} for PQ, {2.72%, 3.60%, 2.42%} for HLG, and {-3.65%, -3.56%, -3.80%} for SVT, respectively. The weighted bin to bit ratio changes for AI/LDB/RA are {17.77%, 18.30%, 17.40%} for PQ, {10.75%, 12.34%, 11.74%} for HLG and {1.68%, 1.88%, 1.37 %} for SVT, respectively.

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1). All the percentage numbers are measured against the HM-16.23.

Introduction

One of the scopes of AHG8: High bit depth, high bit rate, and high frame rate coding is as below.

- Study VVC entropy decoding throughput in the cases of high bit depth, high bit rate, and high frame rate coding.

To investigate CABAC throughput of VTM-13.0 [1] for high bit depth coding, a study was conducted to measure the bin to bit ratio and BD-binrate following JVET-N0049 [3] and JVET-V0150 [4]  under low QP setting (QP=-13,…,12 for 12bit, QP=-33,…,-8 for 16bit) and normal QP setting (QP=22, 27, 32, 37) compared to HM-16.23 [2] . Additionally, delta binrate was also measured by using Bjøntegaard Delta-rate on bitrate vs bin rate.

Simulation results

Tests were conducted by using the VTM-13.0 and HM-16.23 for the low QPs (QP=-13,…,12 for 12bit, QP=-33,…,-8 for 16bit) and normal QPs (QP=22, 27, 32, 37) under the common test condition defined in [5] . Note that to align GOP size between VTM-13.0 and HM-16.23, GOP size is set to 16, and extended precision of HM-16.23 is set to 0 for 12-bit and 1 for 16-bit.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bitrate of a coded sequence

- The average context coded bin rate, bypass bin rate and bitrate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bitrate consumed for the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements for the peak case, for the average case and for the transform coefficients coding part are calculated based on the collected data:

- Weighted BD-bin rate

- {Weighted, Unweighted} bin to bit ratio

- Weighted Delta Binrate, which is measured by using Bjøntegaard Delta-rate on bitrate vs binrate

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1).

The summary results of VTM-13.0 for the normal QPs, and low QPs are provided in Table 1 and Table 2, respectively. For detailed results, please refer to the attached spreadsheets.

Table 1: Summary results of normal QPs (Anchor=HM-16.23 Tested=VTM-13.0, GOP16)

|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| Class H1 | -21.38% | -65.31% | -76.35% | -12.90% | -59.01% | -71.46% | 10.72% | 1.19 | 1.33 | 12.31% | 0.87 | 1.05 | 20.24% | 376% | 166% |
| Class H2 | -31.05% | -54.17% | -53.43% | -23.12% | -48.09% | -47.16% | 15.03% | 1.18 | 1.35 | 13.97% | 0.89 | 1.07 | 19.65% | 374% | 143% |
| Overall | -26.21% | -59.74% | -64.89% | -18.01% | -53.55% | -59.31% | 12.87% | 1.19 | 1.34 | 13.14% | 0.88 | 1.06 | 19.95% | 375% | 154% |
|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| Class H1 | -11.88% | -67.94% | -54.94% | 3.18% | -59.75% | -49.11% | 17.36% | 1.21 | 1.37 | 13.24% | 0.92 | 1.10 | 20.30% | 2863% | 178% |
| Class H2 | -22.45% | -49.20% | -49.99% | -8.05% | -39.58% | -40.39% | 18.52% | 1.21 | 1.41 | 16.53% | 0.94 | 1.15 | 22.00% | 3356% | 158% |
| Overall | -17.17% | -58.57% | -52.47% | -2.43% | -49.67% | -44.75% | 17.94% | 1.21 | 1.39 | 14.89% | 0.93 | 1.13 | 21.16% | 3100% | 168% |

In the normal QPs (Table 1),

- VTM achieves BD-bitrate saving of -17.17% for AI and -26.21% for RA. Note that coding gain over HM-16.23 for RA is relatively decreased than over HM-16.22 due to enabling Temporal Filter at HM-16.23.

- BD-binrate saving are -2.43% for AI and -18.01% for RA, respectively. It indicates VTM consumes less bins for the same PSNR values.

- The unweighted bin to bit ratios of VTM are roughly 13~15% higher than HM’s. The weighted ones are roughly 20% higher than HM’s.

- The weighted delta binrate are roughly 17.94% for AI and 12.87% for RA, respectively. It indicates VTM consumes more bins for the same bitrate values than HM’s.

Table 2: Summary results at low QPs (Anchor=HM-16.23 Tested=VTM-13.0)

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -7.28% | -8.43% | -9.16% | 10.15% | 8.81% | 8.12% | 16.80% | 1.19 | 1.27 | 6.57% | 0.76 | 0.90 | 18.15% | 3421% | 166% |
| PQ422 | -9.94% | -14.14% | -14.75% | 5.28% | 1.18% | 0.35% | 13.63% | 1.19 | 1.26 | 6.13% | 0.74 | 0.87 | 17.39% | 2878% | 160% |
| Overall | -8.61% | -11.28% | -11.96% | 7.71% | 5.00% | 4.23% | 15.21% | 1.19 | 1.27 | 6.35% | 0.75 | 0.88 | 17.77% | 3150% | 163% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.73% | -5.00% | -6.63% | 8.58% | 9.90% | 8.77% | 15.08% | 1.19 | 1.26 | 6.14% | 0.74 | 0.89 | 20.17% | 364% | 178% |
| PQ422 | -7.95% | -11.75% | -12.42% | 4.24% | 0.72% | -0.23% | 10.99% | 1.18 | 1.24 | 4.58% | 0.71 | 0.83 | 16.34% | 469% | 179% |
| Overall | -7.34% | -8.38% | -9.52% | 6.41% | 5.31% | 4.27% | 13.04% | 1.18 | 1.25 | 5.36% | 0.73 | 0.86 | 18.30% | 416% | 178% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.88% | -5.15% | -6.48% | 8.59% | 9.98% | 9.00% | 15.41% | 1.19 | 1.23 | 3.90% | 0.71 | 0.84 | 17.92% | 428% | 180% |
| PQ422 | -8.21% | -12.16% | -12.44% | 4.43% | 0.62% | 0.04% | 11.64% | 1.18 | 1.22 | 3.41% | 0.69 | 0.80 | 16.87% | 588% | 172% |
| Overall | -7.54% | -8.66% | -9.46% | 6.51% | 5.30% | 4.52% | 13.53% | 1.19 | 1.23 | 3.66% | 0.70 | 0.82 | 17.40% | 508% | 176% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -4.63% | -6.64% | -6.95% | 4.16% | 2.43% | 1.98% | 7.55% | 1.20 | 1.24 | 3.18% | 0.72 | 0.80 | 10.71% | 4048% | 167% |
| HLG422 | -5.27% | -8.00% | -8.02% | 3.35% | 1.14% | 0.97% | 7.04% | 1.20 | 1.23 | 2.27% | 0.69 | 0.77 | 10.79% | 3536% | 163% |
| Overall | -4.95% | -7.32% | -7.49% | 3.75% | 1.79% | 1.48% | 7.30% | 1.20 | 1.24 | 2.72% | 0.71 | 0.78 | 10.75% | 3792% | 165% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -6.01% | -8.34% | -7.72% | 3.99% | 1.67% | 2.12% | 8.57% | 1.20 | 1.25 | 4.10% | 0.72 | 0.81 | 12.36% | 449% | 149% |
| HLG422 | -5.48% | -9.48% | -7.96% | 2.66% | -1.17% | -0.04% | 6.58% | 1.20 | 1.23 | 3.10% | 0.69 | 0.78 | 12.32% | 545% | 154% |
| Overall | -5.74% | -8.91% | -7.84% | 3.32% | 0.25% | 1.04% | 7.57% | 1.20 | 1.24 | 3.60% | 0.71 | 0.80 | 12.34% | 497% | 151% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -6.48% | -8.85% | -8.04% | 3.69% | 1.30% | 1.98% | 8.89% | 1.20 | 1.24 | 2.87% | 0.70 | 0.79 | 11.76% | 510% | 150% |
| HLG422 | -6.00% | -9.80% | -8.15% | 2.36% | -1.40% | -0.14% | 6.92% | 1.20 | 1.22 | 1.96% | 0.68 | 0.75 | 11.71% | 678% | 153% |
| Overall | -6.24% | -9.32% | -8.09% | 3.03% | -0.05% | 0.92% | 7.91% | 1.20 | 1.23 | 2.42% | 0.69 | 0.77 | 11.74% | 594% | 151% |

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.36% | -1.56% | -1.58% | 0.14% | 0.64% | 0.63% | 1.61% | 1.19 | 1.16 | -2.84% | 0.46 | 0.47 | 2.47% | 6249% | 154% |
| SVT12 | -4.26% | -3.69% | -3.79% | -3.67% | -3.28% | -3.35% | -1.10% | 1.22 | 1.17 | -4.44% | 0.57 | 0.58 | 1.04% | 5911% | 144% |
| Overall | -3.31% | -2.62% | -2.68% | -1.77% | -1.32% | -1.36% | 0.25% | 1.21 | 1.16 | -3.65% | 0.52 | 0.52 | 1.68% | 6080% | 149% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.29% | -0.78% | -0.77% | -0.52% | 0.35% | 0.36% | 0.81% | 1.19 | 1.16 | -2.61% | 0.45 | 0.47 | 2.84% | 721% | 137% |
| SVT12 | -4.46% | -2.12% | -2.10% | -4.27% | -2.88% | -2.87% | -1.77% | 1.22 | 1.17 | -4.48% | 0.56 | 0.57 | 1.10% | 675% | 126% |
| Overall | -3.37% | -1.45% | -1.44% | -2.39% | -1.26% | -1.25% | -0.48% | 1.21 | 1.16 | -3.56% | 0.51 | 0.52 | 1.88% | 698% | 131% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.40% | -0.95% | -0.94% | -0.28% | 0.56% | 0.56% | 1.11% | 1.18 | 1.15 | -2.68% | 0.45 | 0.46 | 2.34% | 914% | 132% |
| SVT12 | -4.72% | -2.32% | -2.30% | -3.86% | -2.41% | -2.41% | -1.19% | 1.23 | 1.17 | -4.88% | 0.55 | 0.56 | 0.58% | 879% | 137% |
| Overall | -3.56% | -1.63% | -1.62% | -2.07% | -0.93% | -0.92% | -0.04% | 1.21 | 1.16 | -3.80% | 0.50 | 0.51 | 1.37% | 896% | 134% |

In the low QPs (Table2),

- BD-bitrate changes for AI/LDB/RA are {-8.61%, -7.34%, -7.54%} for PQ, {-4.95%, -5.74%, -6.24%} for HLG, and {-3.31%, -3.37%, -3.56%} for SVT, respectively.

- BD-binrate changes for AI/LDB/RA are {7.71%, 6.41%, 6.51%} for PQ, {3.75%, 3.32%, 3.03%} for HLG, and {-1.77%, -2.39%, -2.07%} for SVT, respectively. This indicates that VTM consumes a bit higher number of bins for the same PSNR values compared to HM at higher bitrate except SVT sequences.

- The unweighted bin to bit ratio changes for AI/LDB/RA are {6.35%, 5.36%, 3.66%} for PQ, {2.72%, 3.60%, 2.42%} for HLG, and {-3.65%, -3.56%, -3.80%} for SVT, respectively. The weighted bin to bit ratio changes for AI/LDB/RA are {17.77%, 18.30%, 17.40%} for PQ, {10.75%, 12.34%, 11.74%} for HLG and {1.68%, 1.88%, 1.37%} for SVT, respectively.

- The weighted delta binrate for AI/LDB/RA are {15.21%, 13.04%, 13.53%} for PQ, {7.30%, 7.57%, 7.91%} for HLG, and {0.25%, -0.48%, -0.04%} for SVT, respectively.

Conclusion

This contribution reports the coding performance (BD-bitrate, BD-binrate) and bin-to-bit ratio of VTM-13.0 relative to HM-16.23 for high bit depth coding under low QP setting (QP=-13,…,12 for 12-bit, QP=-33,…,-8 for 16-bit) and normal QP setting (QP=22, 27, 32, 37).

Thanks to the adoption of operation range extension tools, BD-bitrate of VTM over HM has been improved at low QP condition, especially for SVT sequences. However, there is still performance gap between normal QPs and low QPs. It is encouraged to improve coding performance of VVC for high bit-depth coding.

The weighted peak bin to bit ratio of VVC is generally higher than HEVC’s except ultra high bitrate like SVT sequences. It implies VVC CABAC engine needs to deal with bins at faster pace than HEVC. It is encouraged to study CABAC throughput for high bit-depth coding.

Reference

- VTM-13.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-13.0

- HM-16.23, https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tags/HM-16.23

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19,” JVET-N0049, Mar. 2019

- T. Tsukuba, M. Ikeda, T. Suzuki, “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding,” JVET-V0150, Apr. 2021.

- A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding,” JVET-U2018, Jan. 2021

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).