| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0116-v2 |
| --- | --- |

| Title: | CE-3 related: a different alignment position for high throughput mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

CE-3.2 proposes a high throughput mode for high bit depth high bit rate coding. In the CE-3.2 method, the bit alignment mechanism is applied after coding the last significant coefficient position in RRC where last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are first context coded then last_sig_coeff_x_suffix and last_sig_coeff_y_suffix are subsequently bypass coded. This contribution evaluates an alternative solution where the bit alignment process is performed after the context coded bins of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix and right before the bypass-coded bins of last_sig_coeff_x_suffix and last_sig_coeff_y_suffix.

Under the high bit-depth common test condition (CTC), the simulation results show negligible change in BD-bitrate, BD-binrate and Bin2Bit compared with that from CE-3.2.

Introduction

A high throughput mode for high bit depth and high bit rate coding is being proposed in CE-3.2 JVET-W0045 [1]. It is asserted that with the proposed method, all the syntax elements involved in residual coding and the transform skip residual coding, except for the last significant coefficient position in RRC, are coded through the bypass mode and the bit alignment mechanism is applied only once after coding the last significant coefficient position in RRC and at the very beginning of TSRC coding.

Figure 1 shows an example of the RRC bit-stream format of the CE-3.2 method. The coding of the last significant coefficient position includes both context- and bypass- coded bins. After that, all the sub blocks are coded through the bypass mode. The bit alignment mechanism is applied right after the coding of all the syntax elements of the last significant coefficient position, which include both context- and bypass- coded bins, and before coding all the sub blocks by using bypass coded bins.

Figure 1: CE-3.2 example in RRC

From a perspective of bit-stream syntax elements, an example of the CE-3.2 method is shown in Figure 2. The syntax elements last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are context coded, whereas the syntax elements last_sig_coeff_x_suffix and last_sig_coeff_y_suffix, dec_abs_level, coeff_sign_flag and sb_coded_flag are all bypass coded. As shown in Figure 1, the bit alignment mechanism is not applied at the boundary between context coded syntax elements and the bypass coded syntax elements.

Figure 2: syntax elements in CE-3.2 example in RRC

Alternative method

This contribution evaluates an alternative method where the bit alignment process is performed between the last context coded bin and the first bypass coded bin in RRC. This method will make the alignment right at the boundary between context coded syntax elements and bypass coded syntax elements. Figure 3 shows an example of this alternative method in RRC.

Figure 3: Alternative method in RRC

Simulation results

Comparison with CE-3.2

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [3]. The anchor is generated by CE-3.2, the test is produced by the alternative method.

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.37 | 0.37 | 0.00% | 103% | 102% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.37 | 0.37 | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.37 | 0.37 | 0.00% | 102% | 101% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.39 | 0.39 | 0.00% | 99% | 100% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 92% | 99% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 96% | 99% |
| Overall PQ | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.07 | 1.07 | 0.00% | 0.38 | 0.38 | 0.00% | 99% | 100% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.05 | 1.05 | 0.00% | 0.34 | 0.34 | 0.00% | 98% | 98% |
| HLG422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.34 | 0.34 | 0.00% | 99% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.34 | 0.34 | 0.00% | 98% | 98% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 97% | 97% |
| HLG422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 99% | 99% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.34 | 0.34 | 0.00% | 96% | 98% |
| HLG422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 99% | 98% |
| Overall HLG | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.06 | 1.06 | 0.00% | 0.35 | 0.35 | 0.00% | 99% | 98% |

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.04 | 1.04 | 0.00% | 0.29 | 0.29 | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.05 | 1.05 | 0.00% | 0.31 | 0.31 | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.04 | 1.04 | 0.00% | 0.30 | 0.30 | 0.00% | 99% | 100% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.03 | 1.03 | 0.00% | 0.29 | 0.29 | 0.00% | 99% | 98% |
| SVT12 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.05 | 1.05 | 0.00% | 0.31 | 0.31 | 0.00% | 94% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.04 | 1.04 | 0.00% | 0.30 | 0.30 | 0.00% | 96% | 97% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over CE3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BD-bitrate |  |  |  |  |  |  | BD-binrate (average, weighted) |  |  |  |  | Delta Binrate (average) |  |  |  | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.03 | 1.03 | 0.00% | 0.29 | 0.29 | 0.00% | 99% | 101% |
| SVT12 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.05 | 1.05 | 0.00% | 0.31 | 0.31 | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.04 | 1.04 | 0.00% | 0.30 | 0.30 | 0.00% | 100% | 101% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 1.04 | 1.04 | 0.00% | 0.30 | 0.30 | 0.00% | 99% | 99% |

Conclusion

This contribution evaluates an alternative method where the bit alignment process described in W0045 [1] (CE3.2 Report) is performed between the last context coded bin and the first bypass coded bin in RRC. This will make the alignment right at the boundary between context coded syntax elements and bypass coded syntax elements. The simulation results show negligible change in BD-bitrate, BD-binrate and Bin2Bit compared with that from CE-3.2.

References

[1] F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), M. Sarwer, J. Chen, Y. Ye, R. Liao (Alibaba), “CE-3.2: a high throughput mode for high bit depth and high bit rate extensions”, JVET-W0045

[2] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).