# Changes to the specification text:

# Change clause 7.3.2.22 (Sequence parameter set range extension syntax ) as follows (additions are yellow-highlighted):

- 7.3.2.22	Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| sps_high_throughput_mode_enabled_flag | u(1) |
| } |  |

# Change clause 7.3.7 (Slice header syntax ) as follows (additions are yellow-highlighted):

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_high_throughput_mode_enabled_flag ) |  |
| sh_high_throughput_mode_flag | u(1) |
| … |  |

# Change clause 7.3.11.11 (Residual coding syntax) as follows (additions are yellow-highlighted):

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = sh_high_throughput_mode_flag ? 0 : ( ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 ) |  |
| ... |  |
| if( sb_coded_flag[ xS ][ yS ] ) { |  |
| dec_abs_level[ n ] | ae(v) |
| if( updateHist && dec_abs_level[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( dec_abs_level[ n ] ) ) + sps_high_throughput_mode_enabled_flag ? 2 : 0) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| } |  |

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| inferSbCbf = 1 |  |
| RemCcbs = sh_high_throughput_flag ? 0 : ( ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 ) |  |

# In clause 7.4.3.22, add the following after the semantics of sps_persistent_rice_adaptation_enabled_flag :

- 7.4.3.22 Sequence parameter set range extension semantics

…

sps_high_throughput_mode_enabled_flag equal to 1 specifies that sh_high_throughput_mode_flag is present in slice_header( ) syntax structure referring to the SPS. sps_high_throughput_mode_enabled_flag  euqal to 0 specifies that sh_high_throughput_mode_flag is not present in slice_header( ) syntax structure referring to the SPS.

When not present, the value of sps_high_throughput_mode_enabled_flag is inferred to be equal to 0.

In clause 7.4.8, add the following

sh_high_throughput_mode_flag equal to 1 specifies that the high throughput mode is invoked for corresponding slice. sh_high_throughput_mode_flag equal to 0 specifies that the high throughput mode is not invoked for corresponding slice. When not present, the value of sh_high_throughput_mode_flag is inferred to be equal to 0.

# In clause 9.3.1, make the following changes (additions are highlighted in yellow):

- 9.3 CABAC parsing process for slice data

- 9.3.1General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.11.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

When sh_high_throughput_mode_flag is equal to 1, the request for a value of a syntax element is for the first bypass decoding syntax element in residual_ts_coding( ) either sb_coded_flag[][] or abs_remainder[], or the first bypass decoding syntax element dec_abs_level[] in residual_coding( ),  in current transform block, the alignment process prior to bypass decoding as specified in subclause 9.3.4.3.6 is invoked.

In clause 9.3.4.2.1, make the following changes (additions are highlighted in yellow):

- 9.3.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

- 9.3.4.2.1 General

Table 132 – Assignment of ctxInc to syntax elements with context coded bins

| last_sig_coeff_x_prefix | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| last_sig_coeff_y_prefix | 0..22 (clause 9.3.4.2.4) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| sb_coded_flag[ ][ ] sh_high_throughput_mode_flag == 0 | 0..6 (clause 9.3.4.2.6) | na | na | na | na | na |
| sb_coded_flag[ ][ ] sh_high_throughput_mode_flag == 1 | bypass | na | na | na | na | na |

Add new clause 9.3.4.3.6 as follows:

- 9.3.4.3.6 Alignment process prior to aligned bypass decoding

Input to this process is the variable ivlCurrRange.

Output of this process is the updated variable ivlCurrRange.

This process applies prior to the bypass decoding of syntax elements sb_coded_flag[][], or abs_remainder[ ] or dec_abs_level[].

ivlCurrRange is set equal to 256.