| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0118-v2 |
| --- | --- |

| Title: | CE related: High throughput 4:4:4 16 intra profile for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

An All-Intra codec with 4:4:4, 16-bit, and high throughput capabilities is highly desirable in high-end, high-quality content creation applications. This contribution first proposes to establish 4:4:4 16-bit High Throughput Intra Profile in the VVC operation range extensions. It further proposes to enable two coding tools in such a profile, namely the alternative derivation method of last significant coefficient position proposed in CE-1.1, and the bit-aligned full-bypass method proposed in CE-3.2.

Under the high bit-depth common test condition (CTC), the simulation results for SVT 16-bit intra are reported together with BD-bitrate, BD-binrate and Bin2Bit. These test results clearly demonstrated the improved overall performance, including both BD-bitrate and throughput, when running both tools together in high bitrate 4:4:4 16-bit Intra coding.

BD-bitrate: -0.23 % (G), -0.19 % (B), -0.20 % (R),  81 % (EncT), 74 % (DecT)

BD-binrate and Bin2Bit: -41.50% (weighted average Delta Binrate), -12.16 % (peak unweighted Bin2Bit), -41.43 % (peak weighted Bin2Bit)

Introduction

All intra coding is mostly used in content creation applications where visual signals typically are kept at high bit-depth such as 16-bit and the compression quality is usually running at very high level such as visually lossless. During the development of HEVC RExt, a 16-bit all intra profile was proposed in JCTVC-P0058 [1] and “High Throughput 4:4:4 16 Intra” profile, specified in JCTVC-R0128 [2], was adopted in the HEVC Range Extensions in the JCTVC R meeting.

A demand for high throughput was expressed in JVET-R0365 [3] as a requirement for the development of VVC high bit depth and high bit rate extensions.  It was stated in JVET-R0365 that “for such professional equipment, operational bit rate is higher and therefore, higher throughput is required to reduce the implementation complexity.” JVET-V0059 [4] also proposed a method to be used in “High Throughput 4:4:4 16 Intra” profile.

JVET-V0150 [5] reported comprehensive test results that compared VTM with HM running at various QP setting segments. When measuring the “weighted peak bin to bit ratio”, a methodology introduced in JVET-N0049 [6] for assessing entropy decoding complexity, recent test results showed that the weighted peak bin to bit ratio of VTM 13 is still higher for SVT 16-bit sequences compared with HM that was running with the HEVC “High Throughput 4:4:4 16 Intra” configuration. JVET-V0150 stated that “it implies VVC CABAC engine needs to deal with bins at faster pace than HEVC. It is also beneficial to improve CABAC throughput.”

This contribution proposes a 4:4:4 16-bit High Throughput Intra Profile that includes tools designed for high bit rate coding and high throughput capability.

Proposed method

This contribution first proposes to establish High Throughput 4:4:4 16 Intra Profile in VVC operation range extensions. Furthermore, this contribution proposes to enable both methods proposed in CE-1.1 [7] and CE-3.2 [8] in this profile. Specifically,

- CE-1.1 proposes that, for high bit-rate coding, a set of alternative coordinates of the last significant coefficient is measured and coded in reference to the bottom-right corner of zero-out transform block. This method reportedly is simple to implement and can bring a coding gain of {-0.29%, -0.28%, -0.28%} for {G, B, R} in SVT 16-bit AI tests. Accordingly, the syntax element sps_reverse_last_sig_coeff_enabled_flag is set to be 1 in the proposed High Throughput 4:4:4 16 Intra Profile, and the method proposed in CE-1.1 applies.

- CE-3.2 proposes a high throughput mode, with which all the syntax elements involved in the residual coding (including residuals not using transform), except for the last significant coefficient position in RRC, are coded as bypass-coded bins. A bit alignment mechanism is applied only once after coding the last significant coefficient position in RRC and at the very beginning of TSRC coding. As a result, this method can truly enable decoding of all the coefficients simply by using a shift register and multiple coded bins can be decoded in parallel. The simulation results showed improvements of

-37.53 % (weighted average Delta Binrate), -10.57 % (peak unweighted Bin2Bit), -37.45 % (peak weighted Bin2Bit) for SVT 16-bit AI tests. The decoding time reduction is around 25%. Accordingly, the syntax element sps_high_throughput_flag is set to be 1 in the proposed High Throughput 4:4:4 16 Intra Profile, and the method proposed in CE-3.2 applies.

Simulation results

CTC low QP test:

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [2]. QP range is {-13, -8, -3, 2, 7, 12} for 12-bit and {-33, -28, -23, -18, -13, -8} for 16-bit. The anchor is generated by VTM-13, and the test is produced by the proposed method.

SVT-16:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.23% | -0.19% | -0.20% | 81% | 74% |

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) |
| SVT16 | -0.23% | -0.19% | -0.20% | -41.67% | -41.63% | -41.63% | -41.50% | 1.16 | 1.02 | -12.16% | 0.47 | 0.28 | -41.43% |

CTC AI:

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.23% | -0.19% | -0.20% | -41.67% | -41.63% | -41.63% | -41.50% | 1.16 | 1.02 | -12.16% | 0.47 | 0.28 | -41.43% | 84% | 79% |
| SVT12 | 1.18% | 1.41% | 1.39% | -49.83% | -49.73% | -49.72% | -49.80% | 1.17 | 1.02 | -12.54% | 0.58 | 0.29 | -50.64% | 84% | 84% |
| Overall | 0.47% | 0.61% | 0.59% | -45.75% | -45.68% | -45.68% | -45.65% | 1.16 | 1.02 | -12.35% | 0.52 | 0.28 | -46.50% | 84% | 81% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 11.22% | 13.79% | 14.10% | -54.56% | -53.94% | -53.68% | -86.32% | 1.27 | 1.07 | -16.11% | 0.90 | 0.38 | -58.12% | 93% | 91% |
| PQ422 | 9.40% | 11.30% | 11.22% | -54.30% | -53.63% | -53.37% | -57.01% | 1.26 | 1.06 | -16.11% | 0.87 | 0.37 | -57.53% | 90% | 89% |
| Overall | 10.31% | 12.55% | 12.66% | -54.43% | -53.79% | -53.53% | -71.67% | 1.27 | 1.06 | -16.11% | 0.88 | 0.37 | -57.83% | 91% | 90% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 6.44% | 8.36% | 8.91% | -57.74% | -57.24% | -56.96% | -59.71% | 1.24 | 1.04 | -16.49% | 0.80 | 0.32 | -59.52% | 87% | 86% |
| HLG422 | 5.85% | 7.44% | 7.83% | -55.69% | -55.22% | -54.96% | -57.43% | 1.23 | 1.04 | -15.68% | 0.77 | 0.32 | -57.89% | 89% | 86% |
| Overall | 6.15% | 7.90% | 8.37% | -56.72% | -56.23% | -55.96% | -58.57% | 1.24 | 1.04 | -16.09% | 0.78 | 0.32 | -58.72% | 88% | 86% |

Additional lower QP test:

QP range is {-23, -21, -19, -17, -15, -13} for 12-bit and {-47, -44, -41, -38, -35, -32} for 16-bit. The anchor is generated by VTM-13, and the test is produced by the proposed method.

The purpose of this test is to show that the high throughput mode also works satisfactorily for 12-bit coding when the QP is really low or the codec is operating at high bitrates.

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.04% | -0.09% | -0.10% | -32.27% | -32.84% | -32.79% | -33.33% | 1.12 | 1.02 | -9.29% | 0.41 | 0.28 | -33.09% | 84% | 80% |
| SVT12 | -0.56% | -0.31% | -0.38% | -38.42% | -38.57% | -38.35% | -40.43% | 1.16 | 1.02 | -11.61% | 0.47 | 0.28 | -40.17% | 84% | 83% |
| Overall | -0.30% | -0.20% | -0.24% | -35.35% | -35.70% | -35.57% | -36.88% | 1.14 | 1.02 | -10.47% | 0.44 | 0.28 | -36.86% | 84% | 81% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 4.99% | 6.75% | 6.66% | -47.93% | -47.53% | -47.55% | -60.88% | 1.18 | 1.04 | -11.52% | 0.67 | 0.33 | -50.98% | 84% | 93% |
| PQ422 | 3.55% | 4.45% | 4.47% | -46.04% | -45.93% | -45.93% | -48.73% | 1.18 | 1.04 | -11.43% | 0.64 | 0.33 | -48.97% | 89% | 90% |
| Overall | 4.27% | 5.60% | 5.57% | -46.98% | -46.73% | -46.74% | -54.81% | 1.18 | 1.04 | -11.47% | 0.65 | 0.33 | -49.99% | 87% | 92% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.48% | 0.58% | 0.60% | -45.13% | -45.37% | -45.37% | -46.73% | 1.18 | 1.03 | -12.17% | 0.57 | 0.30 | -46.90% | 93% | 83% |
| HLG422 | 0.34% | 0.42% | 0.43% | -44.17% | -44.39% | -44.39% | -45.57% | 1.18 | 1.03 | -12.17% | 0.56 | 0.30 | -45.67% | 87% | 86% |
| Overall | 0.41% | 0.50% | 0.51% | -44.65% | -44.88% | -44.88% | -46.15% | 1.18 | 1.03 | -12.17% | 0.56 | 0.30 | -46.29% | 90% | 85% |

Conclusion

This contribution proposes to establish High throughput 4:4:4 16 Intra Profile in VVC operation range extensions. It is further proposed to include both methods proposed in CE-1.1 and CE-3.2 in the proposed high throughput 4:4:4 16 intra profile. Simulation results show that the BD-bitrate performance is improved and the throughput, measured by the methodology introduced in JVET-N0049 as well as by the decoding time, is also significantly improved. It is suggested to adopt the proposed profile in VVC operation range extensions.

References

[1] K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony), “AHG 5 and 18: Request for 16 bit Profiles for Range Extensions”, JCTVC-P0058

[2] K. Sharman, N. Saunders, J. Gamei (Sony), “High 4:4:4 16 Intra profile specification”, JCTVC-R0128

[3] T. Suzuki, M. Ikeda, Y. Yagasaki (Sony), T. Toma, K. Abe (Panasonic), M. Shima (Canon), “Proposals on VVC extensions for higher fidelity video”, JVET-R0365

[4] M.G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba), “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-V0059

[5] T. Tsukuba, M. Ikeda, T. Suzuki (Sony), “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding”, JVET-V0150

[6] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049

[7] F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), “CE-1.1: coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-W0046

[8] F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), M. Sarwer, J. Chen, Y. Ye, R. Liao (Alibaba), “CE-3.2: a high throughput mode for high bit depth and high bit rate extensions”, JVET-W0045

[9] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).