| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0119-v1 |
| --- | --- |

| Title: | EE2: LFNST extension with large kernel (tests 4.5, 4.6, 4.7, and 4.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the 22th JVET meeting, an LFNST extension with large kernels for large blocks was introduced in JVET-V0119 and included in EE2. Here, LFNST4, LFNST8, and LFNST16 are defined to indicate LFNST kernel sets, which are applied to 4xN/Nx4 (N≥4), 8xN/Nx8 (N≥8), and MxN (M, N≥16), respectively. In this EE test, the following two architectures with the large kernels are proposed:

Arch. A:  (LFNST4, LFNST8, LFNST16*) = (16x16, 16x48, 32x96)

Arch. B:  (LFSNT4, LFNST8*, LFNST16*) = (16x16, 32x64, 32x96)

In the above two architectures, worst case computational complexity is bounded to be 16 multiplications per sample by restricting the number of output coefficients of forward LFNST to be 16 instead of 32 for 8x8 transform block in the case of Arch. B. Specifically, four EE2 tests are performed based on the above mentioned two architectures as follows:

| # | Test |
| --- | --- |
| 4.5 | Extended LFNST with large kernel – method A |
| 4.6 | Extended LFNST with large kernel – method B |
| 4.7 | Replace LFNST in EE base with the proposed method A |
| 4.8 | Replace LFNST in EE base with the proposed method B |

Here, Test 4.5 and 4.6 extend the existing extended LFNST as ECM-1.0 while keeping LFNST index signalling, the number of LFNST sets, and encoder routines as they are. In contrast, Test 4.7 and 4.8 replace the extended LFNST by changing the three aspects as well based on JVET-V0119. The experimental results of Test 4.5-4.8 are summarized as follows:

Tool-off test results:

The tool-off test results of Test 4.5 are reportedly observed that the BD-rates are reduced by 0.04% and 0.03% with 93%/99% and 99%/99% encoding/decoding time changes for AI and RA, respectively.

The tool-off test results of Test 4.6 are reportedly observed that the BD-rates are reduced by 0.23% and 0.16% with 94%/99% and 99%/99% encoding/decoding time changes for AI and RA, respectively.

The tool-off test results of Test 4.7 are reportedly observed that the BD-rates are reduced by 0.08% and 0.04% with 97%/99% and 99%/100% encoding/decoding time changes for AI and RA, respectively.

The tool-off test results of Test 4.8 are reportedly observed that the BD-rates are reduced by 0.26% and 0.18% with 99%/99% and 99%/100% encoding/decoding time changes for AI and RA, respectively.

Tool-on test results:

The tool-on test results of Test 4.5 are reportedly observed that the BD-rates are reduced by 0.95% and 0.56% with 112%/99% and 104%/100% encoding/decoding time changes for AI and RA, respectively.

The tool-on test results of Test 4.6 are reportedly observed that the BD-rates are reduced by 1.16% and 0.68% with 115%/99% and 104%/100% encoding/decoding time changes for AI and RA, respectively.

The tool-on test results of Test 4.7 are reportedly observed that the BD-rates are reduced by 1.00% and 0.55% with 116%/98% and 104%/101% encoding/decoding time changes for AI and RA, respectively.

The tool-on test results of Test 4.8 are reportedly observed that the BD-rates are reduced by 1.21% and 0.69% with 118%/98% and 104%/100% encoding/decoding time changes for AI and RA, respectively.

Proposed architecture

In this section, all relevant architectural features included in Test 4.5-4.8 [3] are elaborated in more detail. Basically, the forward LFNST is applied to top-left low frequency region, which is called Region-Of-Interest (ROI) as in [1]. When LFNST is applied, primary-transformed coefficients that exist in the region other than ROI are zeroed out, which is not changed from the VVC standard.

ROI for LFNST16 and LFNST8

The ROI for LFNST16 is the same as that in [1], which is also depicted in Figure 1. It consists of six 4x4 sub-blocks, which are consecutive in scan order. Since the number of input samples is 96, transform matrix for forward LFNST16 can be Rx96. R is chosen to be 32 in this contribution, 32 coefficients (two 4x4 sub-blocks) are generated from forward LFNST16 accordingly, which are placed following coefficient scan order.

Figure 1  The ROI for LFNST16

The ROI for LFNST8 in Test 4.6 and 4.8 is shown in Figure 2(b), which was also introduced in [1]. Compared with Figure 2(a) that corresponds to VVC LFNST8 case, the bottom-right 4x4 sub-block is further included. The forward LFNST8 matrix can be Rx64 and R is chosen to be 32. The generated coefficients are located in the same manner as with LFNST16.

Figure 2  The ROI for LFNST8 in the case of (a) Test 4.5 and 4.7 and (b) Test 4.6 and 4.8

The configuration of LFNST sets

In Test 4.7 and 4.8, the total number of LFNST sets is 36 and the mapping from intra prediction modes to these sets is shown in Table 1, which is the same as in [1]. The LFNST sets labelled from 2 to 34 are mapped in a symmetric manner, and the 35th set is assigned to all extended wide angle modes. Each LFNST set consists of three kernels, and LFNST4, LFNST8, and LFNST16 have the same number of LFNST sets and kernels per set.

Table 1 The LFNST set mapping in Test 4.7 and 4.8

In Test 4.5 and 4.6, the 35th set dedicated to all the extended wide angle modes is not used and the 2nd set is mapped to them instead, which is the same as in ECM-1.0 and shown in Table 2. Other features regarding LFNST sets are the same as in Test 4.7 and 4.8.

Table 2 The LFNST set mapping in Test 4.5 and 4.6

The worst case computational complexity of 16 multiplications per sample

Basically, the method for maintaining the required multiplication count per sample to be at most 16 is the same as in [1]. In the case of LFNST4, LFNST zero-out is not applied, in contrast to the VVC standard where only half of forward LFNST coefficients, i.e. 8 coefficients, are generated for 4x4 and 8x8 TB. In the case of LFNST8 in Test 4.5 and 4.7, the same kind of LFNST zero-out is also disabled for 8x8 TB. However, in the case of LFNST8 in Test 4.6 and 4.8 where 32x64 matrix is applied, only half of the coefficients, i.e. 16 coefficients, are allowed in order not to violate 16 multiplications per sample. When it comes to LFNST16, LFNST zero-out is not required.

Additional context assignment for LFNST16

In LFNST index signalling in ECM-1.0, truncated unary binarization is applied and contexts for first, second and third bins are shared between LFNST4 and LFNST8. In Test 4.7 and 4.8, context assignment for LFNST index is a little different from that for extended LFNST in ECM-1.0. Specifically, these tests employ two additional contexts for LFNST16, which are used for signalling second and third bin positions, respectively.

Summary of architecture configurations of all the four EE2 tests

The Table 3 summarizes architectural configurations of all the four tests.

Table 3  Summary of architecture configurations of Test 4.5-4.8

| Test | ROI of LFNST16 | ROI of LFNST8 | LFNST set mapping | max. # of mult./sample | Ctx. assign. for LFNST idx |
| --- | --- | --- | --- | --- | --- |
| 4.5 | Fig. 1 | Fig. 2(a) | Table 2 (35-set) | 16 | Same as in ECM-1.0 |
| 4.6 | Fig. 1 | Fig. 2(b) | Table 2 (35-set) | 16 | Same as in ECM-1.0 |
| 4.7 | Fig. 1 | Fig. 2(a) | Table 1 (36-set) | 16 | Section 1.4 |
| 4.8 | Fig. 1 | Fig. 2(b) | Table 1 (36-set) | 16 | Section 1.4 |

Experimental results

The proposed methods have been implemented based on ECM-1.0 reference software according to common test conditions defined in JVET-V2017 [2].

The tool-off and tool-on test results are summarized in Table 4 and Table 5 for AI and RA, respectively.

Table 4  Summary of tool-off and tool-on test results under AI configuration

| Test | Tool-off result for AI |  |  | Tool-on result for AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | EncT | DecT | Y BD-rate | EncT | DecT |
| 4.5 | -0.04% | 93% | 99% | -0.95% | 112% | 99% |
| 4.6 | -0.23% | 94% | 99% | -1.16% | 115% | 99% |
| 4.7 | -0.08% | 97% | 99% | -1.00% | 116% | 98% |
| 4.8 | -0.26% | 99% | 99% | -1.21% | 118% | 98% |

Table 5  Summary of tool-off and too-on test results under RA configuration

| Test | Tool-off result for RA |  |  | Tool-on result for RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | EncT | DecT | Y BD-rate | EncT | DecT |
| 4.5 | -0.03% | 99% | 99% | -0.56% | 104% | 100% |
| 4.6 | -0.16% | 99% | 99% | -0.68% | 104% | 100% |
| 4.7 | -0.04% | 99% | 100% | -0.55% | 104% | 101% |
| 4.8 | -0.18% | 99% | 100% | -0.69% | 104% | 100% |

Tool-off test results

Test 4.5

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.57% | -0.88% | 93% | 98% | -0.15% | -0.32% | -0.40% | 99% | 98% |
| Class A2 | -0.18% | -0.29% | -0.40% | 93% | 100% | -0.12% | -0.15% | -0.27% | 99% | 99% |
| Class B | -0.11% | -0.22% | -0.44% | 93% | 99% | -0.03% | -0.07% | -0.04% | 99% | 100% |
| Class C | 0.38% | 0.19% | 0.07% | 93% | 99% | 0.13% | 0.24% | 0.07% | 99% | 100% |
| Class E | 0.04% | -0.37% | -0.31% | 93% | 98% |  |  |  |  |  |
| Overall | -0.04% | -0.22% | -0.37% | 93% | 99% | -0.03% | -0.05% | -0.13% | 99% | 99% |
| Class D | -0.02% | -0.46% | -0.26% | 92% | 98% | -0.05% | -0.14% | -0.49% | 99% | 100% |
| Class F | 0.31% | 0.02% | 0.03% | 98% | 101% | 0.06% | -0.34% | -0.21% | 99% | 100% |

Test 4.6

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.73% | -1.02% | 95% | 99% | -0.20% | -0.32% | -0.45% | 100% | 99% |
| Class A2 | -0.22% | -0.38% | -0.48% | 95% | 100% | -0.18% | -0.17% | -0.36% | 99% | 98% |
| Class B | -0.26% | -0.56% | -0.71% | 94% | 99% | -0.16% | -0.28% | -0.29% | 99% | 100% |
| Class C | -0.05% | -0.15% | -0.25% | 95% | 99% | -0.11% | -0.12% | -0.15% | 100% | 100% |
| Class E | -0.16% | -0.42% | -0.38% | 94% | 98% |  |  |  |  |  |
| Overall | -0.23% | -0.44% | -0.57% | 94% | 99% | -0.16% | -0.22% | -0.30% | 99% | 99% |
| Class D | -0.20% | -0.60% | -0.52% | 94% | 99% | -0.14% | -0.20% | -0.27% | 100% | 100% |
| Class F | -0.03% | -0.11% | -0.09% | 98% | 101% | -0.06% | -0.33% | -0.24% | 99% | 100% |

Test 4.7

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.70% | -0.94% | 98% | 100% | -0.18% | -0.30% | -0.27% | 99% | 100% |
| Class A2 | -0.24% | -0.37% | -0.37% | 97% | 100% | -0.13% | -0.11% | -0.26% | 99% | 100% |
| Class B | -0.13% | -0.25% | -0.43% | 96% | 99% | -0.06% | -0.05% | -0.06% | 99% | 100% |
| Class C | 0.36% | 0.16% | 0.11% | 96% | 98% | 0.14% | 0.09% | -0.06% | 99% | 100% |
| Class E | -0.02% | -0.32% | -0.30% | 97% | 98% |  |  |  |  |  |
| Overall | -0.08% | -0.27% | -0.36% | 97% | 99% | -0.04% | -0.08% | -0.14% | 99% | 100% |
| Class D | -0.03% | -0.33% | -0.15% | 93% | 97% | -0.04% | 0.00% | -0.27% | 100% | 100% |
| Class F | 0.32% | 0.15% | 0.07% | 102% | 100% | 0.05% | -0.26% | -0.14% | 99% | 100% |

Test 4.8

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -0.69% | -1.10% | 100% | 100% | -0.25% | -0.38% | -0.39% | 100% | 101% |
| Class A2 | -0.26% | -0.43% | -0.47% | 99% | 100% | -0.20% | -0.09% | -0.24% | 100% | 101% |
| Class B | -0.29% | -0.50% | -0.68% | 98% | 99% | -0.15% | -0.30% | -0.25% | 99% | 100% |
| Class C | -0.06% | -0.06% | -0.15% | 98% | 98% | -0.13% | 0.07% | -0.08% | 99% | 100% |
| Class E | -0.18% | -0.65% | -0.51% | 100% | 99% |  |  |  |  |  |
| Overall | -0.26% | -0.45% | -0.57% | 99% | 99% | -0.18% | -0.18% | -0.23% | 99% | 100% |
| Class D | -0.19% | -0.51% | -0.29% | 96% | 97% | -0.10% | -0.03% | -0.03% | 100% | 100% |
| Class F | -0.05% | 0.07% | -0.10% | 103% | 99% | -0.15% | -0.40% | -0.37% | 99% | 100% |

Tool-on test results

Test 4.5

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.14% | -0.93% | -0.95% | 112% | 100% | -0.69% | -0.80% | -0.76% | 105% | 100% |
| Class A2 | -0.75% | -0.91% | -1.28% | 113% | 100% | -0.46% | -0.42% | -0.46% | 104% | 100% |
| Class B | -0.86% | -0.97% | -1.04% | 112% | 99% | -0.56% | -0.62% | -0.49% | 104% | 100% |
| Class C | -1.01% | -0.84% | -0.95% | 113% | 101% | -0.53% | -0.52% | -0.65% | 103% | 100% |
| Class E | -1.04% | -1.32% | -1.03% | 112% | 97% |  |  |  |  |  |
| Overall | -0.95% | -0.98% | -1.04% | 112% | 99% | -0.56% | -0.59% | -0.58% | 104% | 100% |
| Class D | -0.93% | -1.16% | -1.25% | 113% | 100% | -0.39% | -0.60% | -0.62% | 103% | 100% |
| Class F | -0.85% | -0.72% | -0.62% | 105% | 101% | -0.44% | -0.26% | -0.13% | 103% | 100% |

Test 4.6

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | -1.09% | -1.14% | 114% | 100% | -0.74% | -0.59% | -0.85% | 106% | 101% |
| Class A2 | -0.81% | -0.94% | -1.30% | 116% | 100% | -0.52% | -0.33% | -0.51% | 104% | 100% |
| Class B | -1.04% | -1.27% | -1.27% | 115% | 100% | -0.67% | -0.81% | -0.76% | 104% | 100% |
| Class C | -1.44% | -1.06% | -1.19% | 116% | 99% | -0.77% | -0.83% | -0.77% | 103% | 100% |
| Class E | -1.30% | -1.53% | -1.41% | 115% | 98% |  |  |  |  |  |
| Overall | -1.16% | -1.18% | -1.26% | 115% | 99% | -0.68% | -0.68% | -0.73% | 104% | 100% |
| Class D | -1.10% | -1.39% | -1.49% | 116% | 102% | -0.51% | -0.77% | -0.75% | 103% | 100% |
| Class F | -1.11% | -0.77% | -0.72% | 106% | 100% | -0.64% | -0.67% | -0.65% | 103% | 99% |

Test 4.7

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.22% | -0.91% | -1.05% | 117% | 100% | -0.69% | -0.82% | -0.79% | 105% | 101% |
| Class A2 | -0.80% | -0.86% | -1.16% | 117% | 100% | -0.46% | -0.37% | -0.50% | 103% | 102% |
| Class B | -0.89% | -0.85% | -0.95% | 116% | 99% | -0.55% | -0.69% | -0.50% | 103% | 100% |
| Class C | -1.07% | -0.63% | -0.79% | 115% | 97% | -0.51% | -0.45% | -0.54% | 103% | 100% |
| Class E | -1.10% | -1.11% | -0.88% | 118% | 95% |  |  |  |  |  |
| Overall | -1.00% | -0.85% | -0.95% | 116% | 98% | -0.55% | -0.59% | -0.57% | 104% | 101% |
| Class D | -0.91% | -1.15% | -1.22% | 112% | 97% | -0.44% | -0.74% | -0.80% | 103% | 101% |
| Class F | -0.87% | -0.65% | -0.55% | 110% | 98% | -0.43% | -0.32% | -0.26% | 103% | 99% |

Test 4.8

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.29% | -0.97% | -1.25% | 118% | 100% | -0.76% | -0.88% | -0.87% | 105% | 101% |
| Class A2 | -0.86% | -0.89% | -1.30% | 120% | 101% | -0.53% | -0.39% | -0.50% | 104% | 101% |
| Class B | -1.08% | -1.19% | -1.27% | 118% | 100% | -0.66% | -0.82% | -0.78% | 104% | 100% |
| Class C | -1.48% | -0.95% | -1.05% | 115% | 95% | -0.78% | -0.76% | -0.86% | 104% | 99% |
| Class E | -1.33% | -1.34% | -1.19% | 119% | 94% |  |  |  |  |  |
| Overall | -1.21% | -1.07% | -1.21% | 118% | 98% | -0.69% | -0.73% | -0.76% | 104% | 100% |
| Class D | -1.08% | -1.29% | -1.34% | 114% | 95% | -0.48% | -0.76% | -0.61% | 104% | 100% |
| Class F | -1.17% | -0.69% | -0.54% | 110% | 97% | -0.67% | -0.56% | -0.39% | 104% | 99% |

Test results of combination with extended intra MTS of EE2 Test 4.1

The experimental results of the combination tests, i.e. Test 4.7+Test 4.1 and Test 4.8+Test 4.1, are summarized in Table 6.

Table 6  Summary of test results of combination with extended intra MTS of Test 4.1

| Test | Cfg. | Tool-off test results |  |  | Tool-on test results |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | EncT | DecT | Y BD-rate | EncT | DecT |
| Test 4.7 + Test 4.1 | AI | -0.48% | 113% | 99% | -1.36% | 135% | 98% |
|  | RA | -0.24% | 106% | 100% | -0.72% | 113% | 100% |
| Test 4.8 + Test 4.1 | AI | -0.66% | 114% | 100% | -1.56% | 139% | 98% |
|  | RA | -0.38% | 105% | 101% | -0.87% | 113% | 100% |

According to Table 4, Table 5, and Table 6, it can be deduced that the proposed LFNST architectures is additive to the extended intra MTS in terms of coding performance.

Test 4.7 with the extended intra MTS (Test 4.1)

• Tool-off test results

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -0.37% | -0.77% | 118% | 100% | -0.34% | -0.35% | -0.46% | 104% | 101% |
| Class A2 | -0.75% | -0.24% | -0.32% | 114% | 101% | -0.32% | -0.24% | -0.32% | 107% | 100% |
| Class B | -0.53% | -0.03% | -0.16% | 113% | 98% | -0.26% | 0.02% | 0.12% | 105% | 100% |
| Class C | -0.01% | 0.28% | 0.13% | 112% | 100% | -0.09% | 0.25% | -0.03% | 107% | 100% |
| Class E | -0.38% | -0.24% | -0.14% | 110% | 99% |  |  |  |  |  |
| Overall | -0.48% | -0.09% | -0.22% | 113% | 99% | -0.24% | -0.05% | -0.12% | 106% | 100% |
| Class D | -0.45% | -0.28% | 0.04% | 108% | 98% | -0.22% | -0.34% | -0.49% | 106% | 102% |
| Class F | 0.04% | 0.04% | 0.08% | 103% | 100% | -0.11% | -0.38% | -0.07% | 103% | 101% |

• Tool-on test results

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.57% | -0.68% | -0.96% | 137% | 100% | -0.83% | -0.68% | -0.84% | 112% | 101% |
| Class A2 | -1.28% | -0.69% | -1.02% | 135% | 100% | -0.61% | -0.25% | -0.51% | 114% | 100% |
| Class B | -1.24% | -0.59% | -0.72% | 136% | 99% | -0.73% | -0.61% | -0.44% | 112% | 100% |
| Class C | -1.39% | -0.51% | -0.56% | 134% | 97% | -0.69% | -0.43% | -0.44% | 116% | 100% |
| Class E | -1.37% | -1.03% | -0.84% | 135% | 95% |  |  |  |  |  |
| Overall | -1.36% | -0.68% | -0.79% | 135% | 98% | -0.72% | -0.50% | -0.53% | 113% | 100% |
| Class D | -1.27% | -0.98% | -0.86% | 129% | 98% | -0.57% | -0.40% | -0.29% | 114% | 106% |
| Class F | -1.03% | -0.63% | -0.48% | 113% | 98% | -0.57% | -0.55% | -0.46% | 111% | 101% |

Test 4.8 with the extended intra MTS (Test 4.1)

• Tool-off test results

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 CTC |  |  |  |  | Over ECM-1.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.46% | -0.88% | 119% | 100% | -0.44% | -0.29% | -0.61% | 103% | 101% |
| Class A2 | -0.78% | -0.37% | -0.38% | 115% | 101% | -0.36% | -0.27% | -0.40% | 106% | 101% |
| Class B | -0.67% | -0.35% | -0.51% | 114% | 99% | -0.38% | -0.23% | -0.17% | 105% | 101% |
| Class C | -0.45% | 0.13% | -0.08% | 112% | 99% | -0.35% | -0.01% | -0.18% | 107% | 100% |
| Class E | -0.55% | -0.41% | -0.45% | 111% | 99% |  |  |  |  |  |
| Overall | -0.66% | -0.28% | -0.44% | 114% | 100% | -0.38% | -0.19% | -0.31% | 105% | 101% |
| Class D | -0.61% | -0.38% | -0.34% | 109% | 99% | -0.37% | -0.52% | -0.14% | 107% | 102% |
| Class F | -0.24% | -0.03% | 0.04% | 103% | 99% | -0.38% | -0.26% | -0.14% | 103% | 100% |

• Tool-on test results

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2 Tool-on test Anchor |  |  |  |  | Over EE2 Tool-on test Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.63% | -0.71% | -1.13% | 140% | 100% | -0.91% | -0.72% | -0.84% | 112% | 101% |
| Class A2 | -1.34% | -0.75% | -1.05% | 139% | 100% | -0.70% | -0.38% | -0.54% | 114% | 100% |
| Class B | -1.42% | -0.92% | -1.04% | 139% | 99% | -0.86% | -0.91% | -0.54% | 113% | 100% |
| Class C | -1.83% | -0.74% | -0.91% | 136% | 96% | -0.96% | -0.71% | -0.84% | 115% | 98% |
| Class E | -1.60% | -1.27% | -1.04% | 139% | 96% |  |  |  |  |  |
| Overall | -1.56% | -0.87% | -1.03% | 139% | 98% | -0.87% | -0.71% | -0.68% | 113% | 100% |
| Class D | -1.46% | -1.04% | -1.28% | 132% | 96% | -0.69% | -0.78% | -0.65% | 112% | 100% |
| Class F | -1.34% | -0.65% | -0.40% | 113% | 96% | -0.84% | -0.42% | -0.26% | 111% | 98% |

Conclusion

In this contribution, the experimental results of the EE2 test 4.5-4.8 are provided along with the explanation of their configurations. According to both tool-off and tool-on test results, the LFNST extensions based on large kernels reportedly showed substantial gains relative to ECM-1.0 and tool-on test anchor, respectively particularly in high resolution sequences which may be more mainly targeted for next standards. Furthermore, worst case computational complexity for all the tests is no larger than 16 multiplications per sample that is twice than that of the VVC LFNST, which enables simpler implementations. It is recommended to integrate one of the proposed LFNST architectures into next version of ECM software.

References

- M. Koo, J. Zhao, J. Lim and S. Kim, “AHG12: LFNST extension with large kernel”, JVET-V0119, 22nd Meeting, by teleconference, 20–28 Apr. 2021.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, 22nd Meeting, by teleconference, 20–28 Apr. 2021.

- V. Seregin, J. Chen, S. Esenlik, F. Le Leannec, L. Li, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capabilities (EE2)”, JVET-V2024, 22nd Meeting, by teleconference, 20–28 Apr. 2021

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).