| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0121 |
| --- | --- |

| Title: | AHG8: On signalling of sps_ts_residual_coding_rice_present_in_sh_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Wei Chen, Che-Wei Kuo, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, chenwei06, cheweikuo, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution asserted that the signalling of sps_ts_residual_coding_rice_present_in_sh_flag is redundant when the value of sps_transform_skip_enabled_flag is equal to 0. Therefore, the following two options are proposed:

- Condition the presence of the sps_ts_residual_coding_rice_present_in_sh_flag such that it is present only when sps_transform_skip_enabled_flag is equal to 1.

- Add a requirement of bitstream conformance such that the value of sps_ts_residual_coding_rice_present_in_sh_flag shall be equal to 0 when sps_transform_skip_enabled_flag is equal to 0.

A patch for the proposed changes relative to VTM-13.0 is attached.

# Proposal

In last JVET meeting, a slice based Rice parameter selection scheme was adopted from [1] for the transform skip residual coding (TSRC) to improve the efficiency of high bit-depth coding. In the adopted method, one control flag is signaled in sequence parameter set (SPS) to indicate the Rice parameter used for the TSRC and allowed to be adjusted for each slice separately. The signalling of Rice parameter used for the TSRC is currently needed only when transform skip is enabled. Based on this understanding, the signalling of sps_ts_residual_coding_rice_present_in_sh_flag in the SPS is only needed when transform skip present in the SPS (i.e., when the value of sps_transform_skip_enabled_flag is equal to 1). Therefore, sps_ts_residual_coding_rice_present_in_sh_flag should be signalled only when sps_transform_skip_enabled_flag is equal to 1. In order to resolve the issue, the following two options are proposed:

- Condition the presence of the sps_ts_residual_coding_rice_present_in_sh_flag such that it is present only when sps_transform_skip_enabled_flag is equal to 1.

- Add a requirement of bitstream conformance such that the value of sps_ts_residual_coding_rice_present_in_sh_flag shall be equal to 0 when sps_transform_skip_enabled_flag is equal to 0.

The changes to the VVC operation range extensions draft [2] are highlighted below.

## Option-1

# 7.3.2.22	Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| … |  |
| } |  |

## Option-2

#### 7.4.3.22 Sequence parameter set range extension semantics

…

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx_minus1 may be present in slice_header( ) syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx_minus1 is not present in slice_header( ) syntax structures referring to the SPS. When sps_ts_residual_coding_rice_present_in_sh_flag is not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of sps_ts_residual_coding_rice_present_in_sh_flag shall be equal to 0 when sps_transform_skip_enabled_flag is equal to 0.

# References

- H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo and X. Wang, “ CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-V0054, 22th meeting, by teleconference, 20–28 Apr. 2021.

- F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy and Y.-K. Wang, “ VVC operation range extensions (Draft 3)”, JVET-V2005, 22th meeting, by teleconference, 20–28 Apr. 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).