| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0122-v1 |
| --- | --- |

| Title: | EE2-related: On spatial MV propagation and neighboring template block access for template matching and multi-pass DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Cheng-Teh Hsieh, Han Huang, Vadim Seregin, Wei-Jung Chien, Yao-Jen Chang, Zhi Zhang, Yan Zhang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1 (858) 845-4257 {chunchic, thsieh, hanhuang, vseregin, wchien, yjchang, zhizhang, yzh, martak}@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposed a bounding-box approach to limit the number of reference samples fetched from reference pictures within a deterministic size and a replacement method for current block templates by using their corresponding prediction samples through inferred motion vectors for template matching (TM) and multi-pass decoder-side motion vector refinement (MPDMVR) in ECM-1.0. These two were tested on top of ECM-1.0 with the common test condition (JVET-V2024). Partial experimental results of Class B sequences with random access test condition are summarized, as follows.

- Test 1 (Bounding-box approach): 0.87 % (Y), 0.92% (U), 0.97% (V), 91% (EncT), 83%(DecT)

- Test 2 (Test 1+Template replacement): 1.77% (Y), 1.99% (U), 1.98% (V), 92% (EncT), 81% (DecT)

- Test 3 (TM off+MPDMVR off): 3.39% (Y), 3.56% (U), 3.53% (V), 81% (EncT), 60% (DecT)

# Introduction

In JVET-V meeting, coding tools beyond VVC were studied in the context of Exploration Experiments 2 (EE2, JVET-U2024) and new software ECM-1.0 was established as a common platform for further exploration activity. Template matching (TM) and multi-pass DMVR (MPDMVR) are two of them added to motion compensation (MC) that have been studied in the last meeting cycle in JVET-V0120. From the implementation of TM and MPDMVR in ECM-1.0, known major hardware implementation issues yet to be addressed include:

- spatial motion vector propagation by TM and MPDMVR;

- access to neighboring reconstruction samples by TM.

Motion field reconstruction, reference sample fetching and motion compensation are usually arranged in different pipeline stages. Ideally, the motion compensation of each inter block is designed to be done within a given cycle time for decoders to meet a target frame rate. Since reference sample fetching requires accessing external memory and the latency could be much longer than MC, reference samples are usually prefetched to maximize the utilization of MC module (that is to minimize the chance for MC module to stop and wait for loading required reference samples). As Figure 1 (case 1) depicts, every inter block can be processed one after one without waiting and keep the utilization of MC as high as possible.

However, spatial MV prediction in ECM-1.0 allows an inter block to infer refined MVs from its causal neighboring blocks. Motion field reconstruction and reference sample fetching of each inter block have to wait until MC of its causal neighboring blocks are (partially) done in order to get their refinement delta vectors, resulting in reduced decoding throughput due to a failure of aforementioned pipeline hardware design. As shown in Figure 1 (case 2), CU2 can start to reconstruct its motion information and fetch required reference sample only when the MC is done for CU1. Then, the MC module has to stop and wait until CU2 finishes loading reference samples, resulting in accumulated idle time across CUs that could impact the decoding throughput and fail to meet the target frame rate.

A work-around solution is to fetch more reference samples to cover all possible ones that may be used during MC; however, due to spatial MV prediction, the refinement delta vectors (∆1 and ∆2 in Figure 1) may carry over to subsequent blocks and accumulate across CUs, resulting in an indeterminable size of reference samples to be fetched from reference pictures. Another work-around solution is to stop propagating refinement delta vector information to subsequent blocks in the current picture, at the risk of losing MV prediction efficiency.

Accessing neighboring reconstructed samples by TM to generate current block templates also introduces MC latency issue in hardware decoder. In VTM, inter blocks can be reconstructed independently regardless of neighboring block reconstruction (unlike intra block which has to wait until neighboring blocks reconstruction is done to perform its intra prediction). However, neighboring block reconstruction becomes inevitable for inter blocks because TM requires accessing these neighboring reconstructed samples to form its current block template, resulting in reduced decoding throughput due to the inter-dependency between the current block and its neighboring ones.

- MV propagation with and without MV refinement.

# Proposed method

The proposed method to solve aforementioned issues is a two-fold process:

- imposing a bounding-box constraint on the search area of TM and MPDMVR to prevent refined MVs from going beyond a pre-defined area centered at the MVs that are inferable during motion field reconstruction stage;

- replacing current block templates by using the prediction blocks generated by using the inferred MV information from neighboring blocks.

## Constrained search area

A search area constraint is imposed on TM and MPDMVR of ECM-1.0 to limit the number of reference samples fetched from reference pictures within a deterministic bounding box. Two sets of motion information are maintained for each inter block: one of which is propagated MV that is the same as that of ECM-1.0, the other of which is unrefined MV that is used to determine the prefetching area for MC and the bounding box for TM and MPDMVR process. The MV difference between the two is constrained to be within a certain search range (SR), e.g., 8 pixels per direction as in ECM-1.0. The unrefined MV is derived without accessing to any propagated MV. Therefore, it can be determined during the motion field reconstruction stage. Reference samples prefetching can be done based on the unrefined MV. The maximum number of prefetched reference samples is determined by the block size, interpolation filter taps, and SR. During the TM and MPDMVR process, the search area is constrained, so the refined MV cannot go beyond [v0-(SR,SR), v0+(SR,SR)] wherein v0 is the unrefined MV. In such way, the reference samples to be prefetched are always known in advance.

In Figure 2, CU2 infers a MV from its neighboring block (CU1) which is with a propagated refined MV (v1 or v0+∆1) and a unrefined MV (v0), where ∆1 is the delta MV derived by TM and/or MPDMVR. In ECM-1.0, the search process starts from p2 within a search area [p2-(SR,SR), p2+(SR,SR)] pointed to by v1, while the proposed method shifts the search area to [p1-(SR,SR), p1+(SR,SR)] pointed to by v0. Because v0 is unrefined and already known during reference sample fetching stages, the number of required reference samples to be fetched becomes deterministic. Although the sum of delta MVs (i.e., ∆1, ∆2, …) is accumulated across CUs during MC stage, it is always bounded within [-(SR,SR), +(SR,SR)] due to the search area constraint.

Since the MV competition of TM AMVP mode would violate the proposed method (i.e., two AMVP candidates may point to two very different reference blocks, with another two after MVD is added), this proposal disables TM AMVP mode.

- Example of constrained search area of TM and MPDMVR.

## Prediction-based current block templates

Current block templates are replaced by using the prediction blocks generated by using the refined MV information inferred from neighboring blocks. For example, the current block templates of CU2 are generated by using v1 (same as v0+∆1) shown in Figure 2 (b). The interpolation filter used to generate such prediction template is the same as the one used for the final MC.

# Experimental results

The described methods are implemented on top ECM-1.0 software and are tested based on the common test condition as specified in the EE2 document (JVET-V2024). The test results are summarized in the below tables.

- Test 1: Constrained search area.

- Test 2: Test 1 + prediction-based current block templates.

- Test 3: TM off + MPDMVR off (for comparison purpose).

- Performance results of Test 1 (Constrained search area) relative to ECM-1.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.87% | 0.92% | 0.97% | 91% | 83% |
| Class C | 0.71% | 0.74% | 0.71% | 93% | 89% |
| Class E | x | x | x | x | x |
| Overall |  |  |  |  |  |
| Class D | 0.63% | 0.54% | 0.67% | 96% | 91% |
| Class F | 0.28% | 0.30% | 0.29% | 96% | 95% |

- Performance results of Test 2 (Test 1 + prediction-based current block templates) relative to ECM-1.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.77% | 1.99% | 1.98% | 92% | 81% |
| Class C | 1.81% | 2.07% | 2.13% | 95% | 85% |
| Class E | x | x | x | x | x |
| Overall |  |  |  |  |  |
| Class D | 1.67% | 1.95% | 2.07% | 97% | 87% |
| Class F | 0.66% | 0.81% | 0.83% | 98% | 93% |

- Performance results of Test 3 (TM off + MPDMVR off) relative to ECM-1.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.76% | 3.53% | 4.31% | 78% | 65% |
| Class A2 | 5.58% | 5.39% | 5.51% | 79% | 58% |
| Class B | 3.39% | 3.56% | 3.53% | 81% | 60% |
| Class C | 3.66% | 3.89% | 4.15% | 82% | 59% |
| Class E | x | x | x | x | x |
| Overall | 3.97% | 4.01% | 4.25% | 80% | 60% |
| Class D | 4.02% | 4.02% | 4.39% | 83% | 56% |
| Class F | 1.37% | 1.59% | 1.62% | 87% | 74% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).