| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0123-v2 |
| --- | --- |

| Title: | EE2-related: Fusion for template-based intra mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao, Nan Hu, Vadim Seregin, Marta Karczewicz Yang Wang, Kai Zhang, Li Zhang | Tel: Email: | kemicao@qti.qualcomm.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Incorporated, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a fusion of intra modes derived from the template-based intra mode derivation (TIMD) method tested in EE2-2.1. Two modes are derived using TIMD method and are fused with the weights computed from the sum of absolute transformed differences, which is used as the cost function.

This method was implemented on top EE2-2.1 software, it reportedly provides Y, U, V BD rate reduction with encoder and decoder runtime as follows:

AI: {-0.47%, -0.34%, -0.37%} runtime: {124%, 111%}

RA:{-0.19%, -0.08%, -0.23%} runtime: {105%, 102%}

# Introduction

In JVET-V0098, a template-based intra mode derivation (TIMD) method was proposed, it is tested in EE2. For each intra mode in MPMs, the sum of absolute transformed differences (SATD) between prediction and reconstruction samples of the template region shown in Figure 1 is computed and the intra mode with the smallest SATD cost is chosen.

Figure 1. Template and its reference samples used in TIMD

# Proposed Method

Instead of selecting the only one mode with the smallest SATD cost, this contribution proposes to choose the first two modes with the smallest SATD costs for the intra modes derived using TIMD method and then fuse them with the weights, and such weighted intra prediction is used to code the current CU.

The costs of the two selected modes are compared with a threshold, in the test the cost factor of 2 is applied as follows:

costMode2 < 2*costMode1.

If this condition is true, the fusion is applied, otherwise the only mode1 is used.

Weights of the modes are computed from their SATD costs as follows:

weight1 = costMode2/(costMode1+ costMode2)

weight2 = 1 - weight1

# Results

The proposed method is implemented on top of EE2.1 software [2], ECM-1.0 is used as an anchor in the tests. In the below tables, test results for the proposed TIMD fusion method and TIMD method tested in EE2.1 are provided.

Table 1 Test results of TIMD fusion

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.05% | -0.14% | 125% | 105% |
| Class A2 | -0.38% | -0.36% | -0.32% | 125% | 112% |
| Class B | -0.50% | -0.36% | -0.46% | 124% | 110% |
| Class C | -0.46% | -0.32% | -0.40% | 123% | 116% |
| Class E | -0.77% | -0.57% | -0.46% | 123% | 111% |
| Overall | -0.47% | -0.34% | -0.37% | 124% | 111% |
| Class D | -0.40% | -0.33% | -0.14% | 122% | 117% |
| Class F | -0.25% | -0.09% | -0.06% | 112% | 107% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.01% | -0.16% | 105% | 103% |
| Class A2 | -0.17% | -0.16% | -0.23% | 105% | 102% |
| Class B | -0.22% | -0.11% | -0.17% | 104% | 101% |
| Class C | -0.20% | -0.04% | -0.37% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.08% | -0.23% | 105% | 102% |
| Class D | -0.18% | -0.23% | -0.12% | 105% | 103% |
| Class F | -0.12% | -0.41% | -0.14% | 105% | 102% |

Table 2 Test results of TIMD method (EE2.1)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.06% | -0.20% | 124% | 103% |
| Class A2 | -0.28% | -0.25% | -0.28% | 124% | 110% |
| Class B | -0.38% | -0.27% | -0.37% | 123% | 110% |
| Class C | -0.38% | -0.25% | -0.26% | 123% | 116% |
| Class E | -0.61% | -0.54% | -0.47% | 124% | 114% |
| Overall | -0.37% | -0.27% | -0.32% | 123% | 111% |
| Class D | -0.33% | -0.36% | -0.20% | 121% | 113% |
| Class F | -0.19% | 0.01% | -0.09% | 113% | 112% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.03% | -0.09% | 105% | 101% |
| Class A2 | -0.18% | -0.08% | -0.17% | 104% | 101% |
| Class B | -0.19% | -0.01% | -0.13% | 105% | 102% |
| Class C | -0.15% | 0.00% | -0.31% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.01% | -0.18% | 105% | 102% |
| Class D | -0.17% | 0.25% | 0.07% | 105% | 104% |
| Class F | -0.12% | -0.22% | -0.06% | 106% | 103% |

# Conclusions

This document presents a method to fuse modes derived by the TIMD method, this approach achieves 0.1% additional gain in AI configuration compared to the TIMD method without the fusion with the similar to EE2-2.1 runtimes.

# Reference

[1] Y. Wang, L. Zhang, K. Zhang, Z. Deng and N. Zhang, “EE2-related: Template-based intra mode derivation using MPMs”, JVET-V0098, online, Apr. 2021.

[2] V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, M. Winken, J. Ström, X. Xiu, K. Zhang “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)” JVET-V2024, Online, Apr. 2021.

[3] https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tree/ECM-1.0

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).