| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0124-v2 |
| --- | --- |

| Title: | EE2-related: Template-based intra most probable modes sorting |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao, Nan Hu, Vadim Seregin, Marta Karczewicz | Tel: Email: | kemicao@qti.qualcomm.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to apply sorting method on intra most probable modes (MPMs) list. For each intra mode in the list, the sum of absolute transformed differences between prediction and reconstruction samples of a template is computed, based on which MPMs are sorted.

This method was implemented on top ECM-1.0, it reportedly provides Y, U, V BD rate reduction with encoder and decoder runtime as follows:

AI: {-0.26%, -0.30%, -0.32%} runtime: {103%, 122%}

RA: {-0.11%, -0.03%, -0.11%}

# Introduction

MPMs are utilized to code intra prediction mode. In the current ECM framework, MPMs are consisted of intra prediction modes from the neighbors of the current CU and the corresponding derived modes. If the current block uses one of the MPM modes, the index of the chosen intra prediction mode from the MPM list is signalled in a bitstream.

# Proposed Method

Since the index of the chosen MPM mode is signalled in a bitstream, order of the modes in the MPM list has an impact on coding performance.

In JVET-V0098, a template-based intra mode derivation (TIMD) method was proposed, it is tested in EE2. For each intra mode in the MPM list, the sum of absolute transformed differences (SATD) between prediction and reconstruction samples of the template region shown in Figure 1 is computed and the intra mode with the smallest SATD cost is chosen.

Instead of deriving the TIMD intra mode, this proposal utilizes SATD cost to sort intra modes in the ascending order in the MPM list, including the secondary MPM modes, starting after the planar mode, thus the planar mode is always kept as the first mode in the list as in ECM/VTM.

Figure 1. Template and its reference samples used in TIMD

# Results

The proposed method is implemented on top of ECM-1.0 software [3]. Table below shows the test result of MPMs sorting with ECM-1.0 used as an anchor.

Table 1 Test result of the proposed MPM sorting

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.16% | -0.28% | 104% | 113% |
| Class A2 | -0.21% | -0.33% | -0.36% | 106% | 124% |
| Class B | -0.17% | -0.16% | -0.26% | 102% | 119% |
| Class C | -0.23% | -0.26% | -0.30% | 102% | 129% |
| Class E | -0.57% | -0.68% | -0.45% | 101% | 123% |
| Overall | -0.26% | -0.30% | -0.32% | 103% | 122% |
| Class D | -0.29% | -0.32% | -0.30% | 102% | 129% |
| Class F | -0.08% | -0.13% | -0.18% | 100% | 119% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.03% | -0.15% | 102% | 102% |
| Class A2 | -0.12% | -0.09% | -0.27% | #NUM! | #NUM! |
| Class B | -0.11% | 0.03% | 0.02% | 100% | 100% |
| Class C | -0.13% | -0.07% | -0.14% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.03% | -0.11% | #NUM! | #NUM! |
| Class D | -0.13% | -0.32% | -0.16% | 101% | 102% |
| Class F | -0.06% | -0.18% | -0.19% | 101% | 101% |

# Reference

[1] Y. Wang, L. Zhang, K. Zhang, Z. Deng and N. Zhang, “EE2-related: Template-based intra mode derivation using MPMs”, JVET-V0098, online, Apr. 2021

[2] V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, M. Winken, J. Ström, X. Xiu, K. Zhang “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)” JVET-V2024, Online, Apr. 2021.

[3] https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tree/ECM-1.0

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).