| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 Jul. 2021 | Document: JVET-W0127-v3 |
| --- | --- |

| Title: | Ali266: an optimized VVC software encoder implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | S. Fang, J. Guo, Z. Huang, J. Liu, S. Xu, L. Yu, J. Chen, R.-L. Liao, Y. Ye | Tel: Email: | yan.ye@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the performance of Ali266, an optimized software VVC encoder developed by Alibaba. Two sets of content are used to test the Ali266 encoding performance and speed: the JVET CTC content, including class A1, class A2, class B and class C; and internal video-on-demand (VOD) content, including classes 1920x1080, 1080x1920, 1280x720, 720x1280. For the JVET CTC content, Ali266 slow preset is compared to VTM-13.0, and achieves BD rates for {Y, U, V, YUV} of {10.95%, -6.11%, -4.50%, 7.11%} under the Random Access configuration with an average speedup factor of 227 times over VTM. For the internal VOD content, Ali266 fast preset is compared to the open source HEVC software encoder x265 medium preset, and achieves BD rates for {Y, U, V, YUV} of {-32.87%, -46.07%, -42.70%, -35.35%} under the Random Access configuration. Ali266 fast preset is aimed at real-time VVC encoding, and currently achieves average encoding speeds of 19.3 fps for classes 1920x1080/1080x1920 and 37.7 fps for classes 1280x720/720x1080. Based on such data, it is our conclusion that Ali266 is a VVC encoder capable of real-time encoding 720p sequences, and it is suitable for deployment in the real world in the near future.

Introduction

In previous meetings, several optimized software VVC encoder implementations have been presented to the JVET. In JVET-T0099 [1], Fraunhofer HHI released the source code for its optimized VVC encoder (VVenC) and decoder (VVdeC) on GitHub. The initial VVenC v0.1.0.0 provided four predefined quality/speed presets, depending on the chosen quality/speed preset, the PSNR-based YUV BD-rates were between -11.0% and -40.6% compared to HM without QP adaptation for HD and UHD test sequences, and the MS-SSIM-based YUV BD-rates were between -15.4% and -42.1% compared to HM with QP adaptation. In addition, VVenC provided speedups of between 22x and 270x over VTM-10.0. After months of development, the current version of VVenC is v1.0.0 [2], which provides five predefined quality/speed presets, depending on the chosen quality/speed preset, the PSNR-based YUV BD-rates were between -7.9% and -45.1% compared to HM without QP adaptation for HD and UHD test sequences, and the MS-SSIM-based YUV BD-rates were between -12.4% and -46.9% compared to HM with QP adaptation. In addition, VVenC provided speedups of between 12x and 1042x over VTM-12.0. In JVET-V0127 [3], Bytedance presented a software VVC encoder, namely BVC, which provided two predefined quality/speed presets, including a fast preset and a slow preset. With the fast preset, the BD-rate gain over HM was approx. 34.13% on average, and a speedup of 17 times and 131 times over HM and VTM, respectively, was observed. With the slow preset, the BD-rate gain over HM was approx. 41.63% on average, and a speedup of 8x times over VTM could be achieved.

In this contribution, we present Ali266, a software VVC encoder implementation developed by Alibaba aimed at achieving real-time encoding speed.

In order to speed up the encoding speed, a variety of fast encoding algorithms suitable for VVC encoder were employed. Since most of the time-consuming modules such as motion estimation and motion compensation apply the same operations on multiple data, SIMD instructions were widely used to reduce the required clock cycles. Modern platforms are based on multi-core architecture, such as 4 cores, 8 cores, or even more, Ali266 supports multi-threaded encoding at the picture level, at the module level, as well as wavefront parallel processing. In addition, software architecture was redesigned to improve memory access efficiency and overcome performance bottlenecks.

In order to improve encoding performance for real-world applications, some high-level implementations were employed in Ali266, such as motion-compensated temporal filtering (MCTF), adaptive GOP size determination, scene cut detection, screen content detection, etc. Further, quantization control algorithms such as CU-tree, adaptive QP offset determination, chroma QP offset determination, etc., were employed and optimized to improve subjective video quality.

Bitstreams encoded by Ali266 conform to VVC standard and can be decoded by VTM-11.0 reference software decoder.

Simulation results

Performance and speed test: JVET CTC content

The encoding performance of Ali266 slow preset focuses on class A1, A2, B, and C with random access encoding as defined in the JVET common test conditions (CTC) [4], which is compared to VTM-13.0. YUV-PSNR is a weighted average with weighting factors of 6, 1 and 1 for the Y-PSNR, U-PSNR and V-PSNR respectively.

The PSNR BD-rates of Ali266 over VTM-13.0 is shown in Table 1. Here positive number represents bit-rate increase at the same PSNR with respect to the VTM-13.0 anchor, while negative number represents bit-rate saving.

As a good trade-off between encoder runtime and PSNR BD-rate performance, it could be seen that with the slow preset, the BD-rate loss over VTM-13.0 is only 7.11%, with an average speedup of more than 227x.

Table 1: Encoding performance of the Ali266 encoder @ slow preset

| Class | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to VTM |  |  |  | Speedup |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs. VTM |
| A1 | 12.82 | -2.81 | 1.88 | 8.55 | 317 |
| A2 | 15.34 | -4.10 | -2.08 | 11.08 | 288 |
| B | 9.07 | -8.56 | -8.64 | 5.18 | 239 |
| C | 8.60 | -7.02 | -5.93 | 5.47 | 138 |
| Overall | 10.95 | -6.11 | -4.50 | 7.11 | 227 |

Performance and speed test: internal VOD content

The encoding performance of Ali266 fast preset focuses on 720p and 1080p sequences in both landscape and portrait modes. These two kinds of resolution are widely used for the real-world VOD content. We selected the open source HEVC software encoder x265 with the medium preset as anchor.

The PSNR BD-rates of Ali266 fast preset over x265 medium preset is shown in Table 2. With the fast preset, the BD-rate gain over x265 is approx. 35.35%. Currently, Ali266 achieves average encoding speeds of 19.99 and 18.61 fps for classes 1920x1080 and 1080x1920, and 38.77 and 36.66 fps for classes 1280x720 and 720x1280, respectively. Especially, the average encoding speed reaches 37.72 fps for 720p sequences. Therefore, it is our conclusion that software VVC encoder is capable of real-time encoding 720p sequences, and is suitable for deployment in the real world in the near future.

Since modern platforms are based on multi-core architecture, Ali266 supports multi-threaded encoding. To further evaluate the impact of multiple threads, we tested the encoding speed under different number of threads, including 1, 4, and 12, and the speedup factor compared to 1 thread is shown in Figure 1.

Table 2: Encoding performance of the Ali266 encoder @ fast preset

| Class | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to x265 @ medium |  |  |  | Speed |
|  | Y (%) | U (%) | V (%) | YUV (%) | fps |
| 1920x1080 | -32.37 | -46.22 | -42.19 | -34.99 | 19.99 |
| 1080x1920 | -35.30 | -43.99 | -39.85 | -36.58 | 18.61 |
| 1280x720 | -31.43 | -48.34 | -46.09 | -35.12 | 38.77 |
| 720x1280 | -32.36 | -45.72 | -42.68 | -34.73 | 36.66 |
| Overall | -32.87 | -46.07 | -42.70 | -35.35 | -- |

Figure 1. The speedup of the Ali266 encoder @ fast preset under different number of threads.

Conclusion

This contribution demonstrates Ali266’s encoding performance and speed for the JVET CTC content and internal VOD content. With the slow preset of Ali266, the average YUV-PSNR-based BD-rate over VTM-13.0 is 7.11%, and the average speedup factor achieves 227 times over VTM. With the fast preset, the average YUV-PSNR-based BD-rate over x265 with the medium preset is -35.35%, and the average encoding speed of 720p sequences achieves 37.72 fps. The simulation results show that Ali266 is capable of real-time encoding for 720p sequences. We plan to continue to improve Ali266’s encoding speed for a wider range of video content, and to make it suitable for deployment in the near future.

References

- A. Wiechowski el al., “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations,” JVET-T0099,  Joint Video Experts Team (JVET), October 2020.

- VVenC software repository. Available online: https://github.com/fraunhoferhhi/vvenc.

- Jing Cui el al., “An optimized VVC encoder implementation”, JVET-V0127, Joint Video Experts Team (JVET), April 2021.

- F. Bossen el al., “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Joint Video Experts Team (JVET), October 2020.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).