| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0129-v2 |
| --- | --- |

| Title: | AHG10: Using original samples for ALF optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Dmytro Rusanovskyy, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VTM-13.0, motion compensated temporal pre-filtering (MCTF) is used for random access (RA) configuration and samples filtered by MCTF are used for sample adaptive offset (SAO) and adaptive loop filter (ALF) optimizations in common test condition (CTC). It is configurable to use original samples for SAO and ALF optimization as JVET-V0095. However, in HM and ECM, filtered samples are used for SAO and bilateral filter optimization and original ones are used for ALF in CTC. To align different reference softwares, this contribution proposes to use original samples only for ALF optimization and enable it in CTC for VTM. The proposed method was tested under SDR, HDR, and high bit depth/high bit rate test conditions, the results of the informal subjective evaluation are reported as well.

# Introduction

In VTM-13.0, when MCTF is enabled for RA configuration, filtered samples by MCTF are used for sample adaptive offset (SAO) and adaptive loop filter (ALF) optimizations in common test condition (CTC). As JVET-V0095, it is configurable for VTM to use the original samples SAO and ALF parameters estimation. However, in HM and ECM, filtered samples are used for SAO and bilateral filter optimization in CTC. To align VTM with HM and ECM, this contribution proposes to use original samples only for ALF optimization. Simulation results with SDR, HDR, and high bit depth/high bit rate testing conditions are reported. The results of the informal subjective evaluation are reported as well. It is also suggested to enable the option in CTC for VTM.

# Simulation Results

## VTM-13.0 SDR

### Using original samples for SAO and ALF optimization

This option can be enabled in VTM-13.0.

Anchor: VTM-13.0 under common test condition (CTC) defined in JVET-T2010.

Test: Anchor + AlfSaoTrueOrg = 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.13% | -0.12% | 101% | 100% |
| Class A2 | -0.02% | 0.04% | 0.09% | 100% | 100% |
| Class B | -0.11% | 0.05% | -0.15% | 101% | 102% |
| Class C | -0.54% | -0.29% | -0.45% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.08% | -0.18% | 100% | 101% |
| Class D | -0.81% | -0.82% | -0.66% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

### Using original samples for only ALF optimization

Anchor: VTM-13.0 under common test condition (CTC) defined in JVET-T2010.

Test: Anchor + AlfSaoTrueOrg = 1 + using filtered sample for SAO optimization.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.07% | 0.10% | 104% | 103% |
| Class A2 | -0.07% | 0.05% | 0.16% | 103% | 103% |
| Class B | -0.16% | 0.14% | 0.03% | 103% | 107% |
| Class C | -0.51% | -0.28% | -0.52% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.03% | -0.08% | 102% | 104% |
| Class D | -0.86% | -0.61% | -0.68% | 98% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

## VTM-13.0 HDR

### Using original samples for SAO and ALF optimization

This option can be enabled in VTM-13.0.

Anchor: VTM-13.0 under CTC defined in JVET-V2011.

Test: Anchor + AlfSaoTrueOrg = 1

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | -0.16% | -0.15% | -0.04% | 0.41% | -0.15% | -0.15% | 0.40% | 100% | 99% |
| Class H2 |  |  |  |  |  | 0.02% | 0.08% | 0.60% | 100% | 100% |
| Overall | 0.00% | -0.16% | -0.15% | -0.04% | 0.41% | -0.09% | -0.07% | 0.48% | 100% | 99% |

### Using original samples for only ALF optimization

Anchor: VTM-13.0 under CTC defined in JVET-V2011.

Test: Anchor + AlfSaoTrueOrg = 1 + using filtered sample for SAO optimization.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.04% | -0.17% | -0.19% | 0.00% | 0.26% | -0.18% | -0.12% | 0.29% | 100% | 99% |
| Class H2 |  |  |  |  |  | -0.03% | 0.10% | 0.48% | 100% | 100% |
| Overall | 0.04% | -0.17% | -0.19% | 0.00% | 0.26% | -0.12% | -0.04% | 0.36% | 100% | 100% |

## VTM-13.0 high bit depth and high bit rate coding (Normal QP)

### Using original samples for SAO and ALF optimization

This option can be enabled in VTM-13.0.

Anchor: VTM-13.0 under CTC defined in JVET-U2018.

Test: Anchor + AlfSaoTrueOrg = 1

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.22% | -0.23% | -0.19% | 0.24% | 0.44% | -0.17% | 0.19% | 0.36% | 102% | 101% |
| Class H2 |  |  |  |  |  | 0.01% | 0.41% | 0.59% | 104% | 104% |
| Overall | 0.22% | -0.23% | -0.19% | 0.24% | 0.44% | -0.08% | 0.30% | 0.47% | 103% | 102% |

### Using original samples for only ALF optimization

Anchor: VTM-13.0 under CTC defined in JVET-U2018.

Test: Anchor + AlfSaoTrueOrg = 1 + using filtered sample for SAO optimization.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.19% | -0.22% | -0.19% | 0.05% | 0.54% | -0.16% | 0.07% | 0.49% | 101% | 99% |
| Class H2 |  |  |  |  |  | -0.13% | 0.01% | 0.36% | 101% | 100% |
| Overall | 0.19% | -0.22% | -0.19% | 0.05% | 0.54% | -0.14% | 0.04% | 0.43% | 101% | 100% |

# Subjective evaluation

Reconstructed SDR sequences were checked, and no subjective differences between the anchor and the tests were observed.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.