| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0132-v3 |
| --- | --- |

| Title: | AHG11: 1.5x/2.0x Upsample method for NN-Based Super-Resolution Post-Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Tel: Email: | yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution presents several experimental results of NN-based super-resolution post-filter. In VVC, RPR (Reference Picture Re-sampling) has been introduced, and in several 4K sequences with low bit-rate, it was reported that there are some coding gains by changing the resolution of the whole sequence. In this experiment, the performance of NN-based filters with different network structures are compared. As shown in experimental results, the use of interpolation for scaling in feature space can reduce the number of parameters more than 30% without significant loss of performance.

Introduction

In this contribution, we report the results of testing the performance of NN-based super-resolution post-filters with different network components.

In order to improve the coding efficiency, video sequences are sometimes coded in a reduced size and then expanded by the post filter. For example, there are RPR and NN-based super-resolution filters.

NN-based super-resolution post-filters that use Complex Convolution for enlargement tend to increase the number of parameters when the scaling factor is not a power of two or is not an integer. Since an increase in the number of parameters leads to a decrease in coding efficiency, a method to perform magnification with a higher degree of freedom of the scaling factor using fewer parameters is necessary.

Therefore, we measured the performance of using bicubic interpolation in feature space as a simpler method for enlargement with various magnifications and compared it with the enlargement method by convolution. Since bicubic interpolation does not require any parameters and can reduce the size of the network parameters.

Experiments

The super-resolution 1.5x post-filter used in this experiment is based on a simplified ESRGAN [1] as shown in Figure 1. The loss function was the L1 norm and part of the BVI-DVC test set [2] was used as training sequences. The patch size and batch size of training was 96 (for HR) and 32, respectively. The number of training iterations was 100,000.

Figure 1  Base structure of tested neural networks

We compared the performance of the Upsample layer in the network shown in Figure 1 with the configurations shown in Figure 2.

Figure 2  Configurations of upsample layers

- Configuration A: 2x, PixelShuffle

- Configuration B: 2x, Bicubic interpolation(2.0x)

- Configuration C: 1.5x, PixelShuffle + Bicubic interpolation(0.75x)

- Configuration D: 1.5x, Bicubic interpolation(1.5x)

The number of parameters for configurations A-D were 483597, 335884, 483597, and 335884, respectively.

Network Information in Training Stage

| Mandatory | GPU type | NVIDIA GeForce RTX 2080Ti |
| --- | --- | --- |
|  | Framework | PyTorch 1.7.1 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 8 |
|  | Batch size | 32 |
|  | Training time | 3 hours |
|  | Training data information | BVI-DVC (partial) |
|  | Training confiurations for generating compressed training data |  |
| Optional | Learning rate | 2e-4 (halfed every 20k iterations) |
|  | Number of iterations | 100k |
|  | Patch size | 96x96 |
|  | Optimizer | ADAM |
|  | Loss function | L1-loss |

Network Information in Inference Stage

| Mandatory | GPU Type | NVIDIA GeForce RTX 2080Ti |
| --- | --- | --- |
|  | Framework | PyTorch 1.7.1 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | (A)(C) 483597, (B)(D) 335884 |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | (A)(C) 1.84, (B)(D) 1.28 |
|  | MAC per pixels | (approx.) (A)122k, (B) 85k, (C) 122k, (D) 85k |

Tables 1 to 4 show the BD-rate(qp:22-42) performance of Configurations A-D. Each reference data is RPR with the same scale.

Table 1  Experimental results of configuration A (2.0x, PixelShuffle)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0&JVET-V0056 RPR20 QP=22,...,42 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.81% | -2.83% | -4.56% | 100% | 100% |
| Class A2 | -6.31% | -4.49% | -3.25% | 100% | 100% |

Table 2  Experimental results of configuration B (2.0x, Bicubic interpolation)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0&JVET-V0056 RPR20 QP=22,...,42 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.64% | -2.13% | -4.67% | 100% | 100% |
| Class A2 | -6.03% | -4.68% | -2.08% | 100% | 100% |

Table 3  Experimental results of configuration C (1.5x, PixelShuffle+Bicubic interpolation)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0&JVET-V0056 QP=27,...,47 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.43% | 1.76% | -0.12% | 100% | 100% |
| Class A2 | -3.33% | 0.95% | 2.74% | 100% | 100% |

Table 4  Experimental results of configuration D (1.5x, Bicubic interpolation)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0&JVET-V0056 QP=27,...,47 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.71% | 0.67% | -1.62% | 100% | 100% |
| Class A2 | -3.39% | 0.99% | 3.03% | 100% | 100% |

There are some differences in performance depending on the item, but the results are generally similar. By using a interpolation function, which does not require any parameters, instead of PixelShuffle more than 30% of the parameters (147713 parameters) were reduced.

Table 5 shows a comparison of the run time of each NN-based filter. In 2.0x cases, configuration B requires 26% more time than the conventional method, while configuration D takes 24% less time than C. The change of the runtime between 2.0x and 1.5x is smaller for the method using interpolation.

Table 5 Runtime for NN-filter inference

| 2.0x |  |  | ratio |
| --- | --- | --- | --- |
|  | cfg.A | cfg.B | B/A |
| ClassA1 | 24844.64 | 31412.11 | 126.43% |
| ClassA2 | 25048.91 | 31684.13 | 126.49% |
| total | 49893.55 | 63096.24 | 126.46% |
| 1.5x |  |  | ratio |
|  | cfg.C | cfg.D | D/C |
| ClassA1 | 43814.08 | 33613.53 | 76.72% |
| ClassA2 | 44130.41 | 33822.71 | 76.64% |
| total | 87944.49 | 67436.24 | 76.68% |
|  | ratio | ratio |  |
|  | C/A | D/B |  |
| ClassA1 | 176.35% | 107.01% |  |
| ClassA2 | 176.18% | 106.75% |  |
| total | 176.26% | 106.88% |  |

References

[1] X. Wang, K. Yu, S. Wu, J. Gu, Y. Liu, C. Dong, Y. Qiao, and C. Change Loy, “ESRGAN: Enhanced super-resolution generative adversarial networks,” in Proceedings of the European Conference on Computer Vision (ECCV) workshops, 2018.

[2] Di Ma, Fan Zhang and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).