| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0133-v3 |
| --- | --- |

| Title: | Constrained RASL encoding for bitstream switching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Christian Bartnik Adam Wieckowski Karsten Suehring Yago Sanchez Benjamin Bross | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Document JVET-V0060 presented a constrained RASL encoding method that allows for resolution or quality switching in HTTP streaming with open GOP coding structures while avoiding the severe drift artefacts occurring when using unconstrained open GOP encoding for such switching.

The presented constraints entail disabling BDOF, PROF, DVMR and CCLM for RASL pictures as well as restricting the choice of the collocated reference pictures to pictures not preceding the associated CRA picture. It is reported that the constrained RASL encoding method retains most of the unconstrained open GOP coding gains with BD-rate gains of up to -8.57% (64 pictures RAP period) relative to closed GOP coding.

This document presents updated results based on the publicly available VVenC version 0.3.1 and proposes to add a GCI flag to VVC to clearly specify and indicate the necessary constraints and serve as a conformance point for related systems to allow for interoperability.

In response to comments received for JVET-V0060, a note was added to the proposal explaining the constraints intent.

Based on comments received during Track review, revision 1 of this document adds a condition wrt sh_slice_type to the originally proposed constraint (Option 1) to allow intra coded slices of RASL pictures to use CCLM and further adds an alternative constraint (Option 2) that leaves details of the constrained encoding method to the encoder and only ensures suitability of a bitstream for open GOP bitstream switching. In addition, a discussion regarding less-strict variants of the proposed constraint is provided.

Revision 2 of this document adds an SEI message solution as Option 3 to indicate the proposed constraint.

# Introduction

Document JVET-V0060 presented a constrained RASL encoding method that allows for resolution or quality switching in HTTP streaming with open GOP coding structures while avoiding the severe drift artefacts occurring when using unconstrained open GOP encoding for such switching. The following paragraphs give an introduction to open GOP coding in HTTP streaming.

VVC features reference picture resampling (RPR) which was primarily investigated in a conversational use case with LD configuration [1] for the purpose of avoiding sending bitrate-costly Intra coded pictures for resolution changes. This contribution asserts that RPR with additional constraints can be beneficial for streaming applications and proposes signalling to indicate the required constraints and provide a conformance point for related systems such as MPEG-DASH or ISOBMFF.

HTTP adaptive streaming is a major path of video distribution and is based on download of content segments. The content segments are typically offered in multiple variants, referred to as representations, and may differ in terms of bitrate or coded resolution, e.g., 1Mbps@720p, 2Mbps@1080p, 5Mbps@2160p. End clients may respond to bandwidth fluctuations with up- or down switching between content variants in order to achieve continuous playout under constrained network conditions. It is known that open GOP coding structures using RASL pictures provide substantial coding efficiency benefit. Our experiments have shown to provide up to -9,22% BD-rate gain for open GOP vs closed GOP coding with short intra periods (64 pictures) as in live streaming and -2,35% BD-rate gain for longer intra periods (256 pictures) as typical for VOD streaming using the HHI-Berlin test sequence set [2] in CTC RA configuration with VVenC v0.3.1 (medium preset) and resolutions of 720p, 1080p and 2160p.

When open GOP structures are used and clients perform bitstream switching, different reference pictures are used for RASL pictures on decoder-side compared to the encoder side. For instance, in bitrate switching the decoder-side may encounter a deviation of sample values in some reference pictures, while in resolution switching, it may even be required to perform RPR when decoding the concatenated client-side bitstream segment if a RASL picture has a different size than reference pictures of a previous segment. Lack of RPR functionality would result in discontinuous playout from dropping of RASL pictures which is why coding for streaming with earlier standards H.264/AVC and HEVC had to resort to less efficient closed-GOP structures (using RADL or Trailing only). Open GOP RASL pictures at switching points are decoded with drift compared to the encoder side and unconstrained open GOP switching leads to substantial propagating artefacts in the RASL pictures from deviated sample values as well motion information, the latter usually causing very noticeable artefacts.

When resolution switching is supposed to be possible in the client-side concatenated bitstream, the RPR related constraints of DMVR, PROF and BDOF as well as (SB)TMVP (no collocated reference picture prior to associated CRA in decoding order) need to be active for the RASL pictures (even though they are unnecessary on encoder-side). For this, the encoder has to indicate that RPR and resolution change are allowed (sps_ref_pic_resampling_enabled_flag and sps_res_change_in_clvs_allowed_flag equal to 1) and refrains from using said tools for the RASL pictures by disabling them in the picture header of the respective RASL pictures and selecting collocated reference pictures accordingly. These measures preclude all drift in the motion information of the RASL pictures.

# Problem Description

Our experiments have shown that Cross Component Linear Model (CCLM) also introduces very noticeable artefacts in the color components of reconstructed samples when performing bitrate or resolution switching in a streaming scenario. Figure 1 shows per component PSNR of test sequence NeptuneFountain2 when performing open GOP bitrate switching at every IRAP between QP22 and QP32 at 2160p. It is evident that the objective quality of the color components of RASL pictures suffers from severe switching drift.

Figure 1 Per-component PSNR of open GOP bitrate switching with sequence NeptuneFountain2 when RASL pictures adhere to RPR constraints (DMVR, PROF, BDOF disabled, no collocated refpic preceding CRA).

Figure 2 shows the picture with POC 420 of the above bitrate switching example with visible color component artefacts towards the high-motion area at the bottom of the picture. It should be noted that similar artefacts occur when switching between resolutions.

Figure 2: Switching drift from CCLM in color component samples in frame 420 of the Figure 1 example.

CCLM allows to intra-predict the chroma components of a block from the respective luma component using a linear model wherein model parameters are derived from the reconstructed luma sample values. The linear model transforms subsampled luma-samples rec’L into chroma prediction through P(i, j) = a ⋅ rec’L(i, j) + b, where the parameters a and b are derived from neighboring luma and chroma samples as a = ( Yl – Ys ) / ( Xl – Xs) and b = Ys – a ⋅ Xs. with Xl and Xs denoting the average of the two largest and the two smallest neighboring samples, respectively, and Yl and Ys denoting the average of the corresponding chroma sample pairs, respectively.

Due to the parameter derivation process only accounting for the extrema of the neighboring samples values, the process is prone to extensive drift even in case of single-sample drift outliers in the neighboring blocks. Also, because of the linear model, if a is large, the luma drift may be amplified in the chroma domain. In context of bitstream switching in streaming, the drift artefacts may only occur in RASL pictures, i.e. motion-compensated inter-predicted pictures. If the encoder decides to use CCLM, i.e. an intra prediction mode, this would usually be due to a lack of an appropriate motion compensated predictor, implying a region of high temporal activity. In such regions the expected reconstruction drift for open GOP switching is expected to be high, contributing even more to the discussed extensive drift.

Unfortunately, CCLM can only be switched off on SPS level (sps_cclm_enabled_flag) for the entire CLVS which comes at substantial coding efficiency losses of roughly 3.43% BR-rate for CTC RA which mostly stems from the Intra coded pictures which use CCLM for roughly half of the CUs (as measured in CTC AI configuration) [3].

# Proposal

## Description

In order to avoid the above-described drift artefacts while limiting the coding efficiency penalty, RASL picture encoding can be constrained as follows:

- DMVR, PROF, BDOF are disabled by setting ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag in the picture header equal to 1,

- no RASL picture in OlsInScope uses a collocated reference picture preceding its associated CRA picture in decoding order,

- CCLM is disabled by disallowing CUs with cclm_mode_flag equal to 1.

Disabling CCLM through encoder-side constraints only for the drift-affected RASL pictures results in a negligible coding penalty. Applying the constraints above allows open GOP resolution switching while retaining most of the open GOP coding gains compared to closed GOP coding with -8.57% BD-rate for short intra period (64 pictures) and -1,98% BD-rate for longer intra period (256 pictures) under the same conditions as described above. Table 1 summarizes coding efficiency performance of open GOP coding and the proposed constrained open GOP coding over closed GOP coding for various intra periods and resolutions using the HHI-Berlin test sequence set in CTC RA configuration with VVenC v0.3.1 (medium preset) and resolutions of 720p, 1080p and 2160p. Note that from version 0.3 on, VVenC natively features the presented constrained RASL encoding method through command line parameter “--RPR 3”.

Table 1: Coding efficiency results for open GOP and constrained open GOP coding over closed GOP.

| Res. | IRAP Period | Anchor: closed GOP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Open GOP |  |  | Constrained open GOP |  |  |
|  |  | Y | U | V | Y | U | V |
| 2160p | 64 | -8.71% | -10.7% | -10.3% | -8.07% | -9.53% | -8.83% |
| 1080p |  | -9.12% | -11.1% | -10.7% | -8.60% | -9.57% | -8.73% |
| 720p |  | -9.38% | -11.3% | -10.8% | -8.87% | -9.93% | -9.08% |
| Overall |  | -9.07% | -11.1% | -10.6% | -8.51% | -9.68% | -8.88% |
|  |  | -9,22% |  |  | -8,57% |  |  |
| 2160p | 128 | -4.26% | -4.92% | -4.64% | -3.82% | -4.22% | -3.88% |
| 1080p |  | -4.52% | -4.91% | -4.63% | -4.11% | -4.00% | -3.58% |
| 720p |  | -4.68% | -4.90% | -4.75% | -4.24% | -4.00% | -3.76% |
| Overall |  | -4.49% | -4.91% | -4.67% | -4.06% | -4.07% | -3.74% |
|  |  | -4,49% |  |  | -4,02% |  |  |
| 2160p | 256 | -2.23% | -2.43% | -2.26% | -1.91% | -1.96% | -1.74% |
| 1080p |  | -2.40% | -2.32% | -2.14% | -2.05% | -1.67% | -1.41% |
| 720p |  | -2.49% | -2.31% | -2.30% | -2.10% | -1.70% | -1.72% |
| Overall |  | -2.38% | -2.40% | -2.23% | -2.25% | -2.04% | -1.62% |
|  |  | -2,35% |  |  | -1,98% |  |  |

The presented constrained encoding method is able to avoid severe drift artefacts from bitrate or resolution switching. Figure 3 shows per-component PSNR for the example of Figure 1 when RASL pictures are encoded with RPR constraints and without CCLM usage.

Figure 3: Per-component PSNR of open GOP bitrate switching with sequence NeptuneFountain2 when RASL pictures adhere to RPR constraints and do not use CCLM.

In general, using the presented method, the drift affected RASL pictures exhibit a gradual quality transition between variants as evident from the following Figure 4 that shows YUV-PSNR for all RASL pictures averaged over all sequences in the test set, switching points and QPs for up- and down-switching between resolution pairs (720p, 1080p) and (1080p, 2160p).

Figure 4:Quality transition of the presented constrained encoding method for open GOP resolution switching averaged over sequences, switching points, QPs and resolution pairs.

## Discussion

In Track review, it was commented that RASL pictures without reference pictures preceding their associated CRA would not need to adhere to RPR constraints. The issue with such a relaxed constraint variant is that in this case, BDOF and PROF would lead to derivation of incorrect MVs for those RASL pictures when their reference pictures are affected from sample drift. Further, in case of DMVR, this MV drift even propagates and accumulates until the associated CRA. As MV drift inevitably leads to noticeable artefacts,  it is beneficial to preclude use of RPR for those pictures even when using only CRA and RASL pictures as reference that would otherwise not require RPR constraints.

An option to relax the proposed constraint at the cost of allowing MV drift in individual RASL pictures but still precluding MV drift propagation until the associated CRA picture is to limit the constraint to RASL pictures with ph_non_ref_pic_flag equal 0. However, in light of the fact that these pictures typically belong to the highest temporal sublayer with minimal contribution to the bitrate and the proposed constrained already retains >90% of the unconstrained open GOP BR-rate gains, the simple and strict constraint variant as originally proposed was asserted to be a preferable trade-off.

## Proposed Specification changes

## Option 1

It is proposed to add a flag to the general constraint information (GCI) that indicates that the constraints described in Section 3.1 apply to all RASL pictures in scope (DCI: full bitstream, VPS: CVS, SPS: CLVS). The following highlighted changes to the GCI syntax and semantics are proposed:

| general_constraints_info( ) { | Descriptor |
| --- | --- |
| gci_present_flag | u(1) |
| if( gci_present_flag ) { |  |
| […] |  |
| gci_num_reserved_bits | u(8) |
| if( gci_num_reserved_bits > 0 ) { |  |
| gci_constrained_rasl_encoding_flag | u(1) |
| for( i = 0; i < gci_num_reserved_bits – 1; i++ ) |  |
| gci_reserved_zero_bit[ i ] | u(1) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

gci_num_reserved_bits specifies the number of the reserved GCI bits. The value of gci_num_reserved_bits shall be equal to 0 or 1 in bitstreams conforming to this version of this Specification. Other values of gci_num_reserved_bits are reserved for future use by ITU-T | ISO/IEC. Although the value of gci_num_reserved_bits is required to be equal to 0 or 1 in this version of this Specification, decoders conforming to this version of this Specification shall allow the value of gci_num_reserved_bits greater than 1 to appear in the syntax and shall ignore the values of all the gci_reserved_zero_bit[ i ] syntax elements when gci_num_reserved_bits is greater than 1.

gci_constrained_rasl_encoding_flag equal to 1 specifies that all RASL pictures in OlsInScope shall have ph_bdof_disabled_flag, ph_dmvr_disabled_flag, ph_prof_disabled_flag equal to 1 in the respective PHs, and no CU of a RASL picture in a slice with sh_slice_type equal to 0 (B) or 1 (P) shall have cclm_mode_flag equal to 1, and no RASL picture in OlsInScope shall use a collocated reference picture that precedes its associated CRA picture in decoding order. gci_constrained_rasl_encoding_flag equal to 0 does not impose a constraint.

NOTE –  When bitstreams have gci_constrained_rasl_encoding_flag equal to 1, the associated constraints allow for open GOP bitstream switching at IRAP pictures (e.g. varying resolution or quality of the bitstream) in application scenarios such as adaptive streaming with limited decoder-side drift of the reconstructed sample values of RASL pictures which can otherwise exhibit severe visually noticable artefacts when gci_constrained_rasl_encoding_flag equal to 0.

## Option 2

Alternatively, it is proposed to add a flag to the general constraint information (GCI) that indicates suitability for bitstream switching wherein the detailed constraints described in Section 3.1 are put into a note as an example. The following highlighted changes to the GCI semantics are relative to Option 1 proposal:

gci_constrained_rasl_encoding_flag equal to 1 specifies that RASL pictures of the bitstream are encoded in a way that allows seamless playout when performing open GOP bitstream switching to the bitstream at IRAP pictures. gci_constrained_rasl_encoding_flag equal to 0 does not impose a constraint.

NOTE –  The constraint associated with gci_constrained_rasl_encoding_flag equal to 1 can be fulfilled by ensuring that all RASL pictures in OlsInScope have ph_bdof_disabled_flag, ph_dmvr_disabled_flag, ph_prof_disabled_flag equal to 1 in the respective PHs, and no CU of a RASL picture in a slice with sh_slice_type equal to 0 (B) or 1 (P) has cclm_mode_flag equal to 1, and no RASL picture in OlsInScope uses a collocated reference picture that precedes its associated CRA picture in decoding order.

## Option 3

It is proposed to add the following constrained RASL encoding indication SEI message to VVC that indicates that the constraints described in Section 3.1 apply to all RASL pictures in the CVS.

4	Abbreivations

[...]

CREI	Constrained RASL Encoding Indication

[...]

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| […] |  |
| else if( payloadType = = 207 ) |  |
| constrained_rasl_encoding_indication( payloadSize ) |  |

D.2.2	General SEI payload syntax

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| […] | […] |
| Constrained RASL encoding indication | The CVS containing the CREI SEI message and up to but not including the next CVS, in decoding order. |

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 200 to 202, inclusive, 204 to 207, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 19, 45, 129, 133, 137, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 200 to 207, inclusive.

D.8.1	Constrained RASL encoding indication SEI message syntax

| constrained_rasl_encoding_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

D.8.2	Constrained RASL encoding indication SEI message semantics

The presence of the constrained RASL encoding indication (CREI) SEI message in a CVS indicates that a set of encoding constraints as described below applies for all RASL pictures in the CVS.

NOTE –  In applications such as adaptive streaming services, when this set of encoding constraints applies, it is possible to perform bitstream switching at CRA pictures with associated RASL pictures (i.e., the so-called open-GOP random access point pictures) with limited drift for the reconstructed sample values of the RASL pictures, which can otherwise exhibit visually noticable or annoying artefacts when the RASL pictures are encoded without applying the set of constraints.

When a CREI SEI message is present for any picture of an AU of a CVS, a CREI SEI message shall be present for the first picture of the CVSS AU. The CREI SEI message persists in decoding order from the current AU until the end of the CVS.

The presence of the CREI SEI message indicates that the following conditions all apply for each RASL picture in the CVS:

–	The PH syntax structure has ph_dmvr_disabled_flag equal to 1.

–	No CU in a slice with sh_slice_type equal to 0 (B) or 1 (P) has cclm_mode_flag equal to 1.

–	No collocated reference picture precedes the CRA picture associated with the RASL picture in decoding order.

# References

D. Luo, V. Seregin, W. Wan. “Description of Core Experiment 1 (CE1): Reference picture resampling filters“, doc. JVET-Q2021 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 15th meeting: July 2019

B. Bross et al., "AHG4: Multiformat Berlin Test Sequences", JVET-Q0791, JVET Brussels Meeting, Jan 2020, online: https://jvet-experts.org/doc_end_user/current_document.php?id=9637

W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu. “JVET AHG report: Tool reporting procedure and testing (AHG13)“, doc.  JVET-T0013 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)