| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0135-v2 |
| --- | --- |

| Title: | Cross-check on JVET-W0118: CE related: High throughput 4:4:4 16 intra profile for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba | Tel: Email: | Takeshi.Tsukuba@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results for the JVET-W0118 (CE related: High throughput 4:4:4 16 intra profile for VVC) [1] , which proposes to 1) establish 4:4:4 16-bit High Throughput Intra Profile in the VVC operation range extensions, and 2) to enable two coding tools in such a profile, namely the alternative derivation method of last significant coefficient position described in CE-1.1[2] , and the bit-aligned full-bypass method described in CE-3.2 [3] .

The source code provided by the proponent was carefully analyzed and found to be consistent with the description in JVET-W0118. The cross-checker’s results on BD-bitrates and bin statistics matched with the ones reported in JVET-W0118. It was also confirmed that the measured runtime reductions were similar to the ones reported in JVET-W0118.

In v2, results were updated.

Simulation Results

Simulations below were carried out based on the common test condition for high bit depth and high bit rate video coding in JVET-U2018 [4] :

- Test#1: VTM-13.0 vs a combination of CE-1.1 and CE-3.2 under low QP condition (QP={-13, -8, -3, 2, 7, 12} for 12-bit and QP={-33, -28, -23, -18, -13, -8} for 16-bit.

- Test#2: VTM-13.0 vs a combination of CE-1.1 and CE-3.2 under lower QP condition (QP={-23, -21, -19, -17, -15, -13} for 12-bit and QP={-47, -44, -41, -38, -35, -32} for 16-bit.

Additionally, bin statistics were also measured based on the method in JVET-V0150 [5] and JVET-N0049 [6] .

Table 1 shows the coding performance over VTM-13.0 under low QP condition, and Table 2 shows the bin statistics, respectively.

Table 3 shows the coding performance over VTM-13.0 under lower QP condition, and Table 4 shows the bin statistics, respectively.

Test#1 Low QP

Table 1: Coding performance of a combination of CE-1.1 and CE-3.2 over VTM-13.0

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.23% | -0.19% | -0.20% | 85% | 76% |
| SVT12 | 1.18% | 1.41% | 1.39% | 87% | 81% |
| Overall | 0.47% | 0.61% | 0.59% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 6.44% | 8.36% | 8.91% | 87% | 80% |
| HLG422 | 5.85% | 7.44% | 7.83% | 85% | 80% |
| Overall | 6.15% | 7.90% | 8.37% | 86% | 80% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 11.22% | 13.79% | 14.10% | 11.01% | 13.58% | 13.98% | 94% | 88% |
| PQ422 | 9.40% | 11.30% | 11.22% | 9.20% | 11.13% | 11.12% | 90% | 87% |
| Overall | 10.31% | 12.55% | 12.66% | 10.11% | 12.36% | 12.55% | 92% | 88% |

Table 2: Bin statistics of a combination of CE-1.1 and CE-3.2 over VTM-13.0

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.23% | -0.19% | -0.20% | -41.67% | -41.63% | -41.63% | -41.50% | 1.16 | 1.02 | -12.16% | 0.47 | 0.28 | -41.43% | 85% | 76% |
| SVT12 | 1.18% | 1.41% | 1.39% | -49.83% | -49.73% | -49.72% | -49.80% | 1.17 | 1.02 | -12.54% | 0.58 | 0.29 | -50.64% | 87% | 81% |
| Overall | 0.47% | 0.61% | 0.59% | -45.75% | -45.68% | -45.68% | -45.65% | 1.16 | 1.02 | -12.35% | 0.52 | 0.28 | -46.50% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 6.44% | 8.36% | 8.91% | -57.74% | -57.24% | -56.96% | -59.71% | 1.24 | 1.04 | -16.49% | 0.80 | 0.32 | -59.52% | 87% | 80% |
| HLG422 | 5.85% | 7.44% | 7.83% | -55.69% | -55.22% | -54.96% | -57.43% | 1.23 | 1.04 | -15.68% | 0.77 | 0.32 | -57.89% | 85% | 80% |
| Overall | 6.15% | 7.90% | 8.37% | -56.72% | -56.23% | -55.96% | -58.57% | 1.24 | 1.04 | -16.09% | 0.78 | 0.32 | -58.72% | 86% | 80% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 11.01% | 13.58% | 13.98% | -54.56% | -53.94% | -53.68% | -86.32% | 1.27 | 1.07 | -16.11% | 0.90 | 0.38 | -58.12% | 94% | 88% |
| PQ422 | 9.20% | 11.13% | 11.12% | -54.30% | -53.63% | -53.37% | -57.01% | 1.26 | 1.06 | -16.11% | 0.87 | 0.37 | -57.53% | 90% | 87% |
| Overall | 10.11% | 12.36% | 12.55% | -54.43% | -53.79% | -53.53% | -71.67% | 1.27 | 1.06 | -16.11% | 0.88 | 0.37 | -57.83% | 92% | 88% |

Test#2 Lower QP

Table 3：Coding performance of a combination of CE-1.1 and CE-3.2 over VTM-13.0

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.04% | -0.09% | -0.10% | 85% | 77% |
| SVT12 | -0.56% | -0.31% | -0.38% | 87% | 80% |
| Overall | -0.30% | -0.20% | -0.24% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.48% | 0.58% | 0.60% | 94% | 79% |
| HLG422 | 0.34% | 0.42% | 0.43% | 88% | 80% |
| Overall | 0.41% | 0.50% | 0.51% | 91% | 80% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 4.99% | 6.75% | 6.66% | 4.79% | 6.30% | 6.24% | 84% | 91% |
| PQ422 | 3.55% | 4.45% | 4.47% | 3.39% | 4.15% | 4.16% | 93% | 89% |
| Overall | 4.27% | 5.60% | 5.57% | 4.09% | 5.23% | 5.20% | 88% | 90% |

Table 4: Bin statistics of a combination of CE-1.1 and CE-3.2 over VTM-13.0

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.04% | -0.09% | -0.10% | -32.27% | -32.84% | -32.79% | -33.33% | 1.12 | 1.02 | -9.29% | 0.41 | 0.28 | -33.09% | 85% | 77% |
| SVT12 | -0.56% | -0.31% | -0.38% | -38.42% | -38.57% | -38.35% | -40.43% | 1.16 | 1.02 | -11.61% | 0.47 | 0.28 | -40.17% | 87% | 80% |
| Overall | -0.30% | -0.20% | -0.24% | -35.35% | -35.70% | -35.57% | -36.88% | 1.14 | 1.02 | -10.47% | 0.44 | 0.28 | -36.86% | 86% | 79% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 0.48% | 0.58% | 0.60% | -45.13% | -45.37% | -45.37% | -46.73% | 1.18 | 1.03 | -12.17% | 0.57 | 0.30 | -46.90% | 94% | 79% |
| HLG422 | 0.34% | 0.42% | 0.43% | -44.17% | -44.39% | -44.39% | -45.57% | 1.18 | 1.03 | -12.17% | 0.56 | 0.30 | -45.67% | 88% | 80% |
| Overall | 0.41% | 0.50% | 0.51% | -44.65% | -44.88% | -44.88% | -46.15% | 1.18 | 1.03 | -12.17% | 0.56 | 0.30 | -46.29% | 91% | 80% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 4.79% | 6.30% | 6.24% | -47.93% | -47.53% | -47.55% | -60.88% | 1.18 | 1.04 | -11.52% | 0.67 | 0.33 | -50.98% | 84% | 91% |
| PQ422 | 3.39% | 4.15% | 4.16% | -46.04% | -45.93% | -45.93% | -48.73% | 1.18 | 1.04 | -11.43% | 0.64 | 0.33 | -48.97% | 93% | 89% |
| Overall | 4.09% | 5.23% | 5.20% | -46.98% | -46.73% | -46.74% | -54.81% | 1.18 | 1.04 | -11.47% | 0.65 | 0.33 | -49.99% | 88% | 90% |

Reference

- F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang, “CE-3 related: High throughput 4:4:4 16 intra profile for VVC”, JVET-W0118

- F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang, “CE-1.1: coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-W0046

- F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang, M. Sarwer, J. Chen, Y. Ye, R. Liao, “CE-3.2: a high throughput mode for high bit depth and high bit rate extensions”, JVET-W0045

- A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

- T. Tsukuba, M. Ikeda, T. Suzuki (Sony), “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding”, JVET-V0150

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).