| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0145-v1 |
| --- | --- |

| Title: | Report on dry-run results for VVC compression performance verification testing in the HDR PQ and HLG categories |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Andrew Segall Vittorio Baroncini Giacomo Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de asegall@sharplabs.com baroncini@gmx.com |
| Source: | Verification Test Coordinators, Test Laboratories |  |  |

_____________________________

# Abstract

This document reports the dry-run verification test results comparing VVC to its predecessor, HEVC, according to the verification test plan JVET-V2021 that was issued at the previous meeting. The compression performance capabilities of the two specifications are compared based on the HEVC reference software HM-16.23 and the VVC reference software VTM-12.0. A visual assessment of VVC compared to HEVC was performed, and the result of a formal subjective assessment evaluation (with the participation of naïve test subjects) is reported. The assessment includes testing with five sequences with HDR PQ content and five sequences with HDR HLG content. MOS-over-bitrate plots indicate the selected rate points are suitable for the purpose of verification testing of VVC compared to HEVC. The curves show good coverage of the MOS range from slightly perceptible impairments down to annoying impairments for all sequences. The goal of approximately matching MOS values for the HM and VTM rate points has been achieved within the confidence intervals for the large majority of the test sequences and rate points. The Bjøntegaard delta rate figures show an overall gain of about 49% for the HLG test sequences and an overall gain of about 52% the PQ test sequences in this dry-run test. The curves for both the HLG and the PQ test sequences show a substantial and consistent benefit of VTM-12.0 over HM-16.23 in terms of MOS over bitrate. Thereby, the dry run confirms the suitability of the rate points proposed in the verification test plan document.

# Introduction

A major design goal for the development of the VVC standard was to achieve a substantial improvement in compression capability relative to its predecessor, the HEVC standard. This document reports the results of a dry-run of verification tests according to the verification test plan JVET-V2021 [1] issued from the previous meeting to confirm the appropriateness of the presented settings.

A subjective evaluation was conducted at two test sites comparing the VVC Main 10 profile to the HEVC Main 10 profile for the HDR PQ and HDR HLG test categories with random-access configuration.

# Verification test logistics

The HDR PQ and HDR HLG subjective tests were carried out at the following test sites:

- GBTech,

- Vabtech.

The tests were conducted using the DCR test method [9] with a 11-grade impairment scale. The verification test environment and the testing methodology is described in Annex A. The test set-up at the two test sites is shown in Table 1.

More than 60 test subjects participated to the test. All subjects were screened for vision acuity and color vision.

All subjects showed a recently done COVID test. The access to the test laboratory was allowed to two subjects at a time; one staying in the test room and one staying in the rest/ waiting room; the two subjects moved back and forth to the two rooms, trying to keeping in any time the COVID “social distance” of at least one meter.

Table 1: Test site information and set up.

| Test Site | GBTech | Vabtech |
| --- | --- | --- |
| Display, size (resolution setting) | LG 65” CX6LA (3840x2160) | LG 65” CX6LA (3840x2160) |
| Viewing distance | 1 viewer at 1.5H | 1 viewer at 1.5H |
| Viewing angle | 90° (at screen center) | 90° (at screen center) |
| Total number of viewers | 30 (16 females, 14 males; age 18-24), all screened for visual acuity and normal colour vision. | 29 (20 females, 9 males, age 18-24), all screened for visual acuity and normal colour vision. |

The display used the HDMI interface as input.  And, the HDR capability of the display was enabled by signaling the appropriate InfoFrame as specified in [15].  Specifically, the EOTF data byte in the Dynamic Range and Mastering InfoFrame was set to the value of 3 for HLG content, and it was set to the value of 2 for the PQ content.  Additionally, for PQ content, the values for the max_display_mastering_luminance data bytes and Maximum Content Light Level data bytes in the Static Metadata Descriptor were set equal to the “Maximum Display Mastering Luminance” reported for each sequence in Table 3 below.

# Verification test generation

In the test, the HEVC bitstreams were encoded using the HEVC reference software HM-16.23 [2]. For VVC, the bitstreams were encoded using the VTM-12.0 reference software [3].

## Verification test content generation

According to the definition of the HDR tests in the verification test plan [1], motion compensated filtering was enabled for both the HM-16.23 and VTM-12.0 bit-streams. For HM-16.23, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_main10.cfg in the software package was used. For VTM-12.0, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_vtm.cfg in the VTM-12.0 software package was employed. Specific sequence configuration settings as well as settings related to HDR are provided in JVET document JVET-W0041 [4].

## Test sequences and rate points selection

The test sequences and rate points as reported in in the verification test plan [1] have been employed.

### HDR HLG test sequences

Table 1 – HDR HLG test sequences under consideration

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) | 3840×2160 | 60 | 600 | HLG |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 3840×2160 | 60 | 600 | HLG |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 3840×2160 | 60 | 600 | HLG |
| C07_DramaSea | 3840×2160 | 60 | 600 | HLG |
| C16_PaddockFollow | 3840×2160 | 60 | 600 | HLG |

Table 2 – Proposed QPs for HDR HLG test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramOnBridge) | 41, 37, 33, 29, 25 | 43, 39, 35, 31, 27 |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 40, 37, 33, 30, 26 | 42, 39, 35, 32, 28 |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 44, 40, 36, 33, 29 | 47, 43, 39, 35, 31 |
| C07_DramaSea | 39, 35, 31, 26, 21 | 41, 37, 33, 29, 24 |
| C16_PaddockFollow | 42, 37, 31, 27, 23 | 45, 40, 35, 30, 26 |

### HDR PQ test sequences

Table 3 – HDR PQ test sequences under consideration

| Test sequence | Resolution | fps | Frames | Transfer Function | Maximum Display Mastering Luminance |
| --- | --- | --- | --- | --- | --- |
| Chimera HDR5 (DrivingPOV_HDR) | 3840×2160 | 60 | 600 | PQ | 4000 |
| Meridian HDR2 (Car) | 3840×2160 | 60 | 600 | PQ | 4000 |
| Meridian HDR5 (Beach) | 3840×2160 | 60 | 600 | PQ | 4000 |
| Sparks DirtLot | 3840x2160 | 60 | 600 | PQ | 1000 |
| Sparks Welding | 3840x2160 | 60 | 600 | PQ | 1000 |

Table 4 – Proposed QPs for HDR PQ test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Chimera HDR5 (DrivingPOV_HDR) | 42, 35, 31, 25, 20 | 45, 38, 32, 28, 22 |
| Meridian HDR2 (Car) | 35, 31, 27, 24, 20 | 38, 34, 30, 26, 22 |
| Meridian HDR5 (Beach) | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| Sparks DirtLot | 42, 36, 31, 25, 20 | 44, 38, 33, 27, 22 |
| Sparks Welding | 42, 36, 31, 25, 20 | 44, 38, 33, 27, 22 |

# Test method and test design

The test sequences were evaluated using the 11-grade scale as specified in ITU-R BT.500-14, shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale as specified in ITU-R BT.500-14 Table 2-4 [8].

A total of four test sessions were designed for the HDR test, two for the PQ and two for the HLG; all the test sessions were designed inserting a “stabilization phase” as suggested in ITU-R BT.500-14, and were not longer than 12 minutes, to avoid fatigue impact.

# Verification test results

The results of the visual assessment in at the two test sites were evaluated by the verification test coordinators, and were found to be consistent in terms of quality range and quality progression over the rate points as voted by the test subjects. Therefore, the votes of both test sites were merged into a joint evaluation providing the results provided below. The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bitrate of the corresponding bitstream. The ±0.95% confidence intervals for the MOS values are indicated.

The MOS plots are provided below. The bitrate and MOS data can be found in the accompanying csv files.

## HDR HLG plots

Figure 2: Plots of the HDR HLG MOS results

The curves for the HLG test sequences show a substantial and consistent benefit of VTM-12.0 over HM-16.23 in terms of MOS-over-bitrate. The curves show a good coverage of the MOS range from slightly perceptible impairments down to annoying impairments for all sequences. The goal of approximately matching MOS values for the HM and VTM rate points has been achieved within the confidence intervals for virtually all test sequences and all rate points.

## HDR PQ MOS and plots

Figure 2: Plots of the HDR PQ MOS results

The curves for the PQ test sequences show a substantial and consistent benefit of VTM-12.0 over HM-16.23 in terms of MOS-over-bitrate. The curves show a good coverage of the MOS range from slightly perceptible down to annoying impairments for all sequences. The goal of approximately matching MOS values for the HM and VTM rate points has been achieved within the confidence intervals for the large majority of the test sequences and rate points.

## Analysis

The bit rate and MOS for all bit-rate points are collected and the Bjøntegaard delta rate (BD rate) for VTM-12.0 relative to HM-16.23 based on bit rate and MOS is reported. The Bjøntegaard delta rate has been computed with the RDPlot tool [14] using the attached csv-files. It has been confirmed that gains are observed for all test sequences if the BD rate is computed using the MOS+CI values for HM and the MOS-CI values for the VTM.

Table 19: Bjøntegaard delta rate of VTM-12.0 relative to HM-16.23 based on bit rate and MOS

| Sequence | BD-Rate |  | Sequence | BD-Rate |
| --- | --- | --- | --- | --- |
| H3_AMS01 | -45% |  | Chimera HDR5 (DrivingPOV_HDR) | -59% |
| H3_AMS06 | -44% |  | Meridian HDR2 (Car) | -58% |
| C07_DramaSea | -48% |  | Meridian HDR5 (Beach) | -47% |
| C16_PaddockFollow | -53% |  | Sparks DirtLot | -53% |
| RiverByBoat | -56% |  | Sparks Welding | -42% |
| Overall | -49% |  | Overall | -52% |

The Bjøntegaard delta rate figures show an overall gain of about 49% for the HLG test sequences, with a gain span of about 12% points across the set of test sequences. For the PQ test sequences, an overall gain of about 52% is observed. Here, the gain span is 17% points with the challenging test sequence Sparks Welding providing the lowest gain of 42% for VTM-12.0 over HM-16.23. The content of this test sequence includes lots of flying sparks from a welding process that have random characteristics and are therefore harder to handle for motion compensated prediction. It is noted that the PQ test set includes the HDR PQ version of the test sequence DrivingPOV (DrivingPOV_HDR) which shows consistent gains across the SDR UHD, SDR HD RA, and HDR test categories.

# Conclusions

The MOS-over-bitrate plots provided in this document indicate the selected rate points to be suitable for the purpose of verification testing of VVC compared to HEVC. The curves show a good coverage of the MOS range from slightly perceptible impairments down to annoying impairments for all sequences. The goal of approximately matching MOS values for the HM and VTM rate points has been achieved within the confidence intervals for the large majority of the test sequences and rate points. The Bjøntegaard delta rate figures show an overall gain of about 49% for the HLG test sequences and an overall gain of about 52% the PQ test sequences in this dry-run test. The curves for both the HLG and the PQ test sequences show a substantial and consistent benefit of VTM-12.0 over HM-16.23 in terms of MOS-over-bitrate.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, output document JVET-V2021, 22nd JVET meeting, by teleconference, Apr. 2021.

- HEVC Test Model, https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tree/HM-16.23, online, accessed Jun. 2021.

- VVC Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-12.0, online, accessed jun. 2021.

- A. Segall, M. Wien, V. Baroncini, K. Andersson, “AHG4: Status Report on HDR Video Verification Test Preparation,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, input document JVET-W0041

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.2100-2 (2018), Image parameter values for high dynamic range television for use in production and international programme exchange.

- SMPTE ST 2084, High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays, 2014.

- SMPTE ST 2036-1, Ultra High Definition Television – Image Parameter Values for Program Production, 2014.

- ETSI TS 101 154, Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications, 2019.

- K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team, doc. JVET-S2016, teleconference, July 2020.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” in ITU-T SG 16 Q.6 document VCEG-M33, 13th VCEG meeting, Austin, Texas, USA, Apr. 2001.

- J. Ström, K. Andersson, Rickard Sjöberg, A. Segall, F. Bossen, G. J. Sullivan, J.-R. Ohm, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 output document JVET-S2016, 19th JVET meeting by teleconference, June 2020.

- RDPlot, https://github.com/IENT/RDPlot, accessed 2021-06-18.

- CTA 861-G, A DTV Profile for Uncompressed High Speed Digital Interfaces, 2016.

# Acknowldegements

The verification test coordinators wish to thank the organizations who contributed to the VVC verification tests, specifically: Sharp Labs of America, Ericsson, Vabtech, GBtech.

# Annex A

The same evaluation method as for the HEVC verification tests is adopted for the VVC verification tests. The following description is based on JCTVC-Q1011 [A4] with minor adaptations.

## A.1 Test method

The test method adopted for this evaluation is DCR (Degradation Category Rating) [A1].

## A.1.1 Degradation Category Rating (DCR)

This test method is commonly adopted when the material to be evaluated shows a range of visual quality that well distributes across all quality scales. All the video material used for these tests consist of video clips of 10 seconds duration.

This method has been used under the schema of evaluation of the quality; for this reason, a quality rating scale made of 11 levels was adopted, ranging from "0" (lowest quality) to "10" (highest quality), see also Figure 1.

The structure of the Basic Test Cell (BTC) of DCR method is made by two consecutive presentations of the video clip under test; at first the original version of the video clip is displayed, immediately afterwards the coded version of the video clip is presented; then a message displays for 5 seconds asking the viewers to vote. The presentation of the video clips is preceded by a mid-grey screen displaying “Source” for the original and “Test” for the coded version of the sequence under test for one second.

Figure 3 - DCR BTC

## A.2 How to express the visual quality opinion with DCR

The viewers were asked to express their vote putting a mark on a scoring sheet.

The scoring sheet for a DCR test is made of a section for each BTC; each section has a box wherein which the viewer shall write the score ranging from 0 to 10. By writing a score of “10”, the subject will express an opinion of “best” quality, while by writing a score of “0” the subject will express an opinion of “worst” quality, as shown in Figure 1.

The vote has to be written when the message "Vote N" appears on the screen. The number "N" is a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate box of the scoring sheet.

## A.4 Training and stabilization phase

The outcome of a test is highly dependent on a proper training of the test subjects.

For this purpose, each subject has to be trained by means of a short practice (training) session demonstrating the range of qualities to be expected in the test.

The stabilization phase uses the test material of a test session; three BTCs, containing one sample of best quality, one of the worst qualities and one of medium quality, are duplicated at the beginning of the test session. By this way, the test subjects have an immediate impression of the quality range they are expected to evaluate during that session.

The scores of the stabilization phase are discarded.

## A.5 The laboratory set-up

The laboratory for a subjective assessment were set up according to [A1], except for the selection of the display and the video play-out server. Play-out of the UHD video clips was done at the native resolution.

The PCs used to play the video sequence supported the display of 10 bit UHD at 30 and 60 frames per second, without any limitation, or without introducing any additional temporal or visual degradation. At GBTech, the connection between the PC and the display was provided by a 10 bit-capable HDMI connection. At RWTH Aachen University, the display was connected via quad-link SDI.

## A.5.2 Viewing environment

The viewing distance was 1.5H, where H is equal to the height of the active part of the screen, depending on the size of the active part of the screen and its native resolution.

The test laboratories were protected from external visual or audio pollution. Internal general light was low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light was placed behind the monitor, in a way no direct light hits the viewing subjects seated in front of the screen; the light behind the monitor must be dimmed to an intensity as specified in Table 4 of Recommendation ITU-T P.911 (“Typical viewing and listening conditions as used in audio-visual quality assessment”). No other light source was admitted, and in particular any light source directed to the screen or creating reflections.

## A.6 Overall test effort and subjects’ involvement

Each viewing session did not run for more than 20 minutes and the same viewing subject did not participate to the test run for more than six hours in total. Young people were hired as test subjects, selecting them for an age from 16 to 30, mostly students of scientific faculties. Viewing subjects were compensated for their participation to the testing activities.

## A.7 Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, were stored in an Excel spread sheet. For each coding condition the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values were computed in the spread-sheets.

The MOS and CI values are used to draw graphs. The graphs are drawn grouping the results for each video test sequence. No graph grouping results from different video sequences is considered.

From the “raw” data subject reliability should be calculated and the method used to assess subject reliability should be reported. Some criteria for subjective reliability are given in [A2] and [A3].

## A.8 References

- International Telecommunication Union Standardization Sector; Recommendation ITU-T P.910 “Subjective video quality assessment methods for multimedia applications”

- Pseudo Isochromatic Plates, engraved and printed by The Beck Engraving Co., Inc., Philadelphia and New York, United States.

- KIRK (R.E.): Experimental Design – Procedures for the Behavioural Sciences, 2nd Editions, Brooks/Cole Publishing Co., California, 1982.

- Tan, T. K.; Mrak, M.; Baroncini, V. & Ramzan, N, “Report on HEVC compression performance verification testing,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2014.