| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0146-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-W0066 (EE2-5.1: Cross-Component Sample Adaptive Offset) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra | Tel: Email: | akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of JVET-W0066 (EE2-5.1: Cross-Component Sample Adaptive Offset). It is reported that the description of the algorithm matches with the software provided by the proponent and the cross-check results match with the results provided by the proponents. Furthermore, an informal subjective evaluation on few sequences reportedly shows that the proposed algorithm shows no subjective quality degradation when compared with the Anchor ECM-1.0.

# Introduction

Cross-component Sample Adaptive offset (CCSAO) algorithm proposed in JVET-W0066[1] leverages the correlation between the different components to improve the reconstructed quality of the signal.  For a given luma/chroma sample, three candidate samples are selected to classify the given sample into different categories: one collocated Y sample, one collocated U sample, and one collocated V sample. The sample values of these three selected samples are then classified into three different bands and a joint index i is used to indicate the category of the given sample. One offset is signaled and added to the reconstructed samples that fall into that category.

CCSAO operates in parallel to sample adaptive offset (SAO) and Bilateral Filter (BIF). Therefore, all the three filters SAO, BIF and CCSAO take the deblocked filter samples as input. Two designs were studied in the proposal JVET-W0066. In the first design a separate clipping stage is added for the CCSAO filtering stage. In the second design, a single “joint clip” operation is performed after all the offsets from SAO, BIF and CCSAO filters are added to the input deblocked samples.

# Simulation results

Table 2.1 below summarizes the cross-check results of applying CCSAO with an additional clipping stage. Table 2.2 summarizes the cross-check results of applying CCSAO along with a joint clip operation with SAO and BIF. The common test conditions specified in [2] are used for the cross check. 

Comparing the results in Table 2.1 and Table 2.2 reportedly shows that using a “joint clip” operation for SAO, BIF and CCSAO filtering stages has the same BD-Rate impact as Table 2.1 (which uses an additional “clipping” stage for the CCSAO filter), and at the same time it reportedly reduces the additional clip operations.  The decoding times reported in the cross-check results may not be completely reliable as the Anchor and Test were run at different times where the simulation cluster load might be different.

## SAO/BIF joint clip + one additional CCSAO clip

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.89% | -1.43% | 103% | 100% |
| Class A2 | 0.05% | -1.69% | -0.28% | 108% | 107% |
| Class B | 0.14% | -2.30% | -3.16% | 105% | 106% |
| Class C | 0.12% | -1.27% | -1.24% | 102% | 106% |
| Class E | 0.11% | -1.12% | -1.82% | 103% | 106% |
| Overall | 0.10% | -1.54% | -1.74% | 104% | 105% |
| Class D | 0.06% | -0.93% | -0.59% | 102% | 109% |
| Class F | -0.11% | -1.97% | -2.37% | 103% | 100% |
| Class TGM | -0.76% | -1.45% | -1.44% | 106% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -2.15% | -4.25% | 104% | 100% |
| Class A2 | 0.09% | -2.52% | -1.36% | 108% | 104% |
| Class B | 0.12% | -4.49% | -5.02% | 106% | 103% |
| Class C | 0.06% | -2.23% | -1.98% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -3.02% | -3.32% | 104% | 102% |
| Class D | 0.06% | -1.87% | -1.45% | 98% | 97% |
| Class F | -0.23% | -2.90% | -2.95% | 106% | 100% |
| Class TGM | -1.15% | -3.03% | -3.24% | 116% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -11.76% | -11.37% | 107% | 97% |
| Class C | 0.06% | -5.96% | -5.11% | 82% | 86% |
| Class E | -0.18% | -2.94% | -6.82% | 93% | 87% |
| Overall | -0.13% | -7.62% | -8.15% | 95% | 91% |
| Class D | 0.16% | -6.25% | -5.04% | 86% | 89% |
| Class F | -0.05% | -7.17% | -6.64% | 109% | 99% |
| Class TGM | -2.23% | -7.59% | -7.60% | 121% | 101% |

## SAO/BIF/CCSAO joint clip

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.89% | -1.43% | 104% | 101% |
| Class A2 | 0.05% | -1.69% | -0.28% | 107% | 102% |
| Class B | 0.14% | -2.30% | -3.16% | 102% | 103% |
| Class C | 0.12% | -1.27% | -1.24% | 101% | 102% |
| Class E | 0.11% | -1.12% | -1.82% | 102% | 107% |
| Overall | 0.10% | -1.54% | -1.74% | 103% | 103% |
| Class D | 0.06% | -0.93% | -0.59% | 100% | 103% |
| Class F | -0.11% | -1.97% | -2.37% | 101% | 98% |
| Class TGM | -0.76% | -1.45% | -1.44% | 106% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -2.15% | -4.25% | 103% | 100% |
| Class A2 | 0.09% | -2.52% | -1.36% | 106% | 103% |
| Class B | 0.11% | -4.51% | -5.01% | 104% | 101% |
| Class C | 0.05% | -2.22% | -2.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -3.03% | -3.32% | 103% | 101% |
| Class D | 0.05% | -1.87% | -1.45% | 98% | 97% |
| Class F | -0.23% | -2.87% | -2.95% | 105% | 99% |
| Class TGM | -1.15% | -3.03% | -3.24% | 118% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -11.74% | -11.40% | 108% | 99% |
| Class C | 0.06% | -5.96% | -5.11% | 102% | 104% |
| Class E | -0.18% | -2.94% | -6.82% | 103% | 102% |
| Overall | -0.13% | -7.61% | -8.16% | 105% | 101% |
| Class D | 0.16% | -6.25% | -5.04% | 95% | 96% |
| Class F | 0.00% | -7.46% | -7.04% | 106% | 99% |
| Class TGM | -2.23% | -7.59% | -7.60% | 119% | 99% |

# Subjective evaluation

Fig. 3a below shows the artifacts (highlighted in blue oval region) observed on ECM-1.0 for the following configuration: Campfire, POC 207, Random Access, QP 37. 
Fig. 3b reports that the proposed method can further improve the subjective details (more details on the face can be observed)

Fig. 3a.   ECM-1.0                         Fig. 3b. CCSAO with “joint clip”

# Conclusion

Cross check results of CCSAO tool are reported. BD-Rate results of the proponent and the cross-check match exactly. An informal subjective evaluation on few sequences shows that the proposed method reportedly does not introduce any new subjective artifacts. However, detailed subjective analysis including more sequences and more QP points is encouraged to be performed by the proponents. Overall, in the cross-checkers opinion the CCSAO tool especially with the “joint clipping design” has a reasonable BD-Rate-computational complexity trade-off with no serious subjective concerns.

# References

- EE2-5.1: Cross Component Sample Adaptive offset, JVET-W0066, C.-W. Kuo et al.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, May 2021, Teleconference.

- ECM-1.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).