| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0160-v1 |
| --- | --- |

| Title: | Cross-check report on JVET-W0093 (AHG8: On significance, GT1, and GT2 flag coding for high bit depths) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-W0093. The simulation results confirm that BD-rate results match with the proponent’s results.

# Introduction

JVET-W0093 proposes a method to improve the context coding of significance flag, greater than one flag, greater than 2 flag and parity flag [1]. A modification is described which effectively shifts the significance, GT1 and GT2 tests left by a number of bits signalled from the encoder. A non-normative algorithm is also described which computes a suitable value for this shift in a pre-encode step.

# Cross-check reports

The simulation results of JVET-W0093 are verified. The tests were conducted under the common test conditions (CTC) defined in [2]. The results show the BD-rate results match exactly with proponent’s results. The crosscheck results are shown in following tables.

Table 1: Low QP test 1 (PQ)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 100% |
| PQ422 | 0.04% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 100% |
| Overall | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 99% |
| PQ422 | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 100% |
| Overall | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 102% | 100% |
| PQ422 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 100% |
| Overall PQ | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 100% |

Table 2: Low QP test 1 (HLG)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.06% | -0.08% | -0.08% | 103% | 100% |
| HLG422 | -0.10% | -0.11% | -0.12% | 102% | 100% |
| Overall | -0.08% | -0.10% | -0.10% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.03% | 102% | 99% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.02% | 0.03% | 0.03% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 102% | 99% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.01% | 0.03% | 0.03% | 102% | 100% |
| Overall HLG | -0.02% | -0.01% | -0.01% | 102% | 100% |

Table 3: Low QP test 1 (SVT)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.87% | -0.89% | -0.89% | 104% | 99% |
| SVT12 | -0.25% | -0.23% | -0.25% | 104% | 101% |
| Overall | -0.56% | -0.56% | -0.57% | 104% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.87% | -0.80% | -0.80% | 107% | 107% |
| SVT12 | -0.09% | -0.06% | -0.07% | 104% | 102% |
| Overall | -0.48% | -0.43% | -0.44% | 105% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.85% | -0.77% | -0.77% | 107% | 107% |
| SVT12 | -0.10% | -0.05% | -0.06% | 104% | 102% |
| Overall | -0.48% | -0.41% | -0.41% | 105% | 104% |
| Overall RGB | -0.51% | -0.47% | -0.47% | 105% | 103% |

Table 4: Low QP test 2 (PQ)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.05% | 0.06% | 0.06% | 0.04% | 0.06% | 0.06% | 102% | 100% |
| PQ422 | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 100% |
| Overall | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 101% | 99% |
| PQ422 | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 100% |
| Overall | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 102% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 102% | 99% |
| PQ422 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 99% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 99% |
| Overall PQ | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 99% |

Table 5: Low QP test 2 (HLG)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.06% | -0.06% | 102% | 100% |
| HLG422 | -0.08% | -0.09% | -0.10% | 102% | 100% |
| Overall | -0.06% | -0.08% | -0.08% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.02% | 0.03% | 102% | 100% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.02% | 0.03% | 0.03% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 102% | 99% |
| HLG422 | 0.02% | 0.03% | 0.04% | 102% | 100% |
| Overall | 0.01% | 0.03% | 0.03% | 102% | 100% |
| Overall HLG | -0.01% | -0.01% | -0.01% | 102% | 100% |

Table 6: Low QP test 2 (SVT)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.63% | -0.62% | -0.62% | 104% | 101% |
| SVT12 | -0.20% | -0.14% | -0.15% | 103% | 102% |
| Overall | -0.41% | -0.38% | -0.39% | 104% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.75% | -0.68% | -0.69% | 106% | 108% |
| SVT12 | -0.06% | -0.03% | -0.04% | 104% | 103% |
| Overall | -0.41% | -0.36% | -0.36% | 105% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.73% | -0.65% | -0.65% | 106% | 108% |
| SVT12 | -0.08% | -0.02% | -0.03% | 104% | 102% |
| Overall | -0.41% | -0.33% | -0.34% | 105% | 105% |
| Overall RGB | -0.41% | -0.36% | -0.36% | 105% | 104% |

# References

A. Browne, S. Keating, K. Sharman, “ AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, JVET-W0093, July 2021.

A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, January 2021.

# Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.