| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0163-v1 |
| --- | --- |

| Title: | Cross-check report on JVET-W0128 (AHG12: Alternative classifiers for ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-W0128. The partial simulation results confirm that BD-rate results match with the proponent’s results. The slight runtime differences are observed in few cases which may be due to the variation of CPU clusters.

# Introduction

JVET-W0128 proposes a two-classifier method and five different second classifier options are tested [2]. For a luma filter set, a flag is signalled to indicate whether the classifier in ECM-1.0 or the proposed alternative classifier is applied.

# Cross-check reports

The simulation results of JVET-W0128 are verified. The tests were conducted under the common test conditions (CTC) defined in [1]. The test4 and tes5 described in JVET-W0128 are tested. The partial results show the BD-rate results match exactly with proponent’s results. Table 1, and Table 2 show the crosscheck results.

Table 1: results of test 4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.04% | 0.04% | 110% | 98% |
| Class A2 | -0.09% | 0.03% | 0.03% | 106% | 98% |
| Class B | -0.16% | 0.04% | 0.04% | 107% | 99% |
| Class C | -0.18% | 0.00% | 0.00% | 109% | 101% |
| Class E | -0.12% | 0.06% | 0.06% | 112% | 99% |
| Overall | -0.14% | 0.03% | 0.03% | 109% | 99% |
| Class D | -0.03% | 0.00% | 0.00% | 111% | 102% |
| Class F | -0.17% | 0.07% | 0.07% | 108% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | 0.10% | 0.23% | 106% | 101% |
| Class C | -0.21% | 0.01% | -0.11% | 110% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.04% | 0.19% | 117% | 101% |
| Class F | -0.21% | 0.00% | 0.13% | 114% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.29% | -0.56% | -0.14% | 113% | 99% |
| Class E | -0.49% | 0.11% | -0.12% | 118% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.08% | 0.48% | 0.17% | 121% | 101% |
| Class F |  |  |  |  |  |

Table 2: Results of test5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.06% | 0.06% | 106% | 96% |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | 0.08% | 0.08% | 103% | 98% |
| Class C | -0.21% | 0.01% | 0.01% | 101% | 100% |
| Class E | -0.25% | 0.14% | 0.13% | 102% | 97% |
| Overall |  |  |  |  |  |
| Class D | -0.06% | 0.03% | 0.03% | 98% | 102% |
| Class F | -0.90% | 0.30% | 0.30% | 103% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.66% | 0.32% | 0.33% | 102% | 100% |
| Class C | -0.23% | 0.07% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | 0.30% | 0.26% | 96% | 101% |
| Class F |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# References

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, April 2021.

N. Hu, V. Seregin, M. Karczewicz, “AHG12: Alternative classifiers for ALF”, JVET-W0128, July 2021.

# Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.