| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0177 |
| --- | --- |

| Title: | Crosscheck of JVET-W0106: EE2-related: Bilateral matching AMVP-merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo | Tel: Email: | cheweikuo@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

Introduction

This document reports the crosschecking results of JVET-W0106 where bilateral matching AMVP-merge mode is proposed to combine AMVP and merge inter predictors. The final motion vector of the merge inter predictor is selected from the merge candidate list and is refined based on the minimum bilateral matching error between a reference block that is generated by AMVP candidate and a reference block that is generated by the merge candidate. Results over ECM-1.0 with template matching enabled and disabled are crosschecked. All the BD-rate points are perfectly matched with the results provided from the proponents.

Table 1 Results over ECM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.02% | -0.19% | 105% | 98% |
| Class A2 | -0.10% | -0.12% | -0.06% | 106% | 99% |
| Class B | -0.11% | -0.02% | -0.04% | 107% | 99% |
| Class C | -0.14% | -0.07% | -0.22% | 107% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.05% | -0.12% | 106% | 99% |
| Class D | -0.11% | -0.11% | 0.06% | 107% | 100% |
| Class F | -0.08% | 0.00% | 0.02% | 106% | 100% |

Table 2 Results over ECM-1.0 with template matching disabled (both anchor and test set macros TM_MRG and TM_AMVP to 0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.21% | -0.27% | 110% | 101% |
| Class A2 | -0.50% | -0.51% | -0.49% | 110% | 100% |
| Class B | -0.46% | -0.41% | -0.32% | 111% | 101% |
| Class C | -0.56% | -0.58% | -0.46% | 111% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -0.44% | -0.38% | 111% | 101% |
| Class D | -0.65% | -0.69% | -0.91% | 111% | 101% |
| Class F | -0.26% | -0.31% | -0.25% | 109% | 101% |