| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0178 |
| --- | --- |

| Title: | [AHG8] Combination of JVET-W0070 and JVET-W0109 on Constraining SPS flags for HBD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Franck Galpin Tangi Poirier Fabrice Le Leannec Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki | Tel: Email: | karam.naser@InterDigital.com Takeshi.Tsukuba@sony.com |
| Source: | InterDigital, Sony Group Corporation |  |  |

# Abstract

This contribution is a combination of JVET-W0070 and W0109 where SPS flags of HBD are constrained for bit-depth less than or equal to 10.

# Introduction

In VVC operation range extension draft [1], the following 4 SPS flags are added:

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

As these tools are designed for high bit-depth data, this contribution proposes constraining these SPS flags when the input bit-depth is less than 10.

# Proposed draft change

## Option #1

In this option, it is proposed to add a conformance constraint for the SPS flags extended_precision_processing_flag and sps_persistent_rice_adaptation_enabled_flag to remove block level semantics. The following change is proposed (added part in blue and removed part in red):

#### 7.4.3.22 Sequence parameter set range extension semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range is may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

It is requirement of bitstream conformance that the value of extended_precision_processing_flag shall be 0 when BitDepth is less than or equal to 10

The variable ExtendedPrecisionFlag is set equal to extended_precision_processing_flag derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

…

sps_persistent_rice_adaptation_enabled_flag equal to 1 specifies that Rice parameter derivation for the binarization of abs_remainder[ ] and dec_abs_level[ ] is initialized at the start of each TU using statistics accumulated from previous TUs. sps_persistent_rice_adaptation_enabled_flag equal to 0 specifies that no previous TU state is used in Rice parameter derivation. When not present, the value of sps_persistent_rice_adaptation_enabled_flag is inferred to be equal to 0.

It is requirement of bitstream conformance that the value of sps_persistent_rice_adaptation_enabled_flag shall be 0 when BitDepth is less than or equal to 10

# In clause 9.3.2.1, make the following changes (additions are highlighted in yellow):

The context variables of the arithmetic decoding engine are initialized as follows:

- If the CTU is the first CTU in a slice or tile, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0, and the following applies:

–	If sps_persistent_rice_adaptation_enabled_flag is equal to 0, the values of StatCoeff[ idx ] for idx ranging from 0 to 2, inclusive, are initialized to be equal to 0.

–	Otherwise the following applies:

StatCoeff[ idx ] = ( bitDepth > 10 ) ? ( 2 * Floor( Log2( bitDepth − 10 ) ) : 0	(X)

## Option #2

In addition to Option 1, the conformance constraint is further extended to other SPS flags for VVC operation range extension. Specifically, the following change is proposed (added part in blue):

#### 7.4.3.22 Sequence parameter set range extension semantics

…

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx_minus1 may be present in slice_header( ) syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx_minus1 is not present in slice_header( ) syntax structures referring to the SPS. When not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

It is requirement of bitstream conformance that the value of sps_ts_residual_coding_rice_present_in_sh_flag shall be 0 when BitDepth is less than or equal to 10

sps_rrc_rice_extension_flag equal to 1 specifies that an extension of Rice parameter derivation for the binarization of abs_remaining[ ] and dec_abs_level[ ] is enabled. sps_rrc_rice_extension_flag equal to 0 specifies that the extension of Rice parameter derivation for the binarization of abs_remaining[ ] and dec_abs_level[ ] is disabled. When not present, the value of sps_rrc_rice_extension_flag is inferred to be equal to 0.

It is requirement of bitstream conformance that the value of sps_rrc_rice_extension_flag shall be 0 when BitDepth is less than or equal to 10

## Option #3

Instead of constraining the individual SPS flags, the upper flag for range extension (sps_range_extension_flag) can be constrained. The following changes are proposed (added part in blue and removed part in red):

# In clause 7.4.3.4, replace the following:

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension_flag equal to 1 specifies that sps_extension_data_flag syntax elements might be present in the SPS RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. However, some use of sps_extension_flag equal to 1 could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow the value of sps_extension_flag equal to 1 to appear in the syntax.

sps_extension_data_flag could have any value. Its presence and value do not affect the decoding process specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

with the following:

sps_extension_flag equal to 1 specifies that the syntax elements sps_range_extension_flag and sps_extension_7bits are present in the SPS RBSP syntax structure. sps_extension_flag equal to 0 specifies that these syntax elements are not present.

sps_range_extension_flag equal to 1 specifies that the sps_range_extension( ) syntax structure is present in the SPS RBSP syntax structure. sps_range_extension_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of sps_range_extension_flag is inferred to be equal to 0.

It is requirement of bitstream conformance that the value of sps_range_extension_flag shall be 0 when BitDepth is less than or equal to 10

…

#### 7.4.3.22 Sequence parameter set range extension semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range is may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is set equal to extended_precision_processing_flag derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

…

# In clause 9.3.2.1, make the following changes (additions are highlighted in yellow):

The context variables of the arithmetic decoding engine are initialized as follows:

- If the CTU is the first CTU in a slice or tile, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0, and the following applies:

–	If sps_persistent_rice_adaptation_enabled_flag is equal to 0, the values of StatCoeff[ idx ] for idx ranging from 0 to 2, inclusive, are initialized to be equal to 0.

–	Otherwise the following applies:

StatCoeff[ idx ] = ( bitDepth > 10 ) ? ( 2 * Floor( Log2( bitDepth − 10 ) ) : 0	(X)

# Reference

[1] Frank Bossen et al., “VVC operation range extensions (Draft 3)”, JVET-V2005. Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).