| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0180-v2 |
| --- | --- |

| Title: | BoG report on high-throughput video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on high-throughput video coding. The BoG held meetings at the following times during the 23rd JVET meeting:

- July 9th 23:20 – July 10th 1.30

# Mandates

The BoG was established with the following mandates:

- Discuss methods and criteria to analyse the entropy coding throughput.

- Discuss how to measure/analyse the worst-case throughput

- Discuss a common/standard method to measure throughput

# Recommendation

BoG recommended to measure the required clock-rate using following method.

Compute clock rate for parsing:

- For context coded bins: 1.8/1.0

- For bypass coded bins without bypass alignment: 4

- For bypass coded bins with pass alignment: 12

Compute clock rate for debinarization:

- Compute the clock rate using non-flag symbols rate. Currently, it is assumed that 1 non-flag symbol can be processed per cycle and 1 flag symbol can be processed nearly at zero cycle.

Select the maximum clock rate between parsing and debinarization as the required clock rate.

BoG recommend attached excel template to report the result.

Further study is recommended to identify how many bypass coded bins can be processed in a cycle when the bypass alignment method is applied.  It is expected to estimate the number of bypass bin per cycle before and after the bypass alignment. This can be study further for more accurate estimation.

Recommended action item:

The current VTM software does not support the output of symbol rate. The symbol rate of VTM was reported in JVET-W0052. It is asked the proponent of JVET-W0052 to provide the number of both flag and non-flag symbol rate for both VTM-13.0 and CABAC skip-mode.

# Discussion about common methodology

An excel template was presented, where the required clock rate is computed from the following statistics:

- The peak context coded bin rate (peakCtxBinRate)

- The peak bypass bin (i.e. EP) rate and (peakEPBinRate)

- The pick bitrate of a coded sequence.

The clock cycle is estimated as follows:

Clk Cycle in GHz = (peakCtxBinRate/CtxW + peakEPBinRate /EpW)/1000000000

Where,

- CtxW = Number of context coded bins can be parsed in one cycle

- EpW = Number of EP bins can be parsed in one cycle

It was commented (assumption) that a single core CABAC engine can process 1 GHz. It is expected that high bit depth professional application may need to have multiple CABAC process in parallel. One way to increase the throughput is using tile etc.

Another participant commented the benefits of having multiple tile-based technique to improve the throughput.

It was commented that the current VTM software does not support the output of symbol rate. The symbol rate of VTM was reported in JVET-W0052. It is asked the proponent of JVET-W0052 to provide the number of both flag and non-flag symbol rate for both VTM-13.0 and CABAC skip-mode.

De-binarization, Symbol rate, how to measure, set-upper limit?

One way can be to set maximum value based on the symbol rate?

One expert commented that debinarization is not so straight forward to measure. It is required to conduct more study on this. It is required to answer how many symbols per cycle can be parse in each cycle.

One participant commented to set a maximum value of GHz using the symbol rate.

It is also commented that when the proposals are evaluated it is also need to consider bin-to-bit ratio.

It was commented that there is a multiplication process in VVC CABAC (whereas HEVC CABAC there is no multiplication) which can also impact the throughput.

Measure the clock-rata using following method:

Compute clock rate for parsing:

- For context coded bins: 1.8/1.0

- For bypass coded bins without bypass alignment: 4

- For bypass coded bins with pass alignment: 12

Compute clock rate for debinarization:

- Compute the clock rate using non-flag symbols rate. Assume, 1 non-flag symbol can be processed per cycle.

Select the maximum clock rate between parsing and debinarization as the required clock rate.

# Proposals under discussion

JVET-W0044 CE3.1: CABAC-bypass alignment for high bit-depth coding [M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba)]

JVET-W0045 CE-3.2: a high throughput mode for high bit depth and high bit rate extensions [F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), M. Sarwer, J. Chen, Y. Ye, R. Liao (Alibaba)]

JVET-W0051 CE-related: Additional bypass coding for high throughput CABAC [M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba)]

JVET-W0052 CE-related: CABAC skip mode [K. Abe, T. Toma, V. Drugeon (Panasonic)]

JVET-W0114 CE-related: High throughput mode for high bit-depth coding – harmonization of CE-3.2 and CE-1.1 [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

JVET-W0117 CE-3 related: a slice level high throughput mode [F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-W0118 CE-3 related: High throughput 4:4:4 16 intra profile for VVC [F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-W0060 AHG8: A constraint of max CTU size and tile on WPP for high bit rate coding [K. Kondo, M. Ikeda (Sony)]

JVET-W0115 AHG8: A study on Bin-to-Bit ratio of VTM-13.0 for high bit depth coding [T. Tsukuba, S. Keating, M. Ikeda, T. Suzuki (Sony)]

This proposal is reviewed in BoG.

This contribution reports the coding performance (BD-bitrate, BD-binrate) and bin-to-bit ratio of VTM-13.0 relative to HM-16.23 for high bit depth coding under low QP setting (QP=-13,…,12 for 12-bit, QP=-33,…,-8 for 16-bit) and normal QP setting (QP=22, 27, 32, 37). The average results are summarized as follows:

In the normal QPs,

- BD-bitrate changes are -26.21% for AI and -17.17% for RA, respectively.

- BD-binrate changes are -18.01% for AI and -2.43% for RA, respectively.

- The unweighted bin to bit ratios of VTM are roughly 13~15% higher than HM’s. The weighted ones are roughly 20% higher than HM’s.

In the low QPs,

- BD-bitrate changes for AI/LDB/RA is {-8.61%, -7.34%, -7.54%} for PQ, {-4.95%, -5.74%, -6.24%} for HLG, and {-3.31%, -3.37%, -3.56%} for SVT, respectively.

- BD-binrate changes for AI/LDB/RA is {7.71%, 6.41%, 6.51%} for PQ, {3.75%, 3.32%, 3.03%} for HLG, and {-1.77%, -2.39%, -2.07%} for SVT, respectively.

- The unweighted bin to bit ratio changes for AI/LDB/RA are {6.35%, 5.36%, 3.66%} for PQ, {2.72%, 3.60%, 2.42%} for HLG, and {-3.65%, -3.56%, -3.80%} for SVT, respectively. The weighted bin to bit ratio changes for AI/LDB/RA are {17.77%, 18.30%, 17.40%} for PQ, {10.75%, 12.34%, 11.74%} for HLG and {1.68%, 1.88%, 1.37 %} for SVT, respectively.

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1). All the percentage numbers are measured against the HM-16.23.

The brief description of all proposals is given below:

| Proposal | Short Description |
| --- | --- |
| CE3.1 | Bypass alignment in CG level + sb_coded_flag bypass after maximum Ctx limit |
| CE3.2 | Bypass alignment in TB level + bypass coding of all transform levels |
| W0051 | CE3.2 + disable last position coding + bypass coding of TS flag + alignment in TU level |
| W0052 | Fully skip the CABAC engine. |
| W0114 | CE3.2 + CE1.1 + bypass coding of last position |
| W0117 | CE3.2 + a slice level enable/disable flag to enable/disable the high throughput mode |
| W0118 | CE3.2 + CE1.1 |
| W0060-t1 | WPP: 1 |
| W0060-t2 | WPP: 1 and CTU size: 64 |
| W0060-t3 | WPP: 1 and Tile: 2x1 |

# Simulation results

## Simulation results vs. VTM13

Table 1: Simulation results for 12 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.1 | 1.13% | 1.18% | 1.14% | 0.87% | 0.93% | 0.94% | 0.56% | 0.48% | 0.59% | 0.60% |
|  | CE3.2 | 10.33% | 12.57% | 12.70% | 6.34% | 8.09% | 8.59% | 1.63% | 1.51% | 1.70% | 1.68% |
|  | W0051 | 17.07% | 20.69% | 20.51% | 8.17% | 9.75% | 10.20% | 1.64% | 1.76% | 1.74% | 17.07% |
|  | W0052 | 12.05% | 13.80% | 13.96% | 7.61% | 8.17% | 8.82% | 3.09% | 3.40% | 2.94% | 2.92% |
|  | W0114 | 12.43% | 14.37% | 14.66% | 7.14% | 8.42% | 8.94% | 1.76% | 1.76% | 1.76% | 1.75% |
|  | W0117 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.22% | 0.17% | 0.24% | 0.24% |
|  | W0118 | 10.31% | 12.55% | 12.66% | 6.15% | 7.9% | 8.37% | 1.32% | 1.18% | 1.41% | 1.39% |
|  | W0060-t1 | 0.04% | 0.04% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
|  | W0060-t2 | 0.12% | 0.02% | -0.06% | 0.07% | 0.02% | 0.02% | 0.02% | 0.03% | 0.02% | 0.01% |
|  | W0060-t3 | 0.11% | 0.11% | 0.09% | 0.04% | 0.03% | 0.03% | 0.02% | 0.03% | 0.02% | 0.02% |
| LDB | CE3.1 | 1.49% | 1.60% | 1.62% | 1.43% | 1.47% | 1.51% | 0.75% | 0.72% | 0.76% | 0.76% |
|  | CE3.2 | 17.40% | 20.27% | 20.36% | 16.89% | 18.27% | 18.82% | 3.19% | 3.00% | 3.29% | 3.27% |
|  | W0051 | 25.55% | 34.10% | 34.32% | 19.92% | 22.80% | 23.77% | 3.07% | 3.41% | 3.39% | 25.55% |
|  | W0052 | 17.47% | 20.17% | 19.51% | 14.58% | 16.26% | 16.70% | 3.93% | 3.79% | 4.00% | 3.99% |
|  | W0114 | 20.19% | 23.35% | 23.68% | 18.04% | 18.89% | 19.52% | 3.26% | 3.08% | 3.36% | 3.35% |
|  | W0117 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.18% | 0.16% | 0.19% | 0.19% |
|  | W0118 | 17.40% | 20.29% | 20.40% | 16.81% | 18.18% | 18.67% | 2.93% | 2.72% | 3.04% | 3.02% |
|  | W0060-t1 | 0.09% | 0.09% | 0.03% | 0.08% | 0.04% | 0.00% | 0.01% | 0.02% | 0.01% | 0.01% |
|  | W0060-t2 | 0.13% | 0.00% | -0.23% | 0.17% | 0.02% | -0.12% | 0.04% | 0.07% | 0.03% | 0.02% |
|  | W0060-t3 | 0.22% | 0.21% | 0.12% | 0.12% | 0.10% | 0.01% | 0.03% | 0.04% | 0.02% | 0.02% |
| RA | CE3.1 | 1.49% | 1.57% | 1.58% | 1.43% | 1.47% | 1.51% | 0.78% | 0.74% | 0.80% | 0.80% |
|  | CE3.2 | 17.11% | 19.76% | 20.20% | 17.19% | 18.16% | 18.61% | 3.59% | 3.40% | 3.69% | 3.67% |
|  | W0051 | 24.97% | 33.22% | 33.56% | 20.08% | 22.72% | 23.28% | 3.53% | 3.86% | 3.84% | 24.97% |
|  | W0052 | 17.21% | 19.75% | 19.52% | 14.87% | 16.29% | 16.64% | 4.11% | 4.07% | 4.14% | 4.13% |
|  | W0114 | 19.83% | 22.63% | 23.32% | 18.35% | 18.72% | 19.26% | 3.78% | 3.61% | 3.88% | 3.86% |
|  | W0117 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.12% | 0.10% | 0.13% | 0.13% |
|  | W0118 | 17.11% | 19.77% | 17.75% | 17.11% | 18.08% | 18.48% | 3.34% | 3.13% | 3.46% | 3.45% |
|  | W0060-t1 | 0.11% | 0.08% | 0.02% | 0.08% | 0.05% | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% |
|  | W0060-t2 | 0.19% | 0.00% | -0.21% | 0.20% | 0.03% | -0.15% | 0.03% | 0.07% | 0.01% | 0.01% |
|  | W0060-t3 | 0.24% | 0.21% | 0.11% | 0.10% | 0.08% | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% |

Table 2: Simulation results for 16 bits data, HBD/HBR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.1 | 0.31% | 0.28% | 0.33% | 0.33% |
|  | CE3.2 | 0.07% | 0.06% | 0.09% | 0.07% |
|  | W0051 | 0.05% | 0.06% | 0.05% | 0.03% |
|  | W0052 | 1.92% | 2.13% | 1.84% | 1.79% |
|  | W0114 | 0.19% | 0.26% | 0.17% | 0.15% |
|  | W0117 | 0.08% | 0.06% | 0.09% | 0.07% |
|  | W0118 | -0.21% | -0.23% | -0.19% | -0.20% |
|  | W0060 test1 | 0.01% | 0.01% | 0.02% | 0.01% |
|  | W0060 test2 | 0.03% | 0.03% | 0.03% | 0.02% |
|  | W0060 test3 | 0.03% | 0.03% | 0.03% | 0.02% |
| LDB | CE3.1 | 0.47% | 0.46% | 0.47% | 0.47% |
|  | CE3.2 | 0.34% | 0.32% | 0.36% | 0.35% |
|  | W0051 | 0.23% | 0.21% | 0.25% | 0.24% |
|  | W0052 | 1.77% | 1.68% | 1.82% | 1.82% |
|  | W0114 | 0.31% | 0.31% | 0.31% | 0.31% |
|  | W0117 | 0.32% | 0.29% | 0.33% | 0.32% |
|  | W0118 | 0.05% | 0.01% | 0.07% | 0.06% |
|  | W0060 t1 | 0.02% | 0.01% | 0.02% | 0.02% |
|  | W0060 t2 | 0.04% | 0.04% | 0.04% | 0.04% |
|  | W0060 t3 | 0.03% | 0.03% | 0.03% | 0.03% |
| RA | CE3.1 | 0.48% | 0.46% | 0.49% | 0.49% |
|  | CE3.2 | 0.39% | 0.35% | 0.41% | 0.41% |
|  | W0051 | 0.28% | 0.24% | 0.30% | 0.29% |
|  | W0052 | 1.76% | 1.75% | 1.77% | 1.77% |
|  | W0114 | 0.35% | 0.35% | 0.35% | 0.35% |
|  | W0117 | 0.28% | 0.23% | 0.30% | 0.30% |
|  | W0118 | 0.10% | 0.04% | 0.13% | 0.12% |
|  | W0060 test1 | 0.01% | 0.02% | 0.01% | 0.01% |
|  | W0060 test2 | 0.04% | 0.06% | 0.03% | 0.03% |
|  | W0060 test3 | 0.03% | 0.04% | 0.03% | 0.03% |

Table 3: Reported run-time estimates for HBD/HBR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE3.1 | 100% | 98% | 100% | 99% | 101% | 99% | 101% | 93% |
|  | CE3.2 | 91% | 88% | 84% | 83% | 86% | 81% | 85% | 76% |
|  | W0051 | 96% | 75% | 86% | 72% | 84% | 78% | 83% | 73% |
|  | W0052 | 87% | 82% | 89% | 70% | 82% | 63% | 80% | 59% |
|  | W0114 | 93% | 87% | 87% | 78% | 83% | 76% | 81% | 71% |
|  | W0117 | 101% | 102% | 101% | 102% | 94% | 94% | 85% | 80% |
|  | W0118 | 93% | 91% | 88% | 84% | 84% | 80% | 81% | 74% |
|  | W0060 t1 | 102% | 100% | 105% | 99% | 104% | 100% | 106% | 100% |
|  | W0060 t2 | 101% | 99% | 103% | 98% | 105% | 99% | 106% | 99% |
|  | W0060 t3 | 99% | 99% | 98% | 100% | 100% | 99% | 102% | 100% |
| LDB | CE3.1 | 100% | 98% | 99% | 99% | 102% | 101% | 101% | 96% |
|  | CE3.2 | 86% | 92% | 73% | 81% | 81% | 83% | 86% | 82% |
|  | W0051 | 82% | 74% | 68% | 71% | 80% | 79% | 86% | 79% |
|  | W0052 | 84% | 87% | 77% | 80% | 70% | 74% | 72% | 70% |
|  | W0114 | 86% | 89% | 70% | 81% | 82% | 80% | 90% | 79% |
|  | W0117 | 100% | 102% | 97% | 98% | 98% | 98% | 86% | 85% |
|  | W0118 | 88% | 92% | 73% | 81% | 82% | 85% | 87% | 82% |
|  | W0060 t1 | 102% | 100% | 107% | 99% | 108% | 101% | 106% | 100% |
|  | W0060 t2 | 95% | 96% | 102% | 96% | 100% | 99% | 100% | 98% |
|  | W0060 t3 | 95% | 100% | 99% | 99% | 101% | 99% | 102% | 99% |
| RA | CE3.1 | 100% | 98% | 99% | 99% | 102% | 101% | 101% | 96% |
|  | CE3.2 | 86% | 90% | 72% | 84% | 82% | 86% | 87% | 86% |
|  | W0051 | 81% | 75% | 66% | 71% | 79% | 79% | 85% | 79% |
|  | W0052 | 86% | 87% | 77% | 80% | 69% | 75% | 70% | 70% |
|  | W0114 | 85% | 89% | 68% | 81% | 80% | 80% | 92% | 78% |
|  | W0117 | 99% | 101% | 99% | 98% | 95% | 100% | 87% | 89% |
|  | W0118 |  |  |  |  |  |  |  |  |
|  | W0060 t1 | 103% | 99% | 105% | 99% | 103% | 99% | 98% | 98% |
|  | W0060 t2 | 101% | 97% | 104% | 98% | 99% | 98% | 100% | 97% |
|  | W0060 t3 | 96% | 100% | 100% | 100% | 101% | 99% | 101% | 98% |

Table 4: Simulation results for HBD/HBR CTC, lossless test configuration.

|  | Test | bit-rate saving |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | HDR PQ | HDR HLG | SVT12 | SVT16 |
| AI | CE3.1 | 0.55% | 0.38% | 0.29% | 0.20% |
|  | CE3.2 | 2.91% | 0.93% | 0.42% | -0.29% |
|  | W0051 | 3.04% | 0.93% | 0.37% | -0.32% |
|  | W0052 | - | - | - | - |
|  | W0114 | 3.10% | 1.19% | 0.28% | -0.11% |
|  | W0117 | 0.22% | 0.86% | 0.42% | -0.29% |
|  | W0118 | 2.54% | 0.59% | 0.16% | -0.47% |
|  | W0060t1 |  |  |  |  |
|  | W0060t2 |  |  |  |  |
|  | W0060t3 |  |  |  |  |
| LDB | CE3.1 | 0.67% | 0.47% | 0.39% | 0.26% |
|  | CE3.2 | 6.11% | 1.29% | 0.39% | -0.28% |
|  | W0051 | 7.10% | 1.16% | 0.29% | -0.35% |
|  | W0052 | - | - | - | - |
|  | W0114 | 6.17% | 1.33% | 0.39% | -0.29% |
|  | W0117 | 0.01% | 1.27% | 0.39% | -0.28% |
|  | W0118 | 5.69% | 0.85% | 0.11% | -0.48% |
|  | W0060t1 |  |  |  |  |
|  | W0060t2 |  |  |  |  |
|  | W0060t3 |  |  |  |  |
| RA | CE3.1 | 0.69% | 0.48% | 0.40% | 0.27% |
|  | CE3.2 | 6.10% | 1.30% | 0.41% | -0.25% |
|  | W0051 | 7.01% | 1.16% | 0.31% | -0.33% |
|  | W0052 | - | - | - | - |
|  | W0114 | 6.12% | 1.32% | 0.41% | -0.26% |
|  | W0117 | 0.01% | 1.28% | 0.41% | -0.25% |
|  | W0118 | 5.69% | 0.86% | 0.14% | -0.45% |
|  | W0060t1 |  |  |  |  |
|  | W0060t2 |  |  |  |  |
|  | W0060t3 |  |  |  |  |

## Simulation results vs. HM16.23

Table 5: Simulation results for 16 bits data, HBD/HBR CTC, LowQP test configuration, HM16.23 with High Throughput 4:4:4 Intra 16 profile.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE3.1 | -1.82% | -2.40% | -1.53% | -1.55% |
|  | CE3.2 | -2.05% | -2.61% | -1.76% | -1.80% |
|  | W0051 | -2.61% | -1.80% | -1.84% | -2.61% |
|  | W0052 | -0.26% | -0.61% | -0.05% | -0.12% |
|  | W0114 | -1.94% | -2.42% | -1.69% | -1.72% |
|  | W0117 | -2.05% | -2.61% | -1.76% | -1.80% |
|  | W0118 | -2.33% | -2.89% | -2.03% | -2.06% |
|  | W0060 t1 | -1.82% | -2.35% | -1.54% | -1.57% |
|  | W0060 t2 | -1.81% | -2.34% | -1.53% | -1.55% |
|  | W0060 t3 | -1.81% | -2.34% | -1.53% | -1.55% |

Table 6: Simulation results for CE3.x tests, 12 bits data, HBD/HBR CTC, LowQP test configuration, HM16.23 with High Throughput 4:4:4 Intra 16 profile

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE3.1 | -7.03% | -9.37% | -10.06% | -4.62% | -6.50% | -6.67% | -2.27% | -4.03% | -2.86% | -2.94% |
|  | CE3.2 | 1.15% | 0.51% | -0.21% | 0.54% | 0.16% | 0.42% | -2.24% | -3.04% | -1.79% | -1.89% |
|  | W0051 | 7.32% | 7.72% | 6.62% | 2.32% | 1.82% | 2.02% | -2.92% | -1.73% | -1.83% | -2.92% |
|  | W0052 | 2.71% | 1.59% | 0.91% | 1.70% | 0.18% | 0.58% | -0.88% | -1.31% | -0.61% | -0.71% |
|  | W0114 | 3.06% | 2.12% | 1.52% | 1.30% | 0.48% | 0.74% | -2.81% | -1.73% | -1.83% | -2.81% |
|  | W0117 | -8.05% | -10.41% | -11.06% | -5.44% | -7.36% | -7.54% | -3.60% | -4.32% | -3.20% | -3.28% |
|  | W0118 | 1.14% | 0.49% | -0.23% | 0.36% | -0.01% | 0.21% | -2.54% | -3.36% | -2.07% | -2.17% |
|  | W0060 test1 | -7.29% | -9.66% | -10.34% | -4.73% | -6.68% | -6.87% | -3.29% | -3.97% | -2.90% | -2.99% |
|  | W0060 test2 | -7.22% | -9.67% | -10.38% | -4.67% | -6.66% | -6.85% | -3.27% | -3.95% | -2.89% | -2.98% |
|  | W0060 test3 | -7.24% | -9.61% | -10.29% | -4.70% | -6.65% | -6.84% | -3.27% | -3.95% | -2.89% | -2.98% |

## Throughput results vs. VTM13

|  | Test | Required clock rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.8 Context coded bins per cycle |  |  |  | 1.0 Context coded bins per cycle |  |  |  |
|  |  | PQ | HLG | SVT12 | SVT16 | PQ | HLG | SVT12 | SVT16 |
|  | VTM-13.0 | 0.592 GHz | 0.885 GHz | 0.935 GHz | 1.220 GHz | 0.887 GHz | 1.250 GHz | 1.236 GHz | 1.531 GHz |
|  |  | % Ratio as compared to VTM-13.0 |  |  |  | % Ratio as compared to VTM-13.0 |  |  |  |
| AI |  | PQ | HLG | SVT12 | SVT16 | PQ | HLG | SVT12 | SVT16 |
|  | CE3.1 | 76% | 69% | 61% | 55% | 85% | 78% | 71% | 64% |
|  | CE3.2 | 31% | 40% | 36% | 32% | 26% | 29% | 29% | 30% |
|  | W0051 | 31% | 40% | 36% | 29% | 21% | 28% | 28% | 24% |
|  | W0052 | 31% | 40% | 36% | 29% | 21% | 28% | 28% | 23% |
|  | W0114 | 31% | 40% | 36% | 29% | 23% | 28% | 28% | 24% |
|  | W0117 | - | - | - | 32% | - | - | - | 30% |
|  | W0118 | 31% | 40% | 36% | 29% | 24% | 28% | 28% | 26% |
|  | W0060 t1 | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% |
|  | W0060 t2 | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% |
|  | W0060 t3 | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% | 100% / 25% |

|  | Test | %diff (Bin to bit ratio) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PQ | HLG | SVT12 | SVT16 |
| AI | CE3.1 | 0% | 1% | 1% | 0% |
|  | CE3.2 | -29% | -20% | -11% | -11% |
|  | W0051 | -34% | -25% | -15% | -14% |
|  | W0052 | -36% | -26% | -16% | -15% |
|  | W0114 | -30% | -22% | -15% | -14% |
|  | W0117 | - | - | - | -11% |
|  | W0118 | -28% | -18% | -14% | -13% |
|  | W0060 t1 | 0% | 0% | 0% | 0% |
|  | W0060 t2 | -1% | 0% | 0% | 0% |
|  | W0060 t3 | 0% | 0% | 0% | 0% |

|  | Test | 16-bit 4:4:4: Worst case data, MinCR = 2 Level 5.1 Max luma sample rate: 534,773,760 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1.8 Context coded bins per cycle |  | 1.0 Context coded bins per cycle |  |
|  |  | Required clock rate (GHz) | Ratio | Required clock rate (GHz) | Ratio |
| AI | VTM-13.0 | 4.565 | 100% | 5.725 | 100% |
|  | CE3.1 | 2.513 | 55% | 3.692 | 64% |
|  | CE3.2 | 1.882 | 41% | 1.882 | 33% |
|  | W0051 | 1.882 | 41% | 1.882 | 33% |
|  | W0052 | 1.882 | 41% | 1.882 | 33% |
|  | W0114 | 1.882 | 41% | 1.882 | 33% |
|  | W0117 | 1.882 | 41% | 1.882 | 33% |
|  | W0118 | 1.882 | 41% | 1.882 | 33% |
|  | W0060 test1 | 4.565 / 1.141 | 100% / 25% | 5.726 / 1.431 | 100% / 25% |
|  | W0060 test2 | 4.565 / 1.141 | 100% / 25% | 5.726 / 1.432 | 100% / 25% |
|  | W0060 test3 | 4.565 / 1.141 | 100% / 25% | 5.727 / 1.432 | 100% / 25% |

|  | Test | 16-bit 4:4:4: Worst case data, MinCR = 2 Level 6.2 Max luma sample rate: 4,278,190,080 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1.8 Context coded bins per cycle |  | 1.0 Context coded bins per cycle |  |
|  |  | Required clock rate (GHz) | Ratio | Required clock rate (GHz) | Ratio |
| AI | VTM-13.0 | 36.517 | 100% | 45.800 | 100% |
|  | CE3.1 | 20.101 | 55% | 29.533 | 64% |
|  | CE3.2 | 15.057 | 41% | 15.057 | 33% |
|  | W0051 | 15.057 | 41% | 15.057 | 33% |
|  | W0052 | 15.057 | 41% | 15.057 | 33% |
|  | W0114 | 15.057 | 41% | 15.057 | 33% |
|  | W0117 | 15.057 | 41% | 15.057 | 33% |
|  | W0118 | 15.057 | 41% | 15.057 | 33% |
|  | W0060 test1 | 36.521 / 9.130 | 100% / 25% | 45.808 / 11.452 | 100% / 25% |
|  | W0060 test2 | 36.521 / 9.130 | 100% / 25% | 45.808 / 11.452 | 100% / 25% |
|  | W0060 test3 | 36.523 / 9.131 | 100% / 25% | 45.814 / 11.453 | 100% / 25% |

|  | Test | 12-bit 4:2:2: Worst case data, MinCR = 2 Level 5.1 Max luma sample rate: 534,773,760 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1.8 Context coded bins per cycle |  | 1.0 Context coded bins per cycle |  |
|  |  | Required clock rate (GHz) | Ratio | Required clock rate (GHz) | Ratio |
| AI | VTM-13.0 | 3.498 | 100% | 4.919 | 100% |
|  | CE3.1 | 2.378 | 68% | 3.823 | 78% |
|  | CE3.2 | 1.517 | 43% | 1.517 | 31% |
|  | W0051 | 1.517 | 43% | 1.517 | 31% |
|  | W0052 | 1.517 | 43% | 1.517 | 31% |
|  | W0114 | 1.517 | 43% | 1.517 | 31% |
|  | W0117 | - | - | - | - |
|  | W0118 | 1.517 | 43% | 1.517 | 31% |
|  | W0060 test1 | 3.498 / 0.875 | 100% / 25% | 4.921 / 1.230 | 100% / 25% |
|  | W0060 test2 | 3.441 / 0.860 | 98% / 25% | 4.837 / 1.209 | 98% / 25% |
|  | W0060 test3 | 3.485 / 0.871 | 100% / 25% | 4.903 / 1.226 | 100% / 25% |

|  | Test | 12-bit 4:2:2: Worst case data, MinCR = 2 Level 6.2 Max luma sample rate: 4,278,190,080 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.8 Context coded bins per cycle |  |  |  | 1.0 Context coded bins per cycle |  |  |  |  |
|  |  | Required clock rate (GHz) | Ratio |  |  | Required clock rate (GHz) |  |  | Ratio |  |
| AI | VTM-13.0 | 27.982 |  | 100% |  | 39.354 |  |  |  | 100% |
|  | CE3.1 | 19.024 |  |  | 68% |  | 30.584 | 78% |  |  |
|  | CE3.2 | 12.134 |  |  | 43% |  | 12.134 | 31% |  |  |
|  | W0051 | 12.134 |  |  | 43% |  | 12.134 | 31% |  |  |
|  | W0052 | 12.134 |  |  | 43% |  | 12.134 | 31% |  |  |
|  | W0114 | 12.134 |  |  | 43% |  | 12.134 | 31% |  |  |
|  | W0117 | - |  |  | - |  | - | - |  |  |
|  | W0118 | 12.134 |  |  | 43% |  | 12.134 | 31% |  |  |
|  | W0060 test1 | 27.988 / 6.997 |  |  | 100% / 25% |  | 39.369 / 9.842 | 100% / 25% |  |  |
|  | W0060 test2 | 27.525 / 6.881 |  |  | 98% / 25% |  | 38.700 / 9.675 | 98% / 25% |  |  |
|  | W0060 test3 | 27.883 / 6.971 |  |  | 100% / 25% |  | 39.225 / 9.806 | 100% / 25% |  |  |