| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0182-v1 |
| --- | --- |

| Title: | DRAFT BoG Report: Neural Networks Video Coding Analysis and Planning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall | Email: | asegall@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a draft report of activities from the BoG on Neural Network Video Coding (NNVC) Analysis and Planning.  The BoG held the following meetings during the 23rd JVET meeting:

- July 9 – 23:20 – July 10 01:20

Recommendations from the BoG are summarized as:

# Mandates

The BoG was established with the following mandates:

- Perform further analysis about the complexity/compression trade-offs of the various loop filter and super resolution proposals and align the reports with AHG11 reporting conditions

- Perform further analysis between luma and chroma gains in detail

- Suggest candidates for an upcoming EE

- Refine the AHG11 reporting conditions as necessary

# Complexity and Compression Trade-Offs and Analysis

The following questions were proposed to be considered in performing the analysis:

- How should we align test results that did not use the AHG11 reporting conditions?

  - Used VTM-11.0

  - Used a different QP range

  - Results were incomplete

- How should we summarize the different luma and chroma performances that are being observed?

  - One potential solution - use a weighted combination of the Y, Cb and Cr BD-rates.  For example, AHG13 previously used 8:1:1

The group agreed that these were the main questions to be answered.

## Discussion

### On results that used VTM-11.0 as an anchor

One participant commented the use of VTM-11.0 as an anchor would not affect the all-intra results.  Additionally, it was commented that the random-access performance would not be affected for all-intra tools in this case.  A second participant agreed with the above.

After subsequent discussion (below), the group decided to remove the random-access results for proposals that used a VTM-11.0 anchor for reporting the summary results.

One participant commented that JVET-W0062 used VTM 12.0 with ALF and SAO disabled as an anchor.  It was decided to re-compute the BD-Rate performance compared to the VTM-11.0-nnvc anchor.

### On the different QP range:

One participant suggested to compute the BD-Rate performance of all proposals over the range of QP27, 32, 37, 42.  This would just be for 4K only results, as all of the other results used a QP range of QP22, 27, 32, 37, 42.

One participant suggested to prepare an additional plot that attempts to compare the super-resolution and in-loop filtering technologies.

One participant suggested that super-resolution method should be required to bring all CTC results going forward.  It was further suggested that the performance of super-resolution methods should be considered after in-loop filters.

The following plan was put forward for reporting:

- Create a summary for all tools using the NNVC CTC.  This would exclude the super-resolution methods, since the super-resolution proposals did not provide results for all sequences and QP points in the CTC.

- Create a summary of all tools for 4K using the NNVC CTC but with a QP range of QP27, 32, 37, 42.  This could then include all of the tools, as the super-resolution methods did not report results for QP22.

- Create a summary of super-resolution tools for 4K using CTC but with a QP range of QP27, 32, 37, 42, 47

The group decided on the above strawman approach.

### On incomplete results

The group decided to remove proposals from the summary that had incomplete results.

### On chroma results

One participant recommended that the group should inspect the chroma BD-rate curves and identify cases where they were crossing.  And, if the curves are crossing – the chroma results should be ignored.

Multiple participants expressed concern on reporting a metric that was averaged over the luma and chroma channels.  It was proposed to report the luma and chroma results separately.

Multiple participants expressed support for reporting a combined metric to the group.

The group decided to report set of luma results and a set of chroma results, where the chroma results would be the average of the Cb/Cr channels.  In addition, the group will check for cases where the chroma curves are crossing – and create a note in these cases that the results are unreliable.

### On complexity reporting

One participant raised a question about complexity reporting in the case that a proposal used multiple models that are selectively enabled.  The question was if the summary should use the worst-case performance or a different metric.

One participant expressed that worst-case performance should be reported.

Multiple participants expressed support for reporting the decoding complexity and average complexity in terms of run-time.

One participant expressed support for capturing more detailed information on complexity.  This could include latency.

One participant recommended that the summary output of the BoG should also capture memory size.

One participant recommended that the summary capture if MAC operations use a floating point or integer operation.

Multiple participants recommended that the group study what is a realistic complexity and architecture for the near-term and long-term.

The group recommended that this should be a mandate of the NNVC AhG group.  And, that the discussion should be incorporated into the EE design as much as possible.

The group decided to report worst-case complexity in the summary.

The group decided to capture the precision of the calculations in the summary.

The group decided to capture the memory size (number of parameters * the precision of the parameters) of all models being used in the proposals.

Next step – off-line activity to begin preparing the aligned summary results.  Following the activity, review and re-visit the summary plan.

# Exploration Experiment Planning

The following issues were proposed to be considered when developing the EE test:

- What is the realistic complexity that should be targeted by the EE?

- What is the near-term and long-term architecture that should be targeted by the EE?

- How should the group work toward a common software?

- What tests should be performed?

- What technologies should be included in the test?

- What is the timeline for the EE?

# Refine the AHG11 Reporting Conditions