| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0186-v1 |
| --- | --- |

| Title: | EE1-related: Report on results of remote viewing session |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien A. Segall V. Baroncini | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de asegall@sharplabs.com baroncini@gmx.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

Remote expert viewing tests were conducted during the 23rd JVET meeting for the exploration activity on DNN-based coding tools on UHD test sequences as defined in EE1. Calls for participation in the subjective viewing were issued on the JVET reflector and verbally during JVET sessions. Overall, 22 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test.

The results reveal performance improvements for the proposals compared to the VTM anchor. The plots consistently reveal a visual benefit of the proposals over the VTM anchor. In the super-resolution category, the visual benefit of JVET-W0105 appears to be comparable to the visual benefit observed for the RPR configuration under test. For the Campfire sequence, this visual benefit appears to be more substantial than for the Tango sequence. The results for the loopfilter category suggest some visual benefit of JVET-W0130 over the VTM anchor.

Multiple participants remarked that the visual quality of the Campfire sequence was very low. Further, the fact that the sequences under test had different length was commented. It is suggested to align the duration of the video sequences to be presented by cropping a suitable 5 sec part out of 30Hz sequences for the visual tests.

# Introduction

Remote expert viewing tests were conducted during the 23rd JVET meeting for the exploration activity on DNN-based coding tools defined in EE1 [2]. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 22 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test [1].

# Test arrangement

EE1-participants were requested to prepare and submit mp4 files of rate points which they considered to be suitable for visually demonstrating the proposed scheme. All sequences were at UHD resolution and taken from the EE1 test set using the RA configurations. According to the EE1 test configuration, the sequences were of different length, with the test sequence Campfire (30 Hz) having 10 s duration while other sequences in RA configuration were of 5 s length.

Table 1: Pairs of mp4 files assessed in the visual test

| Anchor | Proposal |
| --- | --- |
| VTM-lf/DaylightRoad2_3840x2160_60fps_10bit_420_QP42.mp4 | JVET-W0130/DaylightRoad2_3840x2160_60fps_10bit_420_QP42.mp4 |
| VTM-lf/Campfire_3840x2160_30fps_bt709_420_videoRange_QP42.mp4 | JVET-W0130/Campfire_3840x2160_30fps_bt709_420_videoRange_QP42.mp4 |
| VTM-sr/Campfire_3840x2160_30fps_bt709_420_videoRange_QP45.mp4 | JVET-W0105/Campfire_3840x2160_30fps_bt709_videoRange_QP37.mp4 |
| VTM-sr/Tango2_3840x2160_60fps_10bit_420_QP45.mp4 | JVET-W0105/Tango2_3840x2160_60fps_10bit_420_QP37.mp4 |
| VTM-sr/Campfire_3840x2160_30fps_bt709_420_videoRange_QP45.mp4 | RPR/Campfire_3840x2160_30fps_bt709_420_videoRange_QP37.mp4 |
| VTM-sr/Tango2_3840x2160_60fps_10bit_420_QP45.mp4 | RPR/Tango2_3840x2160_60fps_10bit_420_QP37.mp4 |

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM anchor. It was held as an online session via Zoom with the participants having downloaded the (anonymized) mp4 files before hand. The VLC playlists for the sessions were provided to them shortly before the start of the viewing.

In the following, video A and video B denote either the anchor or the proposal from the pairs of mp4 files listed in Table 1. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. Two viewing sessions were conducted. A viewing session consisted of 9 test cells. Each test cell was constructed as follows:

- “Original” (on a mid-gray background, 1 s presentation)

- Original Video

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The participants were supposed to fill the voting sheet and enter their results into an online form after completion of the session. The participants were also asked to enter the used screen size and any specific observations in a comment field of the online form.

The voting sheets were used in order to simplify the workload for the participants during the session.

Figure 1: Voting sheet

Before the start of the test session, the viewers were provided a demo playlist in order to familiarize themselves with the procedure of the test. The viewers were verbally instructed about the procedure and the rating scale in the introduction of the session. They were asked not to stop the video during the test sessions but to use the assigned voting time to record their judgement.

## Voting scores

For evaluation, the votes of the test subjects were mapped to MOS numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality. The scores of the four participants with the lowest Pearson correlation were removed from the evaluation.

## Participants comments

Multiple participants remarked that the visual quality of the Campfire sequence was very low. Further, the fact that the sequences under test had different length was commented. It is suggested to align the duration of the video sequences to be presented by cropping a suitable 5 sec part out of 30Hz sequences for the visual tests.

# Test results

The MOS values resulting from the evaluation are reported in Table 2. According to the assignment of voting scores, a positive number indicates a visual benefit for the proposal over the anchor while a negative number indicates the proposal to be inferior to the anchor. The test included a stabilization phase and a hidden identity test, referred to as “VTM-self” in the table.

Table 2: MOS results for the A/B comparisons under test

| Test | Sequence | MOS | CI |
| --- | --- | --- | --- |
| JVET-W0105 | Campfire | 2.1 | 0.6 |
| JVET-W0105 | Tango | 1.3 | 0.7 |
| JVET-W0130 | Campfire | 1.2 | 0.7 |
| JVET-W0130 | DaylightRoad | 0.8 | 0.5 |
| RPR | Campfire | 2.2 | 0.6 |
| RPR | Tango | 1.0 | 0.4 |
| VTM-self | Campfire | -0.2 | 0.5 |

Figure 2: MOS plots

The plots consistently reveal a visual benefit of the proposals over the VTM anchor. In the super-resolution category, the visual benefit of JVET-W0105 appears to be comparable to the visual benefit observed for the RPR configuration under test. For the Campfire sequence, this visual benefit appears to be more substantial than for the Tango sequence. The results for the the oopfilter category suggest some visual benefit of JVET-W0130 over the VTM anchor.

# Conclusions

The results reveal performance improvements for the proposals compared to the VTM anchor. The plots consistently reveal a visual benefit of the proposals over the VTM anchor. In the super-resolution category, the visual benefit of JVET-W0105 appears to be comparable to the visual benefit observed for the RPR configuration under test. For the Campfire sequence, this visual benefit appears to be more substantial than for the Tango sequence. The results for the loopfilter category suggest some visual benefit of JVET-W0130 over the VTM anchor.

Multiple participants remarked that the visual quality of the Campfire sequence was very low. Further, the fact that the sequences under test had different length was commented. It is suggested to align the duration of the video sequences to be presented by cropping a suitable 5 sec part out of 30Hz sequences for the visual tests.

# References

- J. Jung, M. Wien, V. Baroncini, “Draft SC29/AG5 Guidelines for remote experts viewing sessions,” Doc. JVET-W0185, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, July 2021.

- E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. M, H. Wang, „EE1: Summary of Exploration Experiments on Neural Network-based Video Coding,“ Doc. JVET-W0023, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, Apr. 2021.