| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0187-v1 |
| --- | --- |

| Title: | AVC and HEVC draft text in response to JVET-W0080 and JVET-W0083 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposed draft text |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Ye-Kui Wang | Email: | garysull@microsoft.com yekui.wang@bytedance.com |
| Source: | Microsoft, Bytedance |  |  |

_____________________________

# Abstract

This contribution proposes draft text changes for the AVC and HEVC standards that were prepared in response to JVET-W0080 and JVET-W0083, relative to the text circulated by ITU-T for AAP Last Call.

# Introduction

JVET contributions JVET-W0080 and JVET-W0083 have reported some problems in the text drafted for the VSEI standard (Rec. ITU-T H.274 | ISO/IEC 23002-7).

Most of the reported problems are also evident in the text of AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2) as well as in VSEI. In the ITU-T, these other standards are currently in a “Last Call” stage of the ITU-T Alternative Approval Process for edition 14 of H.264 and edition 8 of H.265, and the Last Call period is closing on the day of submission of this document.

The attached drafts contain suggested edits to address the identified issues, and have been submitted as Microsoft input to the Last Call.

We suggest that these proposed changes are clearly merely editorial in spirit as they are only made to provide obviously missing material, correct small errors, and improve clarity and consistency.

The changes are summarized as follows. Further commentary is available in JVET-W0080 and JVET-W0083.

- Correcting an “off by one” error in the loop counter for ar_num_object_updates, which indicates the number of updated annotated objects for the annotated regions SEI message.

- Adding a description of the purpose of some variables that are derived in the semantics of the annotated regions SEI message

- Adding a missing descriptor to specify the type of coding for the depth_nonlinear_representation_model[ i ] syntax element (the same as used in another specification, so clearly what was intended) for the depth representation information SEI message

- Adding missing range specifications for a few syntax elements of the depth representation information SEI message

- Miscellaneous very small cleanups of consistency, grammar and clarity, and correction of typographical errors.

The edits are marked using edit tracking with a special username identifier. For H.264v14, the edit identifier is “V14+”, and for H.265v8, the edit identifier is “V8+”.

# References

- JVET-W0080 AHG9: some errata and clarification items for Additional SEI messages for VSEI [E. François, M. Radosavljevic (InterDigital)]

- JVET-W0083 AHG9: Bug fixes for some SEI messages in the VSEI amendment [Y.-K. Wang, Y. Wang, L. Zhang (Bytedance)]