| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0188-v1 |
| --- | --- |

| Title: | JVET BoG Report: VVCv2 Conformance Testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on VVCv2 Conformance Testing.  The BoG was held during the 23rd JVET meeting, on July 13, at 21:00 – 23:40 GMT.

The BoG was established with the following mandate:

- Prepare a list of conformance bitstreams for VVC v2 and identify volunteers.

The BoG recommends:

- Review the preliminary list of the identified conformance bitstreams (65 in total) for VVCv2 provided in Table 4.

- Discuss if JVET to produce output document on VVCv2 conformance (as per AhG5 chair suggestion).

- Thank to volunteers offering help in conformance bitstream generation: Alibaba, Interdigital, KDDI and Sony.

# Agenda

- Brief review of relevant aspects of VVCv1 conformance development.

- Identify the VVCv2 adoptions which may require conformance bitstream.

- Identify applicable configurations to be tested, e.g. bitdepths and chroma formats.

- Produce a preliminary list of conformance bitstreams:

  - Provide example of description.

  - Identify volunteers.

- Review notes and document BoG recommendations.

# VVCv1 conformance set

The following documents have been identified as relevant to VVCv1 conformance bitstream development:

- JVET AHG report: Conformance testing (AHG5) J. Boyce, W. Wan, E. Alshina, F. Bossen, I. Moccagatta, K. Kawamura, K. Sühring, X. Xu, JVET-W0005.

  - Timeline, status and active topic for discussion.

- Conformance testing for versatile video coding (Draft 6), J. Boyce, E. Alshina, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan, JVET-U2008.

  - Conformance bitstream generation guidelines (Annex A):

Quote:

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases. Multiple bitstreams should be provided for each feature.

It is recommended to include both low resolution (512x512) and short duration (10 frame) test bitstreams as well as typical resolutions and longer duration test bitstreams for each test category.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

The test bitstreams shall not violate the profile, tier, and level indicated in the bitstream. The minimum level that the bitstream conforms to should be used.

A VTM config file should be included in the .zip file if an unmodified VTM version is used, and the command line used should be included in the .txt file. If a modified version of the VTM is used, the VTM config file should not be included.

Discussion:

- One expert asked if bitstreams file naming convention for VVCv2 bitstreams will be different from VVCv1.

- AhG5 representative suggested that file naming convention may not change, but directory could be different for VVCv2.

- One expert suggested that VVCv2 software may require different compile configuration for reference VVCv2 software.

- One expert suggested that having independent VVCv2 decoder implementation (in addition to VTM decoder) would be helpful for conformance work.

- It was suggested to clearly specify compile configuration. This can be done in Conformance/Reference software output document and in bitstream description.

- AhG5 representative suggested HBD coding community to provide co-chair for AhG5 activity to cover bitstream for VVCv2.

Agreement:

- BoG agreed to follow VVCv1 conformance bitstream generation guidelines and procedure.

- BoG agreed VVCv2 conformance set to be generated in coordination with AhG5.

  - Bitstream categorizations:

AhG5 reports that 104 bitstream categories and 281 bitstreams have been produce for VVCv1 conformance testing.

Coding tools/bitstream categorization provided in Table 5 and 6 of JVET-U2008 seems to be applicable for VVCv2 conformance bitstream preparation:

Quote:

### Bitstreams for Main 10 4:4:4 profile

Table 5: Coding tools for Main 10 4:4:4 profile with 4:4:4 10 bit

| Categories | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- |
| Basic 4:4:4 | 10-bit 4:4:4 | 10b444 | No 4:4:4-specific coding tools enabled |
| SCC coding | Adaptive Color Transform | ACT |  |
| SCC coding | Palette mode | PALETTE |  |
| Entropy | Entropy coding for 4:4:4 main tier | ENT444MAINTIER | Max bins and bits for Main 444 Main Tier |
| Entropy | Entropy coding for 4:4:4 high tier | ENT444HIGHTIER | Max bins and bits for Main 444 High Tier |

Table 6: Additional chroma formats and bit depths Main 10 4:4:4 profile

| Tool description | Feature Name | Bitstream features |
| --- | --- | --- |
| 8b 4:2:2 | 8b422 | 8-bit 4:2:2 in Main 10 4:4:4 profile |
| 8b 4:4:4 | 8b444 | 8-bit 4:4:4 in Main 10 4:4:4 profile |
| 10b 4:2:2 | 10b422 | 10-bit 4:2:2 in Main 10 4:4:4 profile |

### Discussion:

- Should VVCv2 conformance bitstream generation follow VVCv1 conformance testing guidelines?

- Should VVCv2 conformance bitstream generation to follow categorization principles for generation of VVCv2 conformance bitstreams?

Agreement:

- BoG agree with aforementioned approach for bitstream categorization for purpose of VVCv2 conformance test set generation.

# VVCv2 related adoptions:

The following tools have been identified as new adoptions toward VVCv2 and may need to have conformance bitstreams provided:

- Extended Precision Processing (EPP), JVET-V0047.

- Extended Regular Residual Coding (ERRC), JVET-V0106.

- Persistent Regular Residual Coding (PRRC), JVET-V0106.

- Reverse last significant coding (RRLC), JVET-W0046.

- Extended Transform Skip Residual Coding (ETSRC), JVET-V0054.

Other related adoptions/decision:

- Decision on high throughput:

For a possible high throughput profile at this stage of definition (DIS), it would be most appropriate using the solution suggested in JVET-W0060 test 1 (making WPP mandatory).

- HLS clarification, JVET-W0109  “proposal 2 case 1”:

The following adoptions have been identified and categorized as may not be addressed by conformance bitstream.  Adopt: JVET-W0109 “proposal 2 case 1”, up to discretion of editors to align with block-level.

- HLS clean up, JVET-W0070 proposal #3:

A similar approach should be taken for the Rice parameter extension mechanism (or potentially other mechanisms of v2 that currently have low-level check for <= 10 bits).

### Discussion:

One expert suggested that HLS adoption are not fully specified, either revisit or up-to editors choice.

One expert suggested that high throughput aspects are expected to be defined in VVCv3 and no need to address it in VVCv2 conformance set.

One expert suggested the listed HLS adoptions are semantic constrained, and conformance will be included in other bitstream, e.g. 10 bits data being coded in Main 16 4:4:4.

Agreement:

- List of VVCv2 coding tools to be covered by conformance bitstreams:  EPP, ERRC, PRRC, RRLC, ETSRC.

- High throughput not to be covered by the current bitstream list.

- HLS adoptions not to be covered by a separate bitstreams.

# Configurations to be tested:

# The following profiles are expected to be defined in VVCv2:

- Main 12 4:2:0

- Main 10 4:2:0 Intra and Main 12 4:2:0 Intra

- Main 12 4:4:4 and Main 16 4:4:4

- Main 10 4:4:4 Intra, Main 12 4:4:4 Intra, Main 16 4:4:4 Intra

# The following video data formats and coding configurations are expected to be supported profiles in VVCv2:

# Chroma formats: 4:0:0, 4:2:0, 4:2:2, 4:4:4.

# Bit depths:  10, 12, 16.

### Discussion:

One expert asked about the number of profiles 9, not 8.

It was pointed that list of supported profiles will be updated once JVET will make decision.

One expert requested that 4:0:0 need to be tested, BoG agreed.

Agreement:

- BoG agreed on the list of profiles and formats to be covered in the current preliminary list.

# List of VVCv2 conformance bitstreams:

The Table 1 provides a preliminary list of tools mapped against the planned VVCv2 profiles.  YesEach tool of the VVCv2 is being tested with other VVCv2 tools being enabled. The bitstream should exercise a full range of parameters of modes of the tool tested in the category.

Bitstream targeting a specific profile to have maximum bitdepth and chroma format allowed for the given profile, e.g. 12 bit data and 4:4:4 chroma format for bitstreams of Main 12 4:4:4.

Table 1: Planned VVCv2 profiles and tested tools applicability (39 categories)

| Profile | VVCv1 tools set | ERRC | PRRC | ETSRC | EPP | RLSC |
| --- | --- | --- | --- | --- | --- | --- |
| Main 10 4:2:0 Intra | Yes | n/a | n/a | n/a | n/a | n/a |
| Main 10 4:4:4 Intra | Yes | n/a | n/a | n/a | n/a | n/a |
| Main 12 | Yes | n/a | n/a | n/a | n/a | n/a |
| Main 12 Intra | Yes | ? | ? | ? | ? | ? |
| Main 12 Still Picture | Yes | ? | ? | ? | ? | ? |
| Main 12 4:4:4 | Yes | Yes | Yes | Yes | Yes | Yes |
| Main 12 4:4:4 Intra | Yes | Yes | Yes | Yes | Yes | Yes |
| Main 12 4:4:4 Still Picture | Yes | Yes | Yes | Yes | Yes | Yes |
| Main 16 4:4:4 | Yes | Yes | Yes | Yes | Yes | Yes |
| Main 16 4:4:4 Intra | Yes | Yes | Yes | Yes | Yes | Yes |
| Main 16 4:4:4 Still Picture | Yes | Yes | Yes | Yes | Yes | Yes |
| Proponent | Kwai | QC | QC | Kwai | Sharp | OPPO |
|  | QC |  |  |  |  |  |
|  | OPPO |  |  |  |  |  |
|  | Sharp |  |  |  |  |  |

Note: Entries marked by yellow highlight in Table 1 have been modified after the BoG was closed. Changes reflects clarification on suggested VVCv2 profile definitions.

### Discussion:

One expert asked whether VVCv2 coding tools to be disabled for Main12 4:2:0.

It was commented as per profile definition, VVCv2 coding tools would be disabled in Main12 4:2:0 (consumer profile).

There was a discussion about applicability of the tools for Main 12 and 4:2:0 profiles as per JVET and requirements adoption.

Volunteers:

- Proponents of tools are expected to participate in conformance bitstream testing the tool.

- Additional volunteers:

  - Interdigital will contribute bitstreams for EPP.

  - Sony is open to participate in this work and cross-check.

  - Alibaba is open to participate in this work and cross-check.

  - KDDI is open to participate in this work and cross-check.

Agreement:

- Conformance bitstream will not be provided for tool in profiles which does not allow the tool.

- Clarify on tools constrains for Main12 Intra/Still Picture.

- BoG recommend including identified in Table 1 bitstreams (39 in total) to the conformance set.

- List of volunteer for VVCv2 conformance bitstream production are identified. PoC to be provided following the meeting.

To test capability of the decoder complying to Main 16 4:4:4 and Main 12 4:4:4 profiles being able to decode lower bitdepth and chroma formats, additional bitstreams are provided in Table 2 and Table 3 for Main 16 4:4:4 and Main 12 4:4:4 respectively.

Table 2 and 3 specify addition bitstream per category: 1 per description with all tools VVCv2 enabled, unless it is constrained by bitstream constrains e.g. function of bitdepth.

It was discussed, and agreed that no additional bitstreams will be provided for Intra and Still Picture profiles, since Main 16 4:4:4 and Main 12 4:4:4 being a super set of those.

Table 2: Additional chroma formats and bit depths Main 16 4:4:4 profile

| Tool description | Feature Name | Bitstream features |
| --- | --- | --- |
| 8b 4:0:0 | 8b400P16 | 8-bit 4:0:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:2:0 | 8b420P16 | 8-bit 4:2:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:2:2 | 8b422P16 | 8-bit 4:2:2 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:4:4 | 8b444P16 | 8-bit 4:4:4 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:0:0 | 10b400P16 | 10-bit 4:0:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:2:0 | 10b420P16 | 10-bit 4:2:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:2:2 | 10b422P16 | 10-bit 4:2:2 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:4:4 | 10b444P16 | 10-bit 4:4:4 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:0:0 | 12b400P16 | 12-bit 4:0:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:2:0 | 12b420P16 | 12-bit 4:2:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:2:2 | 12b422P16 | 12-bit 4:2:2 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:4:4 | 12b444P16 | 12-bit 4:4:4 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 16b 4:0:0 | 16b400P16 | 16-bit 4:0:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 16b 4:2:0 | 16b420P16 | 16-bit 4:2:0 in Main 16 4:4:4 profile, VVCv2 tools set, RA |
| 16b 4:2:2 | 16b422P16 | 16-bit 4:2:2 in Main 16 4:4:4 profile, VVCv2 tools set, RA |

Table 3: Additional chroma formats and bit depths Main 12 4:4:4 profile

| Tool description | Feature Name | Bitstream features |
| --- | --- | --- |
| 8b 4:0:0 | 8b400P12 | 8-bit 4:0:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:2:0 | 8b420P12 | 8-bit 4:2:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:2:2 | 8b422P12 | 8-bit 4:2:2 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 8b 4:4:4 | 8b444P12 | 8-bit 4:4:4 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:0:0 | 10b400P12 | 10-bit 4:0:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:2:0 | 10b420P12 | 10-bit 4:2:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:2:2 | 10b422P12 | 10-bit 4:2:2 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 10b 4:4:4 | 10b444P12 | 10-bit 4:4:4 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:0:0 | 12b400P12 | 12-bit 4:0:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:2:0 | 12b420P12 | 12-bit 4:2:0 in Main 12 4:4:4 profile, VVCv2 tools set, RA |
| 12b 4:2:2 | 12b422P12 | 12-bit 4:2:2 in Main 12 4:4:4 profile, VVCv2 tools set, RA |

### Discussion:

- Table above specify addition bitstreams: 1 per category with all tools VVCv2 enabled, unless it is constrained by bitstream constrains e.g. function of bitdepth.

- Agreed.

- One expert suggested if onion structure can be reused to reduce the number of conformance bitstreams.

Main16 is capable to decoder Main 12, therefore lower bitdepth and lower chroma formats with profile idc Main12 could be decoded by Main16 compliant decoder.

- Agreement:  Proposal was discussed, but group agreed to include full set of bitstreams for each profile.

- Agreed.

- One expert suggested to include 8 bit video data.

- Agreed.

Agreement:

- BoG recommends including 15 additional bitstreams in Table 2 and 11 additional bitstreams in Table 3 to the conformance test set.

# Suggested example of the bitstream description:

##### Test bitstream TOOL16_A_PROPONENT

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and TOOL enabled. TOOL is tested with all possible luma and chroma block sizes.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with TOOL enabled.

##### Test bitstream TOOL12_A_PROPONENT

Specification: 12 bits video data with 4:4:4 chroma format is being coded with various random access picture types and TOOL enabled. TOOL is tested with all possible luma and chroma block sizes.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with TOOL enabled.

### Discussion:

##### Example of bitstream description provided below are reworded description utilized in VVCv1 conformance bitstream description. Proponents are welcome to provide more information in the description.

One expert suggested that proponents are expected to know their tool well, and are expected to provide conformance bitstream testing corner cases, and define test conditions, e.g. range of QPs.

- Agreed.

One expert mentioned that conformance bitstreams for VVCv1 generation had made a study on coverage of usecases provided by specific bitstreams. Those finding, conclusion may be useful to generation of VVCv2. It is expected that if cross-checker or other expert will identify that a certain bitstream does not provide a full coverage of use cases, proponent or volunteer is invited to provide additional bitstreams covering missing cases.

- Agreed.

# Complete list of suggested VVCv2 bitstreams

Table 4.1. Complete list of suggested VVCv2 conformance bitstreams, tests on coding tools.

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 | VVC1 tools | VVC1 tools | 12b420vvc1 | 12-bit 4:2:0, VVCv1 tools set |
| Main 12 Intra | VVC1 tools | VVC1 tools | 12b420Ivvc1 | 12-bit 4:2:0, VVCv1 tools set |
| Main 12 Still Picture | VVC1 tools | VVC1 tools | 12b420SPvvc1 | 12-bit 4:2:0, VVCv1 tools set |
| Main 12 4:4:4 | VVC1 tools | VVC1 tools | 12b444vvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444errc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444prrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444etsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444epp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 12b444rlsc | 12-bit 4:4:4, VVCv2 set, RLSC test |
| Main 12 4:4:4 Intra | VVC1 tools | VVC1 tools | 12b444Ivvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444Ierrc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444Iprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444Ietsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444Iepp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 12b444Irlsc | 12-bit 4:4:4, VVCv2 set, RLSC test |
| Main 12 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 12b444SPvvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444SPerrc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444SPprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444SPetsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444SPepp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 12b444SPrlsc | 12-bit 4:4:4, VVCv2 set, RLSC test |
| Main 16 4:4:4 | VVC1 tools | VVC1 tools | 16b444vvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444errc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444prrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444etsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444epp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 16b444rlsc | 16-bit 4:4:4, VVCv2 set, RLSC test |
| Main 16 4:4:4 Intra | VVC1 tools | VVC1 tools | 16b444Ivvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444Ierrc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444Iprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444Ietsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444Iepp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 16b444Irlsc | 16-bit 4:4:4, VVCv2 set, RLSC test |
| Main 16 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 16b444SPvvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444SPerrc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444SPprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444SPetsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444SPepp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSC | RLSC | 16b444SPrlsc | 16-bit 4:4:4, VVCv2 set, RLSC test |

Table 4.2. Complete list of suggested VVCv2 conformance bitstreams, tests on chroma formats and bitdepth.

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 16 4:4:4 | Chroma format and bit depth | 8b 4:0:0 | 8b400P16 | 8-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 8b 4:2:0 | 8b420P16 | 8-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 8b 4:2:2 | 8b422P16 | 8-bit 4:2:2, VVCv2 tools set*, RA |
|  |  | 8b 4:4:4 | 8b444P16 | 8-bit 4:4:4, VVCv2 tools set*, RA |
|  |  | 10b 4:0:0 | 10b400P16 | 10-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 10b 4:2:0 | 10b420P16 | 10-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 10b 4:2:2 | 10b422P16 | 10-bit 4:2:2, VVCv2 tools set*, RA |
|  |  | 10b 4:4:4 | 10b444P16 | 10-bit 4:4:4, VVCv2 tools set*, RA |
|  |  | 12b 4:0:0 | 12b400P16 | 12-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 12b 4:2:0 | 12b420P16 | 12-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 12b 4:2:2 | 12b422P16 | 12-bit 4:2:2, VVCv2 tools set, RA |
|  |  | 12b 4:4:4 | 12b444P16 | 12-bit 4:4:4, VVCv2 tools set, RA |
|  |  | 16b 4:0:0 | 16b400P16 | 16-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:0 | 16b420P16 | 16-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:2 | 16b422P16 | 16-bit 4:2:2, VVCv2 tools set, RA |
| Main 12 4:4:4 | Chroma format and bit depth | 8b 4:0:0 | 8b400P12 | 8-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 8b 4:2:0 | 8b420P12 | 8-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 8b 4:2:2 | 8b422P12 | 8-bit 4:2:2, VVCv2 tools set*, RA |
|  |  | 8b 4:4:4 | 8b444P12 | 8-bit 4:4:4, VVCv2 tools set*, RA |
|  |  | 10b 4:0:0 | 10b400P12 | 10-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 10b 4:2:0 | 10b420P12 | 10-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 10b 4:2:2 | 10b422P12 | 10-bit 4:2:2, VVCv2 tools set*, RA |
|  |  | 10b 4:4:4 | 10b444P12 | 10-bit 4:4:4, VVCv2 tools set*, RA |
|  |  | 12b 4:0:0 | 12b400P12 | 12-bit 4:0:0, VVCv2 tools set*, RA |
|  |  | 12b 4:2:0 | 12b420P12 | 12-bit 4:2:0, VVCv2 tools set*, RA |
|  |  | 12b 4:2:2 | 12b422P12 | 12-bit 4:2:2, VVCv2 tools set, RA |

Table 4.2 specifies addition bitstreams with all tools VVCv2 enabled, unless it is constrained by the bitstream constrains e.g. function of bitdepth.