| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W2026-v1 |
| --- | --- |

| Title: | Conformance testing for VVC operation range extensions (Draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Hong-Jheng Jhu Iole Moccagatta Mohammed Sarwer Yue Yu Tianyang Zhou | Email: | dmytror@qti.qualcomm.com jhuhong-jheng@kwai.com iole.moccagatta@intel.com m.sarwer@alibaba-inc.com yue.yu@oppo.com zhou.tianyang@sharp.co.jp |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 1 of the conformance testing specification for Versatile Video Coding (VVC) version 2: operation range extensions, ISO/IEC 23090-3:2021/CDAmd.1.

Due to the size of the electronic attachments, the associated conformance test set data files are provided separately, at the following location:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2/draft_conformance/draft1/

These are also available by ftp at:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2/draft_conformance/draft1/

(user id: avguest, password: Avguest201007)

For ftp access, it is suggested to use FileZilla, for which the site manager feature should be used and ftp encryption should be set to “Require implicit ftp over TLS”.

Annex A of this document contains JVET guidelines for contributing to the conformance bitstream test set. It is not expected to be published as part of the text approved by the parent bodies.

Editorial Notes:

Introduced changes:

- ISO/IEC 23090-3 -> ISO/IEC 23090-3:2021/CDAmd.1

- Versatile video coding -> Versatile video coding — AMENDMENT 1: Operation range extensions

Working items:

- ITU-T H.266.2 - > TBD

- Document number for reference SW TBD.

- Cross-check the list of bitstreams.

- Cross-check marked in yellow highlight description.

- Produce description for missing bitstreams.

Status check on editing:

Table 4 of JVET-W0188 provides list of bitstream.

Yellow highlight – text is provided, green – text is cross-checked.

2021.07.15:  Description for 13 bitstreams is provided, 37are missing.

Table 1. Complete list of suggested VVCv2 conformance bitstreams, tests on coding tools.

| Profile | Category | Tool description | Feature Name | Bitstream features |  |
| --- | --- | --- | --- | --- | --- |
| Main 12 | VVC1 tools | VVC1 tools | 12b420vvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 Intra | VVC1 tools | VVC1 tools | 12b420Ivvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 Still Picture | VVC1 tools | VVC1 tools | 12b420SPvvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 4:4:4 | VVC1 tools | VVC1 tools | 12b444vvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444errc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444prrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444etsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444epp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444rlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 12 4:4:4 Intra | VVC1 tools | VVC1 tools | 12b444Ivvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444Ierrc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444Iprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444Ietsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444Iepp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444Irlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 12 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 12b444SPvvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444SPerrc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444SPprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444SPetsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444SPepp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444SPrlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 | VVC1 tools | VVC1 tools | 16b444vvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444errc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444prrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444etsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444epp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444rlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 Intra | VVC1 tools | VVC1 tools | 16b444Ivvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444Ierrc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444Iprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444Ietsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444Iepp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444Irlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 16b444SPvvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444SPerrc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444SPprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444SPetsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444SPepp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444SPrlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 | Chroma format and bit depth | 10b 4:4:4 | 10b444P16 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P16 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P16 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P16 | 12-bit 4:2:2, VVCv2 tools set, RA |
|  |  | 12b 4:4:4 | 12b444P16 | 12-bit 4:4:4, VVCv2 tools set, RA |
|  |  | 16b 4:0:0 | 16b400P16 | 16-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:0 | 16b420P16 | 16-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:2 | 16b422P16 | 16-bit 4:2:2, VVCv2 tools set, RA |
| Main 12 4:4:4 | Chroma format and bit depth | 10b 4:4:4 | 10b444P12 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P12 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P12 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P12 | 12-bit 4:2:2, VVCv2 tools set, RA |

Conformance specification for ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 Versatile Video Coding — AMENDMENT 1: Operation range extensions

# Scope

This Recommendation | International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	Recommendation ITU-T H.266 (in force), Versatile video coding .

–	ISO/IEC 23090-3:2021/CDAMD.1: in force, Information technology – Coded representation of immersive media– Part 3: Versatile video coding — AMENDMENT 1: Operation range extensions.

–	Recommendation ITU-T H.266.2 (in force), Reference software for ITU-T H.266 Versatile video coding.

–	ISO/IEC 23090-16: in force, Information technology – Coded representation of immersive media – Part 16: Reference software for versatile video coding.

# Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 (particularly in clause 3) apply. The following terms are further clarified for purposes herein as follows.

3.1	bitstream: A Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 video bitstream.

3.2	decoder: A Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 video decoder, i.e., an embodiment of the decoding process specified by Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	encoder: An embodiment of a process, not specified in this Recommendation | International Standard (except in regard to identification of the reference software encoder), that produces a bitstream.

3.4	reference software decoder: The software decoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16.

3.5	reference software encoder: The software encoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16.

# Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms specified in clause 4 of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 and the following abbreviations apply.

AMVP: Adaptive motion vector prediction

CCLM: Cross-component linear model

CIIP: Combined inter/intra prediction

CST: Chroma separate tree

CTC: Common test conditions

DCT: Discrete cosine transform

EPP: Extended Precision Processing

ERRC: Extended Regular Residual Coding

DMVR: Decoder-side motion vector refinement

DQ: Dependent quantization

DST: Discrete sine transform

FTP: File transfer protocol

ISP: Intra subblock partitioning

JCCR: Joint coding of chroma residuals

MMVD: Merge with MVD

MPM: Most probable mode

MVD: Motion vector difference.

PDPC: Position-dependent (intra) prediction combination.

PRRC: Persistent Regular Residual Coding

RPR: Reference picture resampling

RLSCP: Reversed last significant coefficient position

SAD: Sum of Absolute Differences

SBT: Subblock transform

SbTMVP: Subblock based temporal motion vector prediction.

SMVD: Symmetric MVD

TMVP: Temporal motion vector prediction.

ETSRC: Extended Transform Skip Residual Coding

# Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

# Conformance testing for ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1

## Introduction

The following clauses specify normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard and the reference software decoder specified in Rec. ITU-T H.266.2 | ISO/IEC 23090-16.

## Bitstream conformance

Bitstream conformance for Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 is specified by clause C.4 of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

## Decoder conformance

Decoder conformance for Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 is specified by clause C.5 of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

## Procedure to test bitstreams

A bitstream that claims conformance with Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 shall pass the following normative test.

The bitstream shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.  When a bitstream containing decoded picture hash SEI messages is processed by the reference software decoder, the reference software decoder’s hash value for each output picture shall match the hash value signalled in the corresponding decoded picture hash SEI message.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming to the video layer, i.e., that it does indeed meet all the requirements for the video layer (except Annexes C, D and E) specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 including the hypothetical reference decoder (HRD) conformance (based on Annexes C, D and E). These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

A verifier may not necessarily perform all stages of the decoding process specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream has values of general_profile_idc, general_tier_flag, and general_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile, tier, and level is required in Annex A of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 to properly perform the decoding process.

### Contents of the bitstream file

The conformance bitstreams are included in this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

- *.bit – bitstream (mandatory)

- *.txt – description (mandatory)

- *.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

- *.md5 – MD5sum of the bitstream file (mandatory)

- *.opl – output picture log (mandatory)

- *.cfg – config file used to generate bitstream with VTM encoder software (optional, not applicable if VTM encoder release version not used)

### Requirements on output of the decoding process and timing

Two classes of decoder conformance are specified:

–	output order conformance; and

–	output timing conformance.

The output of the decoding process is specified in clause 8 and Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

For output order conformance, it is a requirement that all of the decoded pictures specified for output in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Recommendations (informative)

This clause does not form an integral part of this Recommendation | International Standard.

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations specified in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 that are not an integral part of that Recommendation | International Standard. This clause discusses some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream.

### Static tests for output order conformance

Static tests of a video decoder require testing of the decoded samples, and can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process. The test will check that the values of the samples decoded by the decoder under test shall be identical to the values of the samples decoded by the reference decoder. A hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results as the hash values provided in the .opl files associated with the bitstream.

### 6.5.6	Dynamic tests for output timing conformance

Dynamic tests are applied to check that all the decoded samples are output and that the timing of the output of the decoder's decoded samples conforms to the specification of clause 8 and Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1, and to verify that the HRD models (as specified by the CPB and DPB specification in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1) are not violated when the bits of the bitstream are delivered at the proper rate.

The dynamic test is often easier to perform on a complete decoding system, which may include a systems decoder, a video decoder and a display process. It may be possible to record the output of the display process and to check that display order and timing of decoded pictures are correct at the output of the display process. However, since the display process is not within the normative scope of Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1, there may be cases where the output of the display process differs in timing or value even though the video decoder is conforming. In this case, the output of the video decoder itself (before the display process) would need to be captured in order to perform the dynamic tests on the video decoder. In particular the output order and timing of the decoded pictures shall be correct.

If buffering period and picture timing SEI messages are included in the test bitstream, HRD conformance shall be verified using the values of nal_initial_cpb_removal_delay, nal_initial_cpb_removal_offset, au_cpb_removal_delay_minus1 and pic_dpb_output_delay that are included in the bitstream.

If buffering period and picture timing SEI messages are not included in the bitstream, the following inferences shall be made to generate the missing parameters:

–	fixed_pic_rate_general_flag[ i ]  shall be inferred to be equal to 1.

–	low_delay_hrd_flag[ i ] shall be inferred to be equal to 0.

–	cbr_flag[ subLayerId ][ j ] shall be inferred to be equal to 0.

–	The frame rate of the bitstream shall be inferred to be equal to the frame rate value specified in the .txt file for the bitstream. If this is missing, then a frame rate of either 25 or 30000 ÷ 1001 can be inferred.

–	The bit rate of the bitstream shall be inferred to be equal to the maximum value for the level specified in Table 136 in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

–	CPB and DPB sizes shall be inferred to be equal to the maximum value for the level specified in Table 135 in Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1.

With the above inferences, the HRD shall be operated as follows.

–	The CPB is filled starting at time t = 0, until it is full, before removal of the first access unit. This means that the bp_nal_initial_cpb_removal_delay[ i ][ j ] shall be inferred to be equal to the total CPB buffer size divided by the bit rate divided by 90000 (rounded downwards) and bp_vcl_initial_cpb_removal_offset[ i ][ j ] shall be inferred to be equal to zero.

–	The first access unit is removed at time t = bp_nal_initial_cpb_removal_delay[ i ][ j ] ÷ 90000 and subsequent access units are removed at intervals based on the picture distance

–	Using these inferences, the CPB will not overflow or underflow and the DPB will not overflow.

### 6.5.7	Decoder conformance test of a particular profile, tier, and level

In order for a decoder of a particular profile, tier, and level to claim output order conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 as specified by this Recommendation | International Standard, the decoder shall successfully pass the static test specified in clause 6.5.5 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile, tier, and level combination.

In order for a decoder of a particular profile, tier, and level to claim output timing conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3:2021/CDAMD.1 as specified by this Recommendation | International Standard, the decoder shall successfully pass both the static test specified in clause 6.5.5 and the dynamic test specified in clause 6.5.6 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile, tier, and level. Tables 1 through 5 specify the normative test suites. The profile, tier, and level combination are described in the tables or in the .txt file associated with the bitstream.

## Specification of the test bitstreams

### General

List of the defined VVCv2 conformance bitstreams with a short description and feature name is provided in Table 1.

Table 1. Summary of defined VVCv2 conformance bitstreams.

| Profile | Category | Tool description | Feature Name | Bitstream features |  |
| --- | --- | --- | --- | --- | --- |
| Main 12 | VVC1 tools | VVC1 tools | 12b420vvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 Intra | VVC1 tools | VVC1 tools | 12b420Ivvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 Still Picture | VVC1 tools | VVC1 tools | 12b420SPvvc1 | 12-bit 4:2:0, VVCv1 tools set |  |
| Main 12 4:4:4 | VVC1 tools | VVC1 tools | 12b444vvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444errc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444prrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444etsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444epp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444rlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 12 4:4:4 Intra | VVC1 tools | VVC1 tools | 12b444Ivvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444Ierrc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444Iprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444Ietsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444Iepp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444Irlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 12 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 12b444SPvvc1 | 12-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 12b444SPerrc | 12-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 12b444SPprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 12b444SPetsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 12b444SPepp | 12-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 12b444SPrlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 | VVC1 tools | VVC1 tools | 16b444vvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444errc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444prrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444etsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444epp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444rlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 Intra | VVC1 tools | VVC1 tools | 16b444Ivvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444Ierrc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444Iprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444Ietsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444Iepp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444Irlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 16b444SPvvc1 | 16-bit 4:4:4, VVCv1 tools set |  |
|  | ERRC | ERRC | 16b444SPerrc | 16-bit 4:4:4, VVCv2 set, ERRC test |  |
|  | PRRC | PRRC | 16b444SPprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |  |
|  | ETSRC | ETSRC | 16b444SPetsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |  |
|  | EPP | EPP | 16b444SPepp | 16-bit 4:4:4, VVCv2 set, EPP test |  |
|  | RLSCP | RLSCP | 16b444SPrlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |  |
| Main 16 4:4:4 | Chroma format and bit depth | 10b 4:4:4 | 10b444P16 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P16 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P16 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P16 | 12-bit 4:2:2, VVCv2 tools set, RA |
|  |  | 12b 4:4:4 | 12b444P16 | 12-bit 4:4:4, VVCv2 tools set, RA |
|  |  | 16b 4:0:0 | 16b400P16 | 16-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:0 | 16b420P16 | 16-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:2 | 16b422P16 | 16-bit 4:2:2, VVCv2 tools set, RA |
| Main 12 4:4:4 | Chroma format and bit depth | 10b 4:4:4 | 10b444P12 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P12 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P12 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P12 | 12-bit 4:2:2, VVCv2 tools set, RA |

### Test bitstreams – Main 12 profile

#### VVCv1 tools set

##### Test bitstream 12b420vvc1_A_PROP

Specification: The bitstream is 12-bit 4:2:0 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 12-bit 4:2:0 content.

### Test bitstreams – Main 12 Intra profile

#### VVCv1 tools set

##### Test bitstream 12b420Ivvc1_A_PROP

Specification: The bitstream is 12-bit 4:2:0, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 12-bit 4:2:0 content.

### Test bitstreams –  Main 12 Still Picture profile

#### VVCv1 tools set

##### Test bitstream 12b420SPvvc1_A_PROP

Specification: The bitstream contains a single picture in 12-bit 4:2:0 chroma format and exercises MinCr restriction for level 2.0, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

### Test bitstreams –Main 12 4:4:4 profile

#### VVCv1 tools set

##### Test bitstream 12b444vvc1_A_PROP

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

### Test bitstreams –  Main 12 4:4:4 Intra profile

#### VVCv1 tools set

##### Test bitstream 12b444Ivvc1_A_PROP

Specification: The bitstream is 12-bit 4:4:4, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

### Test bitstreams –  Main 12 4:4:4 Still Picture profile

#### VVCv1 tools set

Test bitstream 12b444SPvvc1_A_PROP

Specification: The bitstream contains a single picture in 12-bit 4:4:4 chroma format and exercises MinCr restriction for level 2.0, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

### Test bitstreams –  Main 16 4:4:4 profile

#### VVCv1 tools set

##### Test bitstream 16b444vvc1_A_PROP

Specification: The bitstream is 16-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 16-bit 4:4:4 content.

##### Extended RRC (ERRC)

##### Test bitstream 16b444errc_A_Qualcomm

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and ERRC enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

#### Persistent RRC (PRRC)

##### Test bitstream 16b444prrc_A_Qualcomm

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and PRRC enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

#### Extended TSRC (ETSRC)

##### Test bitstream 16b444etsrc_A_Kwai

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and ETSRC enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

#### Extended Precision Processing (EPP)

##### Test bitstream 16b444epp_A_Sharp

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and EPP enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with EPP enabled.

#### Reversed Last Significant Coefficient Position (RLSCP)

##### Test bitstream 16b444rlscp_A_OPPO

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and RLSCP enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

### Test bitstreams –  Main 16 4:4:4 Intra profile

#### VVCv1 tools set

##### Test bitstream 16b444Ivvc1_A_PROP

Specification: The bitstream is 16-bit 4:4:4, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder

Purpose: Check that the decoder can properly decode 16-bit 4:4:4 content.

### Test bitstreams –  Main 16 4:4:4 Still Picture profile

#### VVCv1 tools set

##### Test bitstream 16b444SPvvc1_A_PROP

Specification: The bitstream contains a single picture in 16-bit 4:4:4 chroma format and exercises MinCr restriction for level 2.0, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

# Annex A
Conformance bitstream generation guidelines

  - General

This annex provides conformance bitstream generation guidelines.

  - Procedure

    - FTP site

An FTP site at ITU-T is used to exchange bitstreams. The site for FTP download of bitstreams is:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

(user id: avguest, password: Avguest201007)

FTP encryption should be set to “Require implicit ftp over TLS”. If using FileZilla, set user id and FTP encryption by configuring site manager as follows:

The site is also accessible by HTTP (without a password) at:

http://wftp3.itu.int/av-arch/jvet-site/bitstream_exchange/VVCv2

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation will be available at this directory. It includes the list of bitstreams, configuration features and settings, and status of verification.

The FTP site for uploading bitstream files is as follows:

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, password: Avguest201007)

Once a bitstream is uploaded to the dropbox, please send email to jvet-conformance@lists.rwth-aachen.de  to notify the coordinators of its availability.

    - Bitstreams and associated files

Volunteers should upload the bitstream and associated files in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

*.bit – bitstream as described in section A.2.3 (mandatory)

*.txt – description (mandatory)

*.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

*.md5 – MD5sum of the bitstream file (mandatory)

*.opl – output picture log as described in section A.2.4 (mandatory)

*.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

All bitstreams are required to use the decoded picture hash SEI message.

All files in the zip archive should be in the top level, without a subfolder.

The .md5 file should contain only the MD5sum value and no additional characters.

The following naming convention is to be used.

Feature_BistreamID_Source_Version

- Feature is summarized in Table 1

- BitstreamID is a single capital letter, to distinguish among multiple bitstreams provided for a particular feature.

- Source is the name of the organization who provided the bitstream.

- Version is the version number of the file. The version number should be 1 for the first version. The version number should be incremented when a bitstream is replaced because of an identified problem or updated for compatibility with a new version of the VTM.

For example, for the feature “feature”, bitstream “A”, from source “organization”, in its version “1”, the archive is named "feature_A_organizationA_1.zip", and the files would be named

feature_A_organization_1.bit

feature_A_organization_1.txt

feature_A_organization_1.yuv.md5

feature_A_organization_1.md5

feature_A_organization_1.opl

feature_A_organization_1.cfg

    - Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information is to be provided:

- Bitstream file name

- Explanation of bitstream features

- Profile, tier, and level for the bitstream

- Max picture width and height, and any additional picture width and height values used (i.e., for reference picture resample or scalability)

- Picture rate

- VTM decoder release version number able to correctly decode the bitstream with identical output, or description of VTM decoder incompatibility (preferably including a corresponding error tracking issue number.)

- VTM release version number used to generate the bitstream, if applicable

- Contact name and email of bitstream submitter

    - Output picture log file for each output layer set (name.opl)

Each output picture log file contains one row for each output picture in the bitstream, in output order.

Each row contains the following information, as Comma Separated Values (CSV).

- layer ID

- PicOrderCntVal

- Width of cropped output picture in luma samples

- Height of cropped output picture in chroma samples

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

The reference software provides support in the decoder to output an .opl file using the -opl command line argument. For example: DecoderApp -b feature_A_organization_1.bit -opl feature_A_organization_1.opl

    - Recommendations

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases. Multiple bitstreams should be provided for each feature.

It is recommended to include both low resolution (512x512) and short duration (10 frame) test bitstreams as well as typical resolutions and longer duration test bitstreams for each test category.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

In VTM software (VTM-13.0 and later), the high bit-depth operation range extension supports can be enabled by defining the following macro in the source code (TypeDef.h)

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED			1The examples of the configurations files to enable high-bit depth processing can be found in “cfg\hbd” directory of VTM software.

The test bitstreams shall not violate the profile, tier, and level indicated in the bitstream. The minimum level that the bitstream conforms to should be used.

A VTM config file should be included in the .zip file if an unmodified VTM version is used, and the command line used should be included in the .txt file. If a modified version of the VTM is used, the VTM config file should not be included.

This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The conformance bitstreams needed for this Recommendation are available at the following link:  https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2/draft_conformance/. The bitstreams can also be downloaded from the ITU-T Test Signal Database.