| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0044-v1 |
| --- | --- |

| Title: | Update on open, optimized VVC implementations VVenC and VVdeC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Schierl | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin |  |  |

_____________________________

# Abstract

This document provides updated information on features, coding efficiency and runtime for version 1.1.0 of the open VVC software encoder VVenC released in August 2021 and version 1.2.0 of the open VVC software decoder VVdeC released in September 2021.

Main changes for VVenC v1.1.0 since version 1.0.0 include:

- Faster presets: 20% for faster, 15% for fast, 7% for medium.

- Bi-prediction with CU-level weights (BCW) added as new coding tool

- Improved 2-pass Rate Control (RC) and Screen Content Coding

- Various bugfixes and improvements

Without QP adaptation for subjective optimization and 8 threads the following PSNR-based YUV BD-rates compared to HM-16.23 (GOP16+MCTF) as well as speedup factors compared to HM-16.23 and VTM-13.0 (GOP32+MCTF) are reported for different presets:

Faster		HD:   −2.5%, 160x (HM), 1200x (VTM)	UHD:   −9.3%, 190x (HM), 1300x (VTM)

Fast		HD: −21.7%,   83x (HM),   630x (VTM)	UHD: −23.8%,   97x (HM),   700x (VTM)

Medium	HD: −31.3%,   23x (HM),   170x (VTM)	UHD: −34.3%,   32x (HM),   230x (VTM)

Slow		HD: −35.1%,   7.7x (HM),    59x (VTM)	UHD: −38.0%,   11x (HM),     76x (VTM)

Slower		HD: −38.7%,   1.7x (HM),    13x (VTM)	UHD: −41.5%,   2.3x (HM),     16x (VTM)

With QP adaptation for subjective optimization and 8 threads, the following MS-SSIM-based YUV BD-rates compared to HM-16.23 (GOP16+MCTF) as well as a speedup factors compared to HM-16.23 and VTM-13.0 (GOP32+MCTF) are reported for different presets:

Faster		HD: −14.4%, 150x (HM), 1200x (VTM)	UHD: −11.0%	170x (HM), 1400x (VTM)

Fast		HD: −28.4%,   80x (HM),   630x (VTM)	UHD: −26.2%	  93x (HM),   750x (VTM)

Medium	HD: −36.3%,   22x (HM),   180x (VTM)	UHD: −36.9%,	  30x (HM),   240x (VTM)

Slow		HD: −39.9%,  7.6x (HM),     60x (VTM)	UHD: −40.7%,	  10x (HM),     82x (VTM)

Slower		HD: −42.5%,  1.6x (HM),     13x (VTM)	UHD: −43.9%,	 2.2x (HM),    18x (VTM)

Main changes for VVdeC v1.2.0 since version 1.1.2 include:

- WebAssembly to enable decoding and playback in web browsers.

- Various fixes and improvements:

  - Added and improved some SSE41 SIMD

  - Minor improvements to ALF, DMVR and LMCS processing

  - Added option to remove padding from YUV on output

# Introduction

The VVC test model (VTM) [1] serves as a common reference implementation, i.e. a test bed for evaluation and verification of proposed technologies during standardization. While VTM used to be the only publicly available encoder and decoder implementation of the VVC standard, it is aimed at correctness, completeness and readability and should not serve as a real-world example of a VVC encoder and decoder. The Fraunhofer Versatile Video Encoder (VVenC) [2][3] and Decoder (VVdeC) [4] development was initiated to provide publicly available, fast and efficient VVC software implementations. A first version 0.1 was released in September 2020 and an overview can be found in JVET-T0099 [5]. An updated overview for version 0.2.1 is provided in JVET-U0135 [6] and for version 1.0.0 in JVET-W0184 [7].

Bit-streams encoded with VVenC can be decoded by any VVC standard compliant decoder, e.g. the VTM-13.0 reference software decoder. Alternatively, VVdeC can be used.

VVenC features (version 1.1.0):

- Encoder implementation with five predefined quality/speed presets.

- Optimized VVC encoder providing speedups between 15x and 1300x over VTM-13.0 for HD and UHD test sequences depending on the chosen quality/speed preset.

- Perceptual optimization to improve subjective video quality based on the extended perceptually weighted peak signal-to-noise ratio (XPSNR) [8].

- Frame-level rate control supporting VBR encoding.

- Expert mode encoder interface available, allowing fine-grained control of the encoding process.

- Main 10 profile with all Main 10 coding tools for SCC and HDR/WCG as well as constrained reference picture resampling (RPR) encoding mode to enable low-drift adaptive resolution switching with open GOPs [9][10].

VVdeC features (version 1.2.0):

Supported:

- Main10 profile

In development/experimental:

- Other chroma sub-sampling formats (422, 444)

# VVenC - Fraunhofer Versatile Video Encoder

## Getting Started

### How to obtain VVenC?

The software is hosted at GitHub under:

https://github.com/fraunhoferhhi/vvenc

To clone the project use:

git clone https://github.com/fraunhoferhhi/vvenc

### How to build VVenC?

The software uses CMake to create platform-specific build files. A working CMake installation is required for building the software. Download CMake from http://www.cmake.org/ and install it. The following targets are supported: Windows (Visual Studio), Linux (gcc) and MacOS (clang).

How to build for Windows?

In order to compile the software for Windows, Visual Studio 15 2017 or higher and CMake Version 3.13 or higher are required. Install gnuwin32 that provides make for Windows. To build the software open a command prompt window, change into the project directory and use:

make install-release

This will create the statically linked release version of the encoder applications in the install/bin/release-static/ subdirectory.

How to build for Linux/MacOS?

In order to compile the software for Linux, gcc-5.0 or higher and CMake Version 3.13 or higher are required. For MacOS Xcode and CMake Version 3.13 or higher are required. To simplify the build process a Makefile with predefined targets is available. To build the VVenC encoder applications open a terminal, change into the project directory and use:

make install-release

This will create the statically linked release version of the encoder applications in the install/bin/release-static/ subdirectory.

### How to use VVenC?

The encoder project includes two encoder executables, a standard encoder (vvencapp) and a full featured expert encoder (vvencFFapp).

How to use the standard encoder?

The standard encoder (vvencapp) can be used in one of five predefined presets. Each preset represents a different tradeoff between encoder runtime and video quality. In the slowest preset, the encoder reaches the highest compression gain, whilst in the fastest preset the runtime is significantly decreased. These preset configurations have been determined based on the Pareto optimal configuration set of the encoder configuration space, which is detailed in [10]. A list of the main encoder command line parameters is shown in Table I.

Table I: List of important encoder options. For full list please use the --help option.

| Option | Default | Description |
| --- | --- | --- |
| --help,-h | - | Show basic help |
| --input <str> | not set | Raw yuv input file |
| --size <wxh> | 1920x1080 | Input file resolution (width x height) |
| --framerate <int> | 60 | Temporal rate of input file. Required for rate control and calculation of output bit-rate |
| --format <str> | yuv420 | Set input format to YUV 4:2:0 8bit (yuv420) or YUV 4:2:0 10bit (yuv420_10) |
| --output <str> | not set | Bit-stream output file |
| --preset <str> | medium | Select preset for specific encoding setting (faster, fast, medium, slow, slower) |
| --qp <int> | 32 | Quantization parameter (0..51) |
| --bitrate <int> | 0 | Bitrate for rate control (0 constant QP encoding rate control off, otherwise bits per second). Rate control requires correct framerate. |
| --passes <int> | 2 | Number of passes used for rate control |
| --qpa <int> | 1 | Perceptual QP adaptation (QPA) to improve subjective video quality (0: off, 1: on) |
| --hdr <str> | off | HDR mode of the input signal: SDR (off), HDR10 with PQ transfer function and BT.709 color primaries (pq or hdr10) or BT.2020 color primaries (pq_2020 or hdr10_2020), HLG transfer function and BT.709 color primaries (hlg) or BT.2020 color primaries (hlg_2020) |
| --threads <int> | size ≥ 720p: 8 else : 4 | Number of threads (1..N) |

Example usage: Given a YUV 4:2:0 input file with a bit-depth of 8bit and a resolution of 176x144 pixels, the following call will encode the input file with the medium speedup preset:

vvencapp --preset medium -i BUS_176x144_75@15.yuv -s 176x144 -o str.266

How to use the full featured expert mode encoder?

The expert mode encoder (vvencFFapp) is based on the VTM configuration scheme. Most of the parameters have been kept similar to VTM, but for some parameters, additional modes are available. Furthermore, not supported options have been removed. Example configuration files for the expert mode encoder can be found in the cfg sub-directory (see Table II).

Example usage: In order to start your first experiments with the expert mode encoder, adapt the sequence.cfg configuration file to your input YUV source file and use the following command:

vvencFFapp -c randomaccess_medium.cfg -c sequence.cfg

Table II: Expert mode encoder configuration files provided in the ./cfg sub-directory.

| Configuration File | Description |
| --- | --- |
| sequence.cfg | Sequence specific configuration parameters. Must be always adapted to the input sequence. |
| randomaccess_[faster, fast, medium, slow, slower].cfg | Random access configuration for different presets. Each configuration file corresponds to one of the 5 preset modes. |
| qpa.cfg | Perceptually optimized QPA configuration file. |
| rc1p.cfg | Single pass rate control configuration, overriding default fix qp setup. |
| rc2p.cfg | Two pass rate control configuration, overriding default fix QP setup. |

## Encoder Performance

The encoder performance tests focus on the most relevant HD (1920x1080) and UHD (3840x2160) resolution use cases, in the following denoted as HD4K use case, with random access encoding as defined in the JVET common test conditions (CTC) [11]. Unless stated otherwise, all presented results are shown for the CTC test sequences, i.e. classes A1 and A2 for UHD and class B for HD sequences. Constant QP encoding is used with QP values of 22, 27, 32 and 37 according to VTM CTC. Reported rate distortion results are calculated as Bjøntegaard delta rate (BD-rate) [12]. Multi-threading results for VVenC and x265 have been generated with 8 threads enabled. In HM and VTM multi-threading is not supported. Unless indicated otherwise, all tests have been performed on Dell Super Micro servers with Intel Xeon processors E5-2697A v4 @2.6GHz.

### PSNR Optimized Use Case

The PSNR BD-rate gain of VVenC over the HEVC test model reference software HM-16.23 is shown in Figure 1. PSNR BD-rate represents the approximate average bit-rate savings between two encoders for the same objective quality (PSNR). Here lower values mean larger bit-rate savings with respect to the HM-16.23 anchor. Please note the logarithmic scale of the relative encoder runtime in comparison to HM-16.23.

With the slower preset and multi-threading with 8 threads enabled the BD-rate gain of VVenC over HM is similar to VTM-13 common test conditions (CTC), but a speedup of more than 15x for HD4K sequences is achieved over VTM.

With the faster preset and multi-threading with 8 threads the BD-rate gain over HM is still approx. 10.7%, with a speedup of more than 1300x for HD4K over VTM. Comparing the runtime with HM gives a speedup of around 170x.

As a good tradeoff between encoder runtime and BD-rate performance, we recommend the medium preset with multi-threading enabled. Here, the BD-rate gain over HM is approx. 32.9%, which is quite close to the slower preset and VTM CTC, but in comparison to VTM-13.0 the encoder runs 200x faster for HD4K sequences. Compared to HM-16.23 this is an encoder runtime speedup of 27x. A summary of all results is shown in Table III.

Table III: PSNR BD-rate and multi-threaded (8 threads) encoder speedup for HD, UHD and both test sequences.

| Preset | HD |  |  | UHD |  |  | HD4K |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | PSNR BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | PSNR BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM |
| FASTER | -2.5% | 160x | 1200x | -9.3% | 190x | 1300x | -6.2% | 170x | 1300x |
| FAST | -21.7% | 83x | 630x | -23.8% | 97x | 700x | -22.8% | 90x | 670x |
| MEDIUM | -31.3% | 23x | 170x | -34.3% | 32x | 230x | -32.9% | 27x | 200x |
| SLOW | -35.1% | 7.7x | 59x | -38.0% | 11x | 76x | -36.7% | 9.2x | 68x |
| SLOWER | -38.7% | 1.7x | 13x | -41.5% | 2.3x | 16x | -40.2% | 2.0x | 15x |

Figure 1: PSNR BD-rate gain and relative encoder runtime in comparison to HM-16.23 (JVET HD and UHD test sequences, MCTF enabled for HM and VTM) for VVenC and VTM-13. Results are given for the 5 preset options: faster, fast, medium, slow and slower. Lower PSNR BD-rate values mean a better compression for the same objective quality in terms of PSNR. VVenC is running multi-threaded using 6 threads for version < 0.3 and 8 threads for version >0.2.1.

Additionally, Figure 4 includes multi-threaded results for the HEVC open-source encoder x265 v3.4 at comparable speed presets [13]. For the comparison with VVenC, also x265 was configured to run with 8 threads. Besides sequence-specific parameters, the following parameter settings have been used:

--preset {1,2,3,…,9}  --tune psnr  --crf {17,22,27,32}--keyint 1s  
--min-keyint 1s  --profile main10  --output-depth 10

Figure 4: PSNR BD-rate gain and relative encoder runtime in comparison to HM-16.23 (JVET HD and UHD test sequences) for VVenC v1.1.0 and x265 v3.4. Lower PSNR BD-rate values mean a better compression for the same objective quality in terms of PSNR. VVenC and x265 are running multi-threaded using 8 threads.

### Perceptually Optimized Quantization Parameter Adaptation

To improve the perceived (subjective) coding quality, VVenC supports a low-complexity quantization parameter adaptation (QPA) algorithm based on a model of the human visual system. To evaluate the quality of the perceptually optimized quantization parameter adaptation (PQPA), multiscale structural similarity measure (MS-SSIM) [14] is used.

In Figure 5 the MS-SSIM BD-rate gain of VVenC over the HEVC test model reference software HM-16.23 is shown (lower is better). For VTM simulation, common test conditions CTC with additional PQPA is used (--PerceptQPA=1). With PQPA enabled, the speedups achieved over HM and VTM are similar to the Non-PQPA results presented in the previous section. This demonstrates the low-complexity nature of the PQPA algorithm. Comparing the MS-SSIM based BD-rates, additional bitrate reduction can be achieved. Especially for the slower preset, an MS-SSIM BD-rate gain of more than 2% over VTM CTC without PQPA is realized. We recommend using the medium preset with multi-threading and PQPA enabled as a good tradeoff between encoder runtime and resulting perceived video quality. A summary of the MS-SSIM results for PQPA is shown in Table IV

Figure 5: MS-SSIM BD-rate gain and encoder runtime in comparison to HM-16.23 (JVET HD and UHD test sequences, MCTF enabled for HM and VTM) for VVenC and VTM-13, both with perceptually optimized quantization parameter adaptation (QPA) enabled. VVenC results are given for the 5 preset options: faster, fast, medium, slow and slower. Lower MS-SSIM BD-rate values mean a better compression for the same quality in terms of MS-SSIM. VVenC is running multi-threaded using 6 threads for version < 0.3 and 8 threads for version >0.2.1.

Additionally, Figure 6 includes multi-threaded results for the HEVC open-source encoder x265 v3.4 tuned for SSIM at comparable speed presets [13]. For the comparison with VVenC, also x265 was configured to run with 8 threads. Besides sequence-specific parameters, the following parameter settings have been used:

--preset {1,2,3,…,9}  --tune ssim  --crf {17,22,27,32}  --keyint 1s  
--min-keyint 1s  --profile main10  --output-depth 10

Figure 6: MS-SSIM BD-rate gain and encoder runtime in comparison to HM-16.22 (JVET HD and UHD test sequences) for VVenC v1.0.0 with QPA enabled and x265 v3.4 with --tune=ssim. Lower MS-SSIM BD-rate values mean a better compression for the same quality in terms of MS-SSIM. VVenC and x265 are running multi-threaded using 8 threads.

Table IV: MS-SSIM BD-rate gain and multi-threaded encoder speedup for HD, UHD and both test sequences for VVenC with perceptually optimized QPA enabled.

| Preset | HD |  |  | UHD |  |  | HD4K |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MS-SSIM BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | MS-SSIM BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | MS-SSIM BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM |
| FASTER | -14.4% | 150x | 1200x | -11.0% | 170x | 1400x | -12.5% | 160x | 1300x |
| FAST | -28.4% | 80x | 630x | -26.2% | 93x | 750x | -27.2% | 87x | 690x |
| MEDIUM | -36.3% | 22x | 180x | -36.9% | 30x | 240x | -36.6% | 26x | 210x |
| SLOW | -39.9% | 7.6x | 60x | -40.7% | 10x | 82x | -40.3% | 9.0x | 72x |
| SLOWER | -42.5% | 1.6x | 13x | -43.9% | 2.2x | 18x | -43.3% | 1.9x | 15x |

### Multi-Threading

In Figure 7 the multi-threading performance scaling is shown for the faster and fast presets for up to 16 threads and up to 32 threads for HD and UHD test sequences respectively (for version 0.3.0.0). Additionally, the scaling for the medium preset for up to 16 threads is given. The advanced threading since version 0.3 greatly improves scaling over earlier versions. It is apparent, that the threading performance is better with the fast and faster presets when utilizing more than 8 cores for HD or 12 for UHD.

Figure 7: Multi-threading speedup dependent on the number of used threads for version 0.3.0.0. The tests were performed on high-performance computer with an AMD EPYC 7502P 32-core processor.

While the current multi-threading approach has only minimal influence on the compression performance of less than 1% bit-rate increase for the same visual quality, there are additional options allowing to significantly increase the multi-threading performance.

When operating the encoder in an area where the speedup curve saturates, the multi-threading performance can be improved by setting some additional parameters. For all presets --WaveFrontSynchro=1 can be enabled, which reduces the encoding lag of CTU lines within a picture. Additionally, for medium, slow and slower presets, --CTUSize=64 can be set, allowing it to achieve a scaling similar to the faster and fast presets. Both options are only available in the expert app and come with a gain trade-off. The latter option is enabled by default for the fast and faster preset.

### Rate Control

To support encoding with a predefined target rate instead of fixed QP in which the final bitrate is generally unknown beforehand, VVenC includes one- and two-pass rate control. The one-pass rate control uses limited information about the sequence and is intended for applications where encoder latency is a critical factor. On the other hand, two-pass rate control includes the first pass in which the statistics for the encoded sequence are collected. This information is then used in the second pass to improve the rate control performance at a cost of the encoding time increase.

PSNR BD-rate results of both rate control versions for HD4K content over a fixed QP VVenC encoding are shown in Table V. It should be noted that 10-second versions of CTC sequences from classes A1 and A2 are used for the rate control tests. The target rates for the rate control runs were set based on the resulting target rates from the fixed QP runs. All runs were executed using 8 threads. It can be seen that the BD-rate performance of the two-pass version is almost the same as that of fixed QP encoding. At the same time, the relative complexity of the two-pass approach is decreasing as presets become slower due to the constant complexity of the first pass. The one-pass approach achieves acceptable BD-rate performance while keeping the encoding runtime overhead low, especially for faster presets. The average bitrate deviation for one-pass approach is around 1.3%, while in the two-pass case it is around 0.7%.

Table V: PSNR BD-rate and relative encoding runtime for 1- and 2-pass rate control on HD4K sequences in comparison to a VVenC fixed QP encoding for all presets. Both encoders were running with 8 threads.

|  | HD4K |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1-pass RC |  | 2-pass RC |  |
| Preset | PSNR BD-rate vs. Fixed QP | Encoding Time vs. Fixed QP | PSNR BD-rate vs. Fixed QP | Encoding Time vs. Fixed QP |
| faster | 7.65% | 114% | -2.30% | 187% |
| fast | 7.33% | 118% | 0.16% | 148% |
| medium | 7.00% | 133% | 0.44% | 122% |
| slow | 7.21% | 121% | 0.48% | 103% |
| slower | 7.37% | 126% | 0.60% | 102% |

For the improved perceived video quality, the rate control can be used in combination with PQPA. The MS-SSIM BD-rate results for a combination of rate control and QPA over a fixed QP VVenC encoding with QPA are shown in Table VI. The results are similar to the results shown in Table V, apart from higher BD-rates for one-pass rate control.

Table VI: MS-SSIM BD-rate and relative encoding runtime for 1- and 2-pass rate control on HD4K sequences in comparison to a VVenC fixed QP encoding with QPA for all presets. Both encoders were running with 8 threads.

|  | HD4K |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1-pass RC |  | 2-pass RC |  |
| Preset | MS-SSIM BD-rate vs. Fixed QP with QPA | Encoding Time vs. Fixed QP with QPA | MS-SSIM BD-rate vs. Fixed QP with QPA | Encoding Time vs. Fixed QP with QPA |
| faster | 16.63% | 110% | -1.53% | 179% |
| fast | 16.32% | 118% | 1.46% | 144% |
| medium | 15.20% | 118% | 1.41% | 114% |
| slow | 15.90% | 118% | 1.44% | 102% |
| slower | 16.21% | 122% | 1.46% | 101% |

## License

Please see the file https://github.com/fraunhoferhhi/vvenc/blob/master/LICENSE.txt in the repository for the terms of use of the contents of the VVenC repository.

For more information, please contact: vvc@hhi.fraunhofer.de

# VVdeC - Fraunhofer Versatile Video Decoder

## Getting Started

### How to obtain VVdeC?

The software is hosted at GitHub under:

https://github.com/fraunhoferhhi/vvdec

To clone the project use:

git clone https://github.com/fraunhoferhhi/vvdec

### How to build VVdeC?

The software uses CMake to create platform-specific build files. A working CMake installation is required for building the software. Download CMake from http://www.cmake.org/ and install it. The following targets are supported: Windows (Visual Studio), Linux (gcc) and MacOS (clang).

How to build for Windows?

In order to compile the software for Windows, Visual Studio 15 2017 or higher and CMake Version 3.12 or higher are required. Install gnuwin32 that provides make for Windows. To build the software open a command prompt window, change into the project directory and use:

make install-release

This will create the statically linked release version of the decoder applications in the install/bin/release-static/ subdirectory.

How to build for Linux/MacOS?

In order to compile the software for Linux, gcc-7.0 or higher and CMake Version 3.12 or higher are required. For MacOS Xcode and CMake Version 3.12 or higher are required. To simplify the build process a Makefile with predefined targets is available. To build the VVdeC decoder applications open a terminal, change into the project directory and use:

make install-release

This will create the statically linked release version of the decoder applications in the install/bin/release-static/ subdirectory.

### Testing

The software contains a simple test-suite testing the decoder conformance against the official set of conformance test sequences [16]. To run the test-suite, the project must be built enabling the download of the test-sequences:

make <target> enable-bitstream-download=1

The actual test can then be run by calling:

make test-all

### How to use VVdeC?

Standalone decoder application

In addition to the C-library, the VVdeC project provides a sample application, which allows to decode raw VVC bitstreams into raw YUV files (for 10-bit input, the output is yuv420p10le format in ffmpeg; for 8-bit input, the output is yuv420p).

Table V: List of important decoder options. For a full list please use the --help option.

| Option | Default | Description |
| --- | --- | --- |
| --help,-h | - | Show basic help |
| --bitstream <str> | - | Raw bitstream input file |
| --output <str> | - | The name of the output raw yuv file (yuv420p10le format) |
| --threads <int> | -1 | Size of the threadpool allocated for decoding. Set to 0 for single-threaded execution and -1 to allocate one thread per core available |
| --parsedelay <int> | -1 | Maximal number of parsed frames waiting for reconstruction. Increases the decoding latency but improves MT scaling. Set to -1 to allow one parse frame delay per core available. |
| --Seidecodedpicturehash | not set | If the bitstream contains Decoded Picture Hash SEI information, the switch enables the decoder to check the values against the reconstruction |
| --loops <int> | 0 | Expert: decode the bitstream file multiple times |
| --verbosity <int> | 3 | Verbosity level |

Example usage: Given a compliant VVC input file str.266, the following call will decode the file into raw YUV raw_yuv.yuv:

vvdeccapp -b str.266 -o raw_yuv.yuv

Library

The VVdeC project provides an easy to use C-library that can be built either as a shared or as a static library. The interface functions are defined in the header file vvdec/vvdec.h. A quick overview of the order of function calls to initialize and use the decoder library can be found in the project wiki on GitHub:

https://github.com/fraunhoferhhi/vvdec/wiki/How-to-use-VVdeC

Alternatively, for an example of a more full featured decoder application, look into the source code of the provided vvdecapp.

WebAssembly Runtime

VVdeC explicitly supports WebAssembly as a compilation target using Emscripten. The Emscripten SDK needs to be installed, activated and in the PATH as documented on the Emscripten site (https://emscripten.org/). Then, building VVdeC for the WebAssembly runtime is straightforward:

emcmake cmake -B build/wasm

cmake --build build/wasm

The resulting binary and support files vvdecapp.wasm, vvdeapp.worker.js, and vvdecapp.js can be included in a website to build a browser based VVC player. The WASM module exposes an interface similar to the native library but slightly more object-oriented to be easier to use from JavaScript code running in the browser. The JavaScript specific call flow for using the decoder library can also be found in the project wiki:

https://github.com/fraunhoferhhi/vvdec/wiki/How-to-use-VVdeC#3-webassembly-runtime

The WebAssembly decoder needs a reasonably fast machine for decoding high-resolution videos and the performance highly depends on the used browser or JavaScript implementation. It is currently limited to a maximum resolution of FullHD due to the memory constraints of the 32bit WebAssembly runtime.

## Decoder Performance

The encoder performance tests focus on the most relevant HD (1920x1080) and UHD (3840x2160) resolution use cases with random access encoding as defined in the JVET common test conditions (CTC) [12]. All presented results are shown for the CTC test sequences, i.e. classes A1 and A2 for UHD and class B for HD sequences. The test bitstreams were encoded using the VTM-10.0 reference encoder with constant QP encoding with every second QP value between 21 and 43. The bitstreams were decoded on two different computers with different number of threads used for decoding.

### HD Results

Figure 3: The achieved FPS of the decoding of 10-bit HD sequences depending on the bitrate and number of threads used for decoding on an Intel Xeon W-2155 processor with 10 physical CPU cores.

Figure 4: The achieved FPS of the decoding of 10-bit HD sequences depending on the bitrate and number of threads used for decoding on an AMD EPYC 7502P processor with 32 physical CPU cores.

### UHD Results

Figure 5: The achieved FPS of the decoding of 10-bit UHD sequences depending on the bitrate and number of threads used for decoding on an Intel Xeon W-2155 processor with 10 physical CPU cores.

Figure 6: The achieved FPS of the decoding of 10-bit UHD sequences depending on the bitrate and number of threads used for decoding on an AMD EPYC 7502P processor with 32 physical CPU cores.

## License

Please see the file https://github.com/fraunhoferhhi/vvdec/blob/master/LICENSE.txt in the repository for the terms of use of the contents of the VVdeC repository.

For more information, please contact: vvc@hhi.fraunhofer.de

# References

- VTM software repository, version VTM-12.0. Available online: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- VVenC software repository. Available online: https://github.com/fraunhoferhhi/vvenc

- A Wieckowski et al., “Vvenc: An Open And Optimized Vvc Encoder Implementation” 2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW), 2021, pp. 1-2, doi: 10.1109/ICMEW53276.2021.9455944.

- VVdeC software repository. Available online: https://github.com/fraunhoferhhi/vvdec

- Adam Wieckowski, et al., “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations,” document JVET-T0099, Joint Video Experts Team (JVET), Oct. 2020.

- Adam Wieckowski, et al., “Update on open, optimized VVC implementations VVenC and VVdeC,” document JVET-U0135, Joint Video Experts Team (JVET), Jan. 2021.

- Adam Wieckowski, et al., “Update on open, optimized VVC implementations VVenC and VVdeC,” document JVET-W0184, Joint Video Experts Team (JVET), Jul. 2021.

- C. R. Helmrich et al., “A study of the extended perceptually weighted peak signal-to-noise ratio (XPSNR) for video compression with different resolutions and bit depths”,  in ITU Journal: ICT Discoveries, vol. 3, no. 1, May 2020. http://handle.itu.int/11.1002/pub/8153d78b-en

- R. Skupin, C. Bartnik, A. Wieckowski, Y. Sanchez, B. Bross, C. Hellge, and T. Schierl, "Open GOP Resolution Switching in HTTP Adaptive Streaming with VVC," 35th Picture Coding Symposium (PCS), Bristol, US, June-July 2021.

- R. Skupin, C. Bartnik, A. Wieckowski, Y. Sanchez, B. Bross, " Constrained RASL encoding for bitstream switching," document JVET-W0133, Joint Video Experts Team (JVET), July 2021.

- J. Brandenburg et al., “Towards Fast and Efficient VVC Encoding”, IEEE 22nd Workshop on Multimedia Signal Processing (MMSP 2020), Tampere, Finland, 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” document JVET-T2010, Joint Video Experts Team (JVET), Oct. 2020.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” Technical Report VCEG-M33, ITU-T SG16/Q6, Austin, Texas, USA, 2001.

- x265 software repository, version 3.4. Available online: https://github.com/videolan/x265/tree/Release_3.4

- Z. Wang, E. Simoncelli, and A. C. Bovik, “Multi-Scale Structural Similarity for Image Quality Assessment,” inProc. IEEE Asilomar Conf. Signals, Systems, and Comp., Pacific Grove, 2003.

- J. Boyce, E. Alshina, F. Bossen, K. Kawamura, I. Moccagatta, and W. Wan, “Conformance testing for versatile video coding (Draft 6),” document JVET-U2008, Joint Video Experts Team (JVET), Jan. 2021.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).