| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24rd Meeting, by teleconference, 7–16 Oct. 2021 | Document: JVET-X0058-v2 |
| --- | --- |

| Title: | AHG12: Bilateral matching SMVD mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Donghyun Kim Hyoungjin Kwon Jongho Kim Woong Lim Se Yoon Jeong | Tel: Email: | kimddng@etri.re.kr kwonjin@etri.re.kr pooney@etri.re.kr woong.lim@etri.re.kr jsy@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

In this contribution, bilateral matching Symmetric MVD mode (BM-SMVD) is proposed as a sub-mode of SMVD. In the proposed method, only MVP indices for list0 and list1 are signaled and MVs are refined by minimizing bilateral matching error. Performance of the proposed method on top of VTM-13.0 and ECM-2.0 are reported as follows.

- Test 1. BM-SMVD on VTM-13.0
 : -0.08% (Y), -0.03% (U), - 0.08% (V), 101% (EncT), 100% (DecT)

- Test 2. BM-SMVD on ECM-2.0 (TM-off)
 : -0.xx% (Y), -0.xx% (U), - 0.xx% (V), xxx% (EncT), xxx% (DecT)

Introduction

SMVD and DMVR have common properties as follows,

- Both require reference pictures with different temporal directions and equal POC distance.

- In SMVD, MVD0 is signaled and MVD1 is set equal to (-MVD0).

- For DMVR, MV_offset is implicitly derived by minimizing bilateral matching (BM) error. MV0’ and MV1’ are set equal to (MV0 + MV_offset) and (MV1 – MV_offset), respectively.

In this contribution, BM-SMVD mode is proposed as a sub-mode of SMVD mode to reduce the signaling overhead of MVD0 in SMVD.

Proposed method

Whereas mvp_l0_flag, mvp_l1_flag, and MVD0 are signaled in the current SMVD of VVC, only mvp_l0_flag and mvp_l1_flag are explicitly signaled in the proposed BM-SMVD and MV differences are implicitly derived using BM. In the proposed method, BM-SMVD flag is signaled only when SMVD flag is true. When BM-SMVD flag is true, the indices of AMVR and BCW are set to default indices (IMV_OFF and BCW_DEFAULT, respectively).

BM-SMVD on VTM-13.0

For BM refinement, decoder-side motion vector refinement (DMVR) in VVC is used without weighting the SAD cost of initial MV, and Bi-Directional Optical Flow (BDOF) is used for further refinement. BM-SMVD mode is only enabled when both width and height of the current block are greater than or equal to 8. Moreover, to avoid a latency issue due to the inter-dependency between the current block and its neighboring ones [1], the use of the refined motion vectors (MVs) for spatial MV propagation is disabled, as in DMVR.

BM-SMVD on ECM-2.0

On ECM-2.0, DMVR is replaced by Multi-pass DMVR (MPDMVR). Unlike DMVR in VVC, MPDMVR uses refined MVs for both spatial and temporal motion vector propagation. Since MPDMVR is used for BM of BM-SMVD, the refined MVs are used for both temporal and spatial propagation in BM-SMVD. BM-SMVD is enabled without any constraint on the size of the current block. LIC and OBMC are disabled when the current block is BM-SMVD mode.

Simulation results

Performances were evaluated for random access configuration under common test conditions (CTC) [2,3].

- Test 1. BM-SMVD on VTM-13.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.11% | -0.05% | 101% | 100% |
| Class A2 | -0.09% | 0.00% | -0.14% | 101% | 100% |
| Class B | -0.08% | -0.02% | -0.02% | 102% | 100% |
| Class C | -0.06% | 0.01% | -0.14% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.03% | -0.08% | 101% | 100% |
| Class D | -0.11% | -0.13% | -0.07% | 100% | 99% |
| Class F |  |  |  |  |  |

- Test 2. BM-SMVD on ECM-2.0 (both anchor and test disable inter template matching tools)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.22% | -0.11% | 103% | 100% |
| Class C | -0.19% | -0.21% | -0.19% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.21% | -0.12% | -0.26% | 103% | 100% |
| Class F |  |  |  |  |  |

References

[1] C.-C. Chen, etc. “EE2-related: On spatial MV propagation and neighboring template block access for template matching and multi-pass DMVR,” JVET-W0122, Meeting by Teleconference, July 2021.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).