| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0068-v3 |
| --- | --- |

| Title: | EE2-1.1~EE2-1.4: Tests on unsymmetric partitioning methods |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang F. Le Léannec, K. Naser, T. Dumas, A. Robert, F. Galpin, E. François 8910 University Center Lane San Diego, CA 92122, USA 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com fabrice.leleannec@interdigital.com |
| Source: | Bytedance Inc. InterDigital |  |  |

_____________________________

# Abstract

This contribution presents experimental results for EE2-1.1 ~ EE2-1.4, which are tests on two new block partitioning methods: unsymmetric quad-tree (UQT) split and asymmetric binary-tree (ABT), a.k.a unsymmetric binary-tree (UBT) split. On top of ECM-2.0, simulation results (BD-rate changes on the luma component, encoding time and decoding time) are reported as below:

Test 1.1 (Encoder optimization on ECM-2.0):

AI: -0.80%, 0.83%, 0.45%, 95%, 110%; RA: -0.14%, 0.60%,0.78%, 92%, 105%; LB: -0.34%, -1.19%, -1.09%, 96%, 97%.

Test 1.2 (ABT only):

AI: -0.56%, -2.10%, -2.06%, 171%,	87%; RA: -0.34%, 0.41%, 0.54%, 99%, 105%; LB: -0.32%,	-1.03%, -1.08%,	103%, 101%.

Test 1.3 (UQT only):

AI: -0.18%, -0.12%, -0.10%, 170%, 102%; RA: -0.51%, 0.33%, 0.52%, 113%, 102%; LB: -0.27%, -0.16%, -0.34%, 147%, 102%.

Test 1.4 (UQT + ABT):

AI: -0.xx%, 1xx%, 1xx%; RA: -0.66%,	0.03%,	0.21%,	122%, 105%; LB: -0.54%, -1.02%, -1.39%, 114%, 100%.

For RA configuration, three trade-offs are provided for tests 1.1, 1.2 and 1.4, respectively, as shown in the following table.

|  | BDR-Y | BDR-U | BDR-V | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- | --- |
| Test 1.1 - Trade-off 1 | -0.16% | 0.65% | 0.80% | 92% | 103% |
| Test 1.1 - Trade-off 2 | -0.80% | -0.14% | -0.11% | 120% | 105% |
| Test 1.1 - Trade-off 3 | -1.25% | -0.77% | -0.77% | 179% | 106% |
| Test 1.2 - Trade-off 1 | -0.34% | 0.48% | 0.63% | 98% | 105% |
| Test 1.2 - Trade-off 2 | -0.61% | 0.12% | 0.24% | 108% | 102% |
| Test 1.2 - Trade-off 3 | -1.28% | -0.65% | -0.64% | 141% | 99% |
| Test 1.4 - Trade-off 1 |  |  |  |  |  |
| Test 1.4 - Trade-off 2 |  |  |  |  |  |
| Test 1.4 - Trade-off 3 |  |  |  |  |  |

# Introduction

In JVET-W0086 [1], unsymmetric quad-tree (UQT) and binary-tree (UBT) partitioning methods were proposed. In JVET-W0084 [2], the asymmetric binary-tree splitting (ABT) partitioning method was proposed, which is similar to UBT conceptually. In JVET-W0087 [3], UQT and ABT are tested jointly. In this contribution, EE tests on separate and combined implementations of ABT and UQT are reported in test 1.2 ~ test 1.4. In addition, the encoder only optimization methods accompany with UQT and ABT are reported in test 1.1.

# Proposed methods

## ABT

Four types of ABT are proposed as shown in Fig. 1. ABT-H1 and ABT-H2 split a CU with dimension W×H into two child-CUs with dimensions {W×H/4, W×3H/4} and {W×3H/4, W×H/4}, respectively. ABT-V1 and ABT-V2 split a CU with dimension W×H into two child-CUs with dimensions {W/4×H, 3W/4×H} and {3W/4×H, W/4×H}, respectively.

## UQT

Four types of UQT are proposed as shown in Fig. 2. UQT-H1 and UQT-H2 split a CU with dimension W×H into four child-CUs with dimensions {W×H/8, W×H/2, W×H/4, W×H/8} and {W×H/8, W×H/4, W×H/2, W×H/8}, respectively. UQT-V1 and UQT-V2 split a CU with dimension W×H into four child-CUs with dimensions {W/8×H, W/2×H, W/4×H, W/8×H} and {W/8×H, W/4×H, W/2×H, W/8×H}, respectively.

| ABT-H1 | ABT-H2 |
| --- | --- |
| ABT-V1 | ABT-V2 |

Fig. 1 ABTs.

| UQT-H1 | UQT-H2 |
| --- | --- |
| UQT-V1 | UQT-V2 |

Fig. 2 UQTs.

## Constraints and optimizations

For all tests 1.1 to 1.4, the following coding tree configurations are used in order to optimize the trade-off between encoding time and coding efficiency.

- The maximum MTT hierarchy depth can be configured by temporal layers in the RA and LDB configurations. To do so, a maxim MTT hierarchy depth by temporal ID parameter, called MaxMTTHierarchyDepthByTid, is signaled at sequence level and specifies the maximum MTT hierarchy depth value for B pictures in each temporal layer.

- In random access configuration, the encoding partitioning related coding parameters are differentiated by class, as described in the following table.

| Class A | Class B | Classes C and D |
| --- | --- | --- |
| CTUSize: 256 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxMTTHierarchyDepthByTid: 333333 MaxBTLumaISlice: 64 MaxBTChromaISlice: 64 MaxBTNonISlice: 128 MaxTTLumaISlice: 32 MaxTTChromaISlice: 32 MaxTTNonISlice: 64 MaxABTLumaISlice: 64 MaxABTChromaISlice: 64 MaxABTNonISlice: 64 |

In addition to this encoder configuration commonly used for encoder only test and normative test, some heuristics specific to ABT/UQT partitioning are used, as described in contribution JVET-W0087.

# Experimental results

Four simulations are tested on top of ECM-2.0 [4], according to the common test conditions [5] as below:

Test 1.1: Encoder optimization on ECM-2.0;

Test 1.2: ABT only;

Test 1.3: UQT only;

Test 1.4: UQT + ABT.

The results of test 1.1 ~ test 1.4 are tabulated in Table 1 ~ Table 4, respectively.

Table 1: Performance of Test 1.1 (Encoder only)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -3.43% | -3.84% | 185% | 79% | -0.55% | 0.95% | 0.88% | 88% | 107% |  |  |  |  |  |
| Class A2 | -0.64% | -2.83% | -1.96% | 143% | 78% | -0.39% | -0.14% | 0.53% | 91% | 103% |  |  |  |  |  |
| Class B | -0.50% | -2.21% | -2.41% | 147% | 94% | 0.22% | 0.62% | 0.55% | 89% | 106% | -0.39% | -0.88% | -0.73% | 96% | 89% |
| Class C | -0.07% | -0.54% | -0.60% | 122% | 69% | -0.19% | 1.06% | 1.27% | 101% | 96% | 0.00% | -0.02% | 0.17% | 92% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  | -0.73% | -3.28% | -3.35% | 102% | 101% |
| Overall |  |  |  |  |  | -0.16% | 0.65% | 0.80% | 92% | 103% | -0.34% | -1.19% | -1.09% | 96% | 97% |
| Class D | -0.01% | -0.31% | -0.33% | 138% | 87% | -0.10% | 0.24% | 0.88% | 104% | 102% | 0.20% | -0.02% | -0.42% | 86% | 93% |

Table 2: Performance of Test 1.2 (ABT only, default trade-off)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -3.32% | -3.70% | 207% | 83% | -0.80% | 0.83% | 0.45% | 95% | 110% |  |  |  |  |  |
| Class A2 | -0.70% | -2.91% | -1.97% | 165% | 83% | -0.54% | -0.34% | 0.35% | 101% | 107% |  |  |  |  |  |
| Class B | -0.54% | -2.26% | -2.46% | 173% | 104% | -0.08% | 0.31% | 0.40% | 96% | 105% | -0.40% | -0.70% | -0.54% | 101% | 96% |
| Class C | -0.13% | -0.62% | -0.68% | 153% | 79% | -0.19% | 1.06% | 1.27% | 101% | 100% | -0.07% | -0.19% | -0.30% | 102% | 105% |
| Class E | -0.54% | -1.75% | -1.69% | 168% | 80% |  |  |  |  |  | -0.52% | -2.69% | -3.02% | 108% | 104% |
| Overall | -0.56% | -2.10% | -2.06% | 171% | 87% | -0.34% | 0.48% | 0.63% | 98% | 105% | -0.32% | -1.03% | -1.08% | 103% | 101% |
| Class D | -0.03% | -0.29% | -0.36% | 168% | 104% | -0.10% | 0.24% | 0.88% | 109% | 112% | 0.14% | -0.91% | -0.38% | 99% | 98% |

Table 3: Performance of Test 1.3 (UQT only)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | 0.00% | 150% | 99% | -0.80% | 0.83% | 0.75% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.18% | -0.13% | -0.13% | 163% | 103% | -0.67% | -0.44% | 0.29% | 106% | 99% |  |  |  |  |  |
| Class B | -0.18% | -0.07% | -0.15% | 175% | 101% | -0.09% | 0.41% | 0.35% | 107% | 101% | -0.40% | -0.13% | -0.17% | 166% | 101% |
| Class C | -0.23% | -0.20% | -0.13% | 186% | 102% | -0.70% | 0.44% | 0.73% | 136% | 105% | -0.31% | 0.02% | -0.20% | 138% | 102% |
| Class E | -0.27% | -0.17% | -0.05% | 176% | 103% |  |  |  |  |  | 0.01% | -0.42% | -0.82% | 129% | 103% |
| Overall |  |  |  |  |  | -0.51% | 0.33% | 0.52% | 113% | 102% | -0.27% | -0.16% | -0.34% | 147% | 102% |
| Class D | -0.09% | -0.02% | -0.16% | 169% | 104% | -0.45% | -0.14% | 0.10% | 130% | 105% | -0.12% | 0.37% | -0.75% | 138% | 109% |

Table 4: Performance of Test 1.4 (UQT+ABT)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.10% | -3.39% | -3.73% | 228% | 83% | -0.88% | 0.56% | 0.32% | 103% | 110% |  |  |  |  |  |
| Class A2 |  |  |  |  |  | -0.72% | -0.62% | 0.23% | 114% | 107% |  |  |  |  |  |
| Class B | -0.64% | -2.35% | -2.52% | 206% | 102% | -0.24% | 0.14% | 0.05% | 110% | 105% | -0.55% | -0.93% | -1.19% | 116% | 93% |
| Class C | -0.12% | 0.04% | 0.07% | 149% | 74% | -0.99% | -0.04% | 0.30% | 165% | 100% | -0.22% | -0.17% | -0.42% | 112% | 108% |
| Class E | -0.72% | -1.89% | -1.80% | 192% | 69% |  |  |  |  |  | -0.93% | -2.31% | -3.00% | 112% | 103% |
| Overall |  |  |  |  |  | -0.66% | 0.03% | 0.21% | 122% | 105% | -0.54% | -1.02% | -1.39% | 114% | 100% |
| Class D | -0.01% | 0.02% | 0.07% | 150% | 87% | -0.68% | -0.90% | 0.03% | 180% | 100% | -0.13% | -1.03% | -0.11% | 111% | 94% |

In addition to the default results of Table 1, some alternative trade-offs between encoding time and coding efficiency for test 1.1 are provided in Table 5 and Table 6 for RA configuration. These two trade-offs are respectively obtained by settings the parameter MaxMTTHierarchyDepthByTid to 333333 and 444444 for classes A1, A2 and B.

Table 5: Test 1.1 trade-off 2 (Encoder only)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.46% | -0.08% | -0.62% | 130% | 108% |
| Class A2 | -1.36% | -1.31% | -0.70% | 124% | 104% |
| Class B | -0.65% | -0.28% | -0.48% | 127% | 107% |
| Class C | -0.09% | 0.87% | 1.19% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.80% | -0.14% | -0.11% | 120% | 105% |
| Class D | -0.10% | 0.24% | 0.88% | 105% | 103% |

Table 6: Test 1.1 trade-off 3 (Encoder only)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.93% | -0.64% | -1.47% | 216% | 109% |
| Class A2 | -1.83% | -1.91% | -1.42% | 187% | 108% |
| Class B | -1.04% | -0.93% | -0.99% | 187% | 104% |
| Class C | -0.55% | 0.20% | 0.53% | 143% | 105% |
| Class E |  |  |  |  |  |
| Overall | -1.25% | -0.77% | -0.77% | 179% | 106% |
| Class D | -0.33% | -0.30% | 0.39% | 134% | 102% |

In addition to the default results of Table 2, some alternative trade-offs between encoding time and coding efficiency for test 1.2 are provided in Table 7 and Table 8 for RA configuration. These two trade-offs are respectively obtained by settings the parameter MaxMTTHierarchyDepthByTid to 333332 and 333333 for classes A1, A2 and B. In addition, in trade-off 3, block size 24 is allowed, but in the two other trade-offs only the added block size 12 is allowed in addition to dyadic block sizes of ECM-2.0.

Table 7: Test 1.2 trade-off 2 (ABT)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | 0.48% | 0.00% | 102% | 103% |
| Class A2 | -0.99% | -0.80% | -0.14% | 114% | 107% |
| Class B | -0.48% | -0.15% | -0.16% | 114% | 98% |
| Class C | -0.09% | 0.87% | 1.19% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | 0.12% | 0.24% | 108% | 102% |
| Class D | -0.10% | 0.24% | 0.88% | 100% | 100% |

Table 8: Test 1.2 trade-off 3 (ABT)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.02% | -0.58% | -1.39% | 153% | 103% |
| Class A2 | -1.87% | -1.83% | -1.22% | 151% | 99% |
| Class B | -1.30% | -1.13% | -1.13% | 159% | 97% |
| Class C | -0.24% | 0.77% | 0.97% | 107% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.28% | -0.65% | -0.64% | 141% | 99% |
| Class D | -0.10% | 0.24% | 0.88% | 102% | 102% |

In addition to the default results of Table 4, some alternative trade-offs between encoding time and coding efficiency for test 1.4 are provided in Table 9 and Table 10 for RA configuration. These two trade-offs are respectively obtained by settings the parameter MaxMTTHierarchyDepthByTid to 333332 and 333333 for classes A1, A2 and B. In addition, in trade-off 3, block size 24 is allowed, but in the two other trade-offs only the added block size 12 is allowed in addition to dyadic block sizes of ECM-2.0.

Table 9: Test 1.4 trade-off 2 (UQT+ABT)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | 0.46% | 0.13% | 112% | 110% |
| Class A2 | -1.12% | -0.95% | -0.21% | 127% | 107% |
| Class B | -0.59% | -0.54% | -0.55% | 127% | 105% |
| Class C | -0.88% | -0.14% | 0.21% | 165% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.89% | -0.32% | -0.14% | 133% | 105% |
| Class D | -0.68% | -0.90% | 0.03% | 180% | 100% |

Table 10: Test 1.4 trade-off 3 (UQT+ABT)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.10% | -0.56% | -1.39% | 173% | 110% |
| Class A2 |  |  |  |  |  |
| Class B | -1.46% | -1.48% | -1.55% | 186% | 105% |
| Class C | -0.92% | -0.15% | 0.22% | 169% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.68% | -0.90% | 0.03% | 177% | 100% |

# Conclusions

This contribution presents tests of EE2-1.1 ~ EE2-1.4. The proposed methods contribute promising coding gains with acceptable encoding/decoding time. It is recommended to adopt test 1.4 in ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “EE2-1.3/EE2-1.4: Unsymmetric partitioning methods in video coding,” JVET-W0086, Jul. 2021.

- F. Le Léannec, K. Naser, T. Dumas, A. Robert, F. Galpin, E. François, “EE2-1.1 and EE2-1.2: Asymmetric Binary Tree partitioning,” JVET-W0084, Jul. 2021.

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, F. Le Léannec, K. Naser, T. Dumas, A. Robert, “EE2-1.5: A combining test of EE2-1.2 and EE2-1.4b,” JVET-W0087, Jul. 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, Jul. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).