| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0073-v2 |
| --- | --- |

| Title: | AHG2: On specifying the range extensions profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution discusses a few asserted issues in the current text for or related to specifying the range extensions profiles, and proposes the following changes:

- Change the value range for sps_bitdepth_minus8 in its semantics to be 0..8. Do the same for vps_ols_dpb_bitdepth_minus8[ i ].

- Add the following constraint: In a bitstream conforming to the Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profile, the value of ph_inter_slice_allowed_flag shall be equal to 0 for all pictures.

- The specified general_profile_idc values for all the 16-bit profiles are decreased by 1.

- Replace the current decoder capability requirements for the range extension profiles with the following:

Decoders conforming to a format range extensions profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- Any of the following conditions apply:

  - The decoder conforms to the Main 12 profile, and the bitstream is indicated to conform to the Main 10, Main 10 Still Picture, Main 12, Main 12 Intra, or Main 12 Still Picture profile.

  - The decoder conforms to the Main 12 4:4:4 profile, and the bitstream is indicated to conform to the Main 10, Main 10 Still Picture, Main 10 4:4:4, Main 10 4:4:4 Still Picture, Main 12, Main 12 Intra, Main 12 Still Picture, Main 12 4:4:4, Main 12 4:4:4 Intra, or Main 12 4:4:4 Still Picture profile.

  - The decoder conforms to the Main 16 4:4:4 profile, and the bitstream is indicated to conform to Main 10, Main 10 Still Picture, Main 10 4:4:4, Main 10 4:4:4 Still Picture, or any of the format range extensions profile.

  - The decoder conforms to the Main 12 Intra profile, and the bitstream is indicated to conform to the Main 10 Still Picture, Main 12 Intra, or Main 12 Still Picture profile.

  - The decoder conforms to the Main 12 4:4:4 Intra profile, and the bitstream is indicated to conform to the Main 10 Still Picture, Main 10 4:4:4 Still Picture, Main 12 Intra, Main 12 4:4:4 Intra, Main 12 Still Picture, or Main 12 4:4:4 Still Picture profile.

  - The decoder conforms to the Main 16 4:4:4 Intra profile, and the bitstream is indicated to conform to the Main 10 Still Picture, Main 10 4:4:4 Still Picture, Main 12 Intra, Main 12 4:4:4 Intra, Main 16 4:4:4 Intra, Main 12 Still Picture, Main 12 4:4:4 Still Picture, or Main 16 4:4:4 Still Picture profile.

  - The decoder conforms to the Main 12 Still Picture profile, and the bitstream is indicated to conform to the Main 10 Still Picture or Main 12 Still Picture profile.

  - The decoder conforms to the Main 12 4:4:4 Still Picture profile, and the bitstream is indicated to conform to the Main 10 Still Picture, Main 10 4:4:4 Still Picture, Main 12 Still Picture or Main 12 4:4:4 Still Picture profile.

  - The decoder conforms to the Main 16 4:4:4 Still Picture profile, and the bitstream is indicated to conform to the Main 10 Still Picture, Main 10 4:4:4 Still Picture, Main 12 Still Picture, Main 12 4:4:4 Still Picture, or Main 16 4:4:4 Still Picture profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

Decoders conforming to the Main 12 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

- That bitstream is indicated to conform to the Main 10, Main 12, or Main 12 Intra profile, to conform to a tier that is lower than or equal to the specified tier, and to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

- That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

Decoders conforming to the Main 12 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

- That bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Main 12, Main 12 Intra, Main 12 4:4:4, or Main 12 4:4:4 Intra profile, to conform to a tier that is lower than or equal to the specified tier, and to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

- That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

Decoders conforming to the Main 16 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

- That bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Main 12, Main 12 Intra, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 16 4:4:4, or Main 16 4:4:4 Intra profile, to conform to a tier that is lower than or equal to the specified tier, and to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

- That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

- Change Table A.1 such that each cell includes the allowed values of the corresponding syntax element for the corresponding profile.

- Rename general_lower_bit_rate_constraint_flag to sps_higher_bit_rate_allowed_flag, flip the meaning, move it to SPS extension, and merge Table A.2 into Table A.1.

The exact text changes for the proposed changes are included in an attachment to this contribution, with change marks relative to JVET-W2005-v2.

# Introduction

The current draft text for specifying the VVC range extensions profiles is as follows:

### A3.5	Format range extensions profiles

The following profiles, collectively referred to as the format range extensions profiles, are specified in this subclause:

- The Main 12, Main 12 4:4:4 and Main 16 4:4:4 profiles

- The Main 12 Intra, Main 12 4:4:4 Intra and Main 16 4:4:4 Intra profiles

- The Main 12 Still Picture, Main 12 4:4:4 Still Picture and Main 16 4:4:4 Still Picture profiles

Bitstreams conforming to the format range extensions profiles shall obey the following constraints:

- Referenced SPSs shall have ptl_multilayer_enabled_flag equal to 0.

- In bitstreams conforming to the Main 12 Still Picture, Main 12 4:4:4 Still Picture and Main 16 4:4:4 Still Picture profiles, the bitstream shall contain only one picture.

- In bitstreams conforming to the Main 12, Main 12 4:4:4, Main 16 4:4:4, Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, general_level_idc for all values of i in active SPSs shall not be equal to 255 (which indicates level 15.5).

- The tier and level constraints specified for the Main 12, Main 12 4:4:4, Main 16 4:4:4, Main 12 Intra, Main 12 4:4:4 Intra or Main 16 4:4:4 Intra profiles in subclause A.4, as applicable, shall be fulfilled.

Table A.1 – Allowed values for syntax elements in the format range extensions profiles

| Profile for which constraint is specified | sps_chroma_format_idc | sps_bitdepth_minus8 | sps_palette_enabled_flag | sps_extended_precision_flag sps_ts_residual_coding_rice_present_in_sh_flag, sps_rrc_rice_extension_flag, sps_persistent_rice_adaptation_enabled_flag sps_reverse_last_sig_coeff_enabled_flag |
| --- | --- | --- | --- | --- |
| Main 12 | 0 or 1 | 0..4 | 0 | 0 |
| Main 12 4:4:4 | – | 0..4 | – | – |
| Main 16 4:4:4 | – | – | – | – |
| Main 12 Intra | 0 or 1 | 0..4 | 0 | 0 |
| Main 12 4:4:4 Intra | – | 0..4 | – | – |
| Main 16 4:4:4 Intra | – | – | – | – |
| Main 12 Still Picture | 0 or 1 | 0..4 | 0 | 0 |
| Main 12 4:4:4 Still Picture | – | 0..4 | – | – |
| Main 16 4:4:4 Still Picture | – | – | – | – |

Conformance of a bitstream to the Main 12 profile is indicated by general_profile_idc being equal to 2.

Conformance of a bitstream to the Main 12 Intra profile is indicated by general_profile_idc being equal to 10.

Conformance of a bitstream to the Main 12 Still Picture profile is indicated by general_profile_idc being equal to 66.

Conformance of a bitstream to the Main 12 4:4:4 profile is indicated by general_profile_idc being equal to 34.

Conformance of a bitstream to the Main 12 4:4:4 Intra profile is indicated by general_profile_idc being equal to 42.

Conformance of a bitstream to the Main 12 4:4:4 Still Picture profile is indicated by general_profile_idc being equal to 98.

Conformance of a bitstream to the Main 16 4:4:4 profile is indicated by general_profile_idc being equal to 36.

Conformance of a bitstream to the Main 16 4:4:4 Intra profile is indicated by general_profile_idc being equal to 44.

Conformance of a bitstream to the Main 16 4:4:4 Still Picture profile is indicated by general_profile_idc being equal to 100.

All other combinations of the syntax elements in Table A.1 with general_profile_idc equal to 2, 10, 66, 34, 42, 98, 36, 44, or 100 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this document. However, decoders conforming to the format range extensions profiles shall allow other combinations as specified below in this subclause to occur in the bitstream.

Table A.2 – Bitstream indications for conformance to format range extensions profiles

| Profile for which the bitstream indicates conformance | general_lower_bit_rate_constraint_flag |
| --- | --- |
| Main 12 | 1 |
| Main 12 4:4:4 | 1 |
| Main 16 4:4:4 | 1 |
| Main 12 Intra | 0 or 1 |
| Main 12 4:4:4 Intra | 0 or 1 |
| Main 16 4:4:4 Intra | 0 or 1 |
| Main 12 Still Picture | 0 or 1 |
| Main 12 4:4:4 Still Picture | 0 or 1 |
| Main 16 4:4:4 Still Picture | 0 or 1 |

Decoders conforming to a format range extensions profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sub-layer representations for which all of the following conditions apply:

- Any of the following conditions apply:

- The decoder conforms to the Main 12 4:4:4 or Main 16 4:4:4 profile, and the bitstream or sub-layer representation is indicated to conform to the Main 10 profile, or the Main 10 Still Picture profile.

- The decoder conforms to the Main 12 4:4:4 Intra, Main 16 4:4:4 Intra, Main 12 Still Picture, Main 12 4:4:4 Still Picture, or Main 16 4:4:4 Still Picture profile, and the bitstream or sub-layer representation is indicated to conform to the Main 10 Still Picture profile.

- general_profile_idc is equal to 2, 10, 66, 34, 42, 98, 36, 44, or 100 for the bitstream, and the value of each constraint flag listed in Table A.1 is greater than or equal to the value(s) specified in the row of Table A.1 for the format range extensions profile for which the decoder conformance is evaluated.

- The bitstream or sub-layer representation is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream or sub-layer representation is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

The following issues have been observed:

- The currently specified value range for sps_bitdepth_minus8 in its semantics is 0..2, which does not allow bit depth to be greater than 10 bits, i.e., not allowing the new 12-bit and 16-bit profiles.

- For the Main 12 Intra, Main 12 4:4:4 Intra and Main 16 4:4:4 Intra profiles, there lacks a constraint to disallow the use of inter prediction.

- The general_profile_idc values for the range extensions profiles are not fully aligned with the way how the values for the VVC version 1 profiles, for which the rules applied as follows:

  - The value for the Main 10 profile is 1.

  - For any still picture profile, bit 6 (bits are counted starting from 0) of general_profile_idc is set, i.e., the value would be adding 64. Thus the value for the Main 10 Still Picture profile is 65.

  - For any 4:4:4 profile, bit 5 is set (i.e., adding 32).

  - For any multilayer profile, bit 4 is set (i.e., adding 16).

Currently, the rules applied for the range extensions profiles are as follows:

- For any 12-bit profile, bit 1 is set (i.e., adding 2).

- For any 16-bit profile, bit 2 is set (i.e., adding 4).

- For any intra profile, bit 3 is set (i.e., adding 8).

These rules are fine. However, following the same rules in VVC 1, then the general_profile_idc value for the Main 12 profile should be 3 (i.e., 1+2) instead of 2. Similarly for the general_profile_idc values for all other range extensions profiles; their values should all be increased by 1.

During the discussion of JVET-X0073-v1, it was clarified that one interpretation of the current values of general_profile_idc is that the bit 0 equal to 1 indicates a 10-bit profile. That means we have the following set of rules:

- For any 10-bit profile, bit 0 (i.e., the LSB) of general_profile_idc is equal to 1.

- For any 12-bit profile, bit 1 of general_profile_idc is equal to 1.

- For any 16-bit profile, bit 2 of general_profile_idc is equal to 1.

- For any intra profile, bit 3 of general_profile_idc is equal to 1.

- For any multilayer profile, bit 4 of general_profile_idc is equal to 1.

- For any 4:4:4 profile, bit 5 of general_profile_idc is equal to 1.

- For any still picture profile, bit 6 of general_profile_idc is equal to 1.

With that, the current design uses the three LSBs for individually indicating the maximum bit depth, each bit indicating one maximum bit depth value, thus the efficiency is using 3 bits with the capability of specifying 3 different maximum bit depth values.

However, it would be better to use the combination of just the 2 LSBs to indicate 4 different maximum bit depth values: 01 for 10-bit profiles, 10 for 12-bit profiles, 11 for 16-bit profiles, and the value 00 for the two LSBs can be used for another maximum bit depth, e.g., 8-bit profiles if to be specified in the future. And at the same time, bit 2 is also saved and can be used in the future for whatever other purposes, thus allowing for many more future general_profile_idc values following the same set of rules. This slightly changed set of rules is as follows:

- For any 10-bit profile, the 2 LSBs (bits 1 and 0) of general_profile_idc are equal to '01'.

- For any 12-bit profile, the 2 LSBs of general_profile_idc are equal to '10'.

- For any 16-bit profile, the 2 LSBs of general_profile_idc are equal to '11'.

- For any intra profile, bit 3 of general_profile_idc is equal to 1.

- For any multilayer profile, bit 4 of general_profile_idc is equal to 1.

- For any 4:4:4 profile, bit 5 of general_profile_idc is equal to 1.

- For any still picture profile, bit 6 of general_profile_idc is equal to 1.

To apply this slightly changed set of rules, only the general_profile_idc values for the three 16-bit profiles need to be decreased by 1, while the general_profile_idc values for all other profiles can be kept unchanged.

- The requirements of decoders conforming to some range extensions profiles to be able to decode bitstreams conforming to some VVC version 1 profiles is missing, including:

  - Decoder conforming to the Main 12 profile, bitstream conforming to the Main 10 or Main 10 Still Picture profile.

  - Decoder conforming to the Main 12 4:4:4 or Main 16 4:4:4 profile, bitstream conforming to the Main 10 4:4:4 profile or Main 10 4:4:4 Still Picture profile.

  - Decoder conforming to the Main 12 Intra profile, bitstream conforming to the Main 10 Still Picture profile.

  - Decoder conforming to the Main 12 4:4:4 Intra, Main 16 4:4:4 Intra, Main 12 4:4:4 Still Picture, or Main 16 4:4:4 Still Picture profile, bitstream conforming to the Main 10 4:4:4 Still Picture profile.

- The following paragraph is the key part of the text specifying decoder capabilties for decoding of bitstreams conforming to the range extensions profiles:

general_profile_idc is equal to 2, 10, 66, 34, 42, 98, 36, 44, or 100 for the bitstream, and the value of each constraint flag listed in Table A.1 is greater than or equal to the value(s) specified in the row of Table A.1 for the format range extensions profile for which the decoder conformance is evaluated.

It has multiple problems that make the specification incorrect and thus the decoder capabilities are not correctly specified:

  - A greater value of each syntax element in Table A.1 indicates a higher capability instead of a lower capability, thus the use of "greater than or equal to" is incorrect.

  - Table A.1 misses the following two aspects that are required for such a description to work: 1) allowing more than one picture, 2) allowing inter coding. On the other hand, existing flags for controlling these two aspects are 1) gci_one_au_only_constraint_flag (together with ptl_multilayer_enabled_flag equal to 0) and 2) gci_intra_only_constraint_flag, and them a greater value indicates a lower capability, thus including them into Table A.1 would also make it difficult to use such description for correctly specifying the decoder capabilities.

- The requirement of decoders conforming to the Main 12 Still Picture, Main 12 4:4:4 Still Picture, or Main 16 4:4:4 Still Picture profile to be able to decode the first picture of certain bitstreams, similarly as for the Main 10 Still Picture profile, is missing.

- In Table A.1, the symbol '–' is used in cells wherein there is no constraint on the allowed values, i.e., all possible values for the corresponding syntax elements are allowed. To better match with the table caption, i.e., "Allowed values for syntax elements in the format range extensions profiles", all allowed values, e.g., 0..8 for sps_bitdepth_minus8 in those cells for sps_bitdepth_minus8 and currently filled with '–', should be included instead of just '–'. Or ever better, change the table caption to be "Greatest allowed values for syntax elements in the format range extensions profiles" and just include the greatest allowed value in each cell.

During the discussion of JVET-X0073-v1, it was agreed to keep the table caption, and for each cell in the table, all allowed values of the corresponding syntax element for the corresponding profile are included.

- The caption of Table A.2 is "Bitstream indications for conformance to format range extensions profiles". However, in VVC each profile has its own general_profile_idc value, thus this copy & paste of the table caption from HEVC is confusing. It'd be better to change the caption to be something like "Allowed values for general_lower_bit_rate_constraint_flag for format range extensions profiles".

It would probably be even better to merge the only flag (general_lower_bit_rate_constraint_flag) documented in this table into Table A.1.

However, as discussed above, for all the syntax elements currently in Table A.1, a greater value of indicates a higher capability. For consistency, it would seem better to change the name of general_lower_bit_rate_constraint_flag to something like higher_bit_rate_allowed_flag (note that this was used by Tomohiro Ikai in some intermediate version of the DIS text during the editing) and then merge Table A.2 into Table A.1.

During the discussion of JVET-X0073-v1, it was suggested to provide a text for revisit, where the text does the following: rename general_lower_bit_rate_constraint_flag, flip the meaning, move it to SPS extension, and merge Table A.2 into Table A.1.

This contribution proposes changes to resolve the above issues.

# Proposed changes

The proposed changes and actions are summarized as in the abstract.

The exact text changes for the proposed changes are included in an attachment to this contribution, with change marks relative to JVET-W2005-v2.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).