| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0075 |
| --- | --- |

| Title: | Level refinement for VVC operation range extension profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Chujoh Tomoko Aono | Tel: Email: | ikai.tomohiro@sharp.co.jp chujoh.takeshi@sharp.co.jp aono.tomoko@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to refine level definition for VVC operation range profiles about HbrFactor and CpbVcl/NalFactor and MinCrScaleFactor

Introduction

The max bitrate is defined as

BrVclFactor * MaxBR for NAL bitrate

BrNalFactor * MaxBR for VCL bitrate

where MaxBr is defined in “Table 4 – Tier and level limits for the video profiles” and it’s tier and level dependent. BrVclFacor, BrNalFactor and MinCr is defines as follows.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

The variable HbrFactor (1 or 2) is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, Multilayer Main 10 4:4:4, Main 12, Main 12 4:4:4, or Main 16 4:4:4 profile, HbrFactor is set equal to 1.

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag.

In Summary:

Max VCL bitrate = MaxBR * CpbVclFactor * HbrFactor

Max NAL bitrate = MaxBR * CpbNalFactor * HbrFactor

MinCr = MinCrBase * MinCrScaleFactor ÷ HbrFactor.

Proposal

The contribution proposes the followings:

- Enabling HbrFactor equal to 2 configuration in 12 bit / 16 bit 4:4:4 profiles

  - 2 − general_lower_bit_rate_constraint_flag for Main 12 4:4:4, or Main 16 4:4:4 profile

- Refine Main 12, Main 12 Intra, Main 12 Still Picture, Main 12 4:4:4 and Main 12 4:4:4 Intra, Main 12, Main 16 4:4:4 and Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture parameter in Table 5

  - the same CpbVclFactor, CpbNalFactor, FormatCapabilityFactor as HEVC

- Refine MinCrScaleFactor

  - Use 1.0 for Main 12 profiles (12 bit 4:2:0 profiles)

  - Use 0.5 or 0.75

Two options:

Option1 (same value is used for intra profiles/inter profiles)

- Use 0.5 for all 12 bit/16 bit 4:4:4 profiles

Option2 (different value is used for intra profiles/inter profiles)

- Use 0.5 for Intra 12 bit/16 bit 4:4:4 profiles

- Use 0.75 for inter-predictive 12 bit/16 bit 4:4:4 coding profiles

It is noted that

- for 4:4:4 profiles, HEVC uses 0.5 and VVC v1 uses 0.75.

- for 4:2:0 profiles, HEVC and VVC v1 uses 1.0.

Proposed WD

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, Multilayer Main 10 4:4:4, Main 12, Main 12 4:4:4, or Main 16 4:4:4 profile, HbrFactor is set equal to 1.

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag.

WD of Option2 (same value is used for intra profiles/inter profiles)

Table 5 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 12, Main 12 Intra, Main 12 Still Picture | 1 500 | 1 650 | 2.250 | 1.00 |
| Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 3 000 | 3 300 | 4.500 | 0.5 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.5 |

WD of Option2 (different value is used for intra profiles/inter profiles)

Table 5 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 12, Main 12 Intra, Main 12 Still Picture | 1 500 | 1 650 | 2.250 | 1.00 |
| Main 12 4:4:4 | 3 000 | 3 300 | 4.500 | 0.75 |
| Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 3 000 | 3 300 | 4.500 | 0.5 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.75 |
| Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.5 |

Example case

Example of VVC current draft (JVET-W2005) and HEVC MinCR

If we calculate MinCr based on the current MinCrScaleFactor, the MinCr could be 1.5 for VVC and 1.0 in HEVC. To support MinCr equal to 1, we need to set MinCrScaleFactor to 0.5 as HEVC does.

|  |  | Table 4 | Table 5 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MinCrBase (High tier, Level 4.0 or higher) | MinCrScaleFactor | HbrFactor general_lower_bit_rate_constraint_flag | MinCr |
| VVC | 4:4:4 | 4 | 0.75 | 2 (flag = 0) | 1.5 |
| HEVC | 4:4:4 | 4 | 0.5 | 2 (flag = 0) | 1 |

Example of VVC proposed MaxBitrate (High tier)

If we calculate MaxBitrate based on the MaxBr (in Table 4) and CpbVclFactor in proposal and HbrFactor (=2 in maximum case), we get MaxBitrate as follows.

- Main 12 4:4:4 profiles: 300, 960 and 2880 Mbps for HD, 4K and 8K

- Main 16 4:4:4 profiles: 400, 1280 and 3840Mbps for HD, 4K and 8K

If we calculate bitrate based on MinCR= 20, 20 and 25 for HD, 4K and 8K, we got

- 12bit 4:4:4 profiles: 224, 896 and 2867 Mbps for HD, 4K and 8K

- 16bit 4:4:4 profiles: 299, 1194 and 3822 Mbps for HD, 4K and 8K

The number is equal to or less than MaxBitrate.

Thus, it could be said that the proposed CpbVclFactor could support the case MinCR is roughly 20 (HD, 4K) / 25 (8K) or higher.

Table 1. Exampled bitrate (Mbps) depending levels’s CpbVclFactor, HbrFactor and level’s MaxBr

|  | CpbVclFactor | HbrFactor | MaxBR (High tier) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Levels |  |  | 4 | 4.1 | 5 | 5.1 | 5.2 | 6 | 6.1 | 6.2/6/3 |
| Profiles |  |  | 30000 | 50000 | 100000 | 160000 | 240000 | 240000 | 480000 | 800000 |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1000 | 1 | 30 (Mb) | 50 | 100 | 160 | 240 | 240 | 480 | 800 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2500 | 1 | 75 | 125 | 250 | 400 | 600 | 600 | 1200 | 2000 |
| Main 12, Main 12 Intra, Main 12 Still Picture | 1500 | 1 | 45 | 75 | 150 | 240 | 360 | 360 | 720 | 1200 |
| Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 3000 | 2 | 180 | 300 | 600 | 960 | 1440 | 1440 | 2880 | 4800 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4000 | 2 | 240 | 400 | 800 | 1280 | 1920 | 1920 | 3840 | 6400 |
|  |  |  | 4 | 4.1 | 5 | 5.1 | 5.2 | 6 | 6.1 | 6.2 |
| Exampled Bitrate (Mbps) per HD, 4K, 8K @ 60fps with exampled compression ratio (CR) |  |  |  |  |  |  |  |  |  |  |
|  | bpp | CR |  |  |  |  |  |  |  |  |
| HD 444 60fps | 12 | 1 |  | 4479 |  |  |  |  |  |  |
| HD 444 60fps | 16 | 1 |  | 5972 |  |  |  |  |  |  |
| 4K 444 60fps | 12 | 1 |  |  |  | 17916 |  |  |  |  |
| 4K 444 60fps | 16 | 1 |  |  |  | 23888 |  |  |  |  |
| 8K 444 60fps | 12 | 1 |  |  |  |  |  |  | 71664 |  |
| 8K 444 60fps | 16 | 1 |  |  |  |  |  |  | 95551 |  |
| HD 444 60fps | 12 | 20 |  | 224 |  |  |  |  |  |  |
| HD 444 60fps | 16 | 20 |  | 299 |  |  |  |  |  |  |
| 4K 444 60fps | 12 | 20 |  |  |  | 896 |  |  |  |  |
| 4K 444 60fps | 16 | 20 |  |  |  | 1194 |  |  |  |  |
| 8K 444 60fps | 12 | 25 |  |  |  |  |  |  | 2867 |  |
| 8K 444 60fps | 16 | 25 |  |  |  |  |  |  | 3822 |  |

Appendix1 (VVC and HEVC specification)

    - Profile-specific level limits

The following is specified for expressing the constraints in this annex:

- Let the variable FrVal be set equal to 1  300 if general_tier_flag is equal to 0 and set equal to 1  960 otherwise.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, Multilayer Main 10 4:4:4, Main 12, Main 12 4:4:4, or Main 16 4:4:4 profile, HbrFactor is set equal to 1.

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

Table 4 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | 2 |
| 2.0 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3.0 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 800 000 | 8 | 4 |

Table 5 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.75 |

HEVC Table A.10 –Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 12 | 1 500 | 1 650 | 2.250 | 1.0 |
| Main 10 Still Picture | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.5 |
| Main 4:4:4 12 | 3 000 | 3 300 | 4.500 | 0.5 |
| Main 12 Intra | 1 500 | 1 650 | 2.250 | 1.0 |
| Main 4:4:4 Intra | 2 000 | 2 200 | 3.000 | 0.5 |
| Main 4:4:4 10 Intra | 2 500 | 2 750 | 3.750 | 0.5 |
| Main 4:4:4 12 Intra | 3 000 | 3 300 | 4.500 | 0.5 |
| Main 4:4:4 16 Intra | 4 000 | 4 400 | 6.000 | 0.5 |
| Main 4:4:4 16 Still Picture | 4 000 | 4 400 | 6.000 | 0.5 |

References

[1] MPEG, “Requirements for a Future Video Coding Standard v5,” N17074, ISO/IEC JTC1/SC29/WG11 July 2017, Torino, IT.

[2] ITU-T SG16(VCEG), “Requirements for Future Video Coding (FVC)”, VCEG-BD03, ITU-T SG16 Q.7 VCEG 56th Meeting: 17–21 July 2017, Turin, IT”.

[3] T. Ikai, T. Chujoh, T. Aono, S. Deshpande, “Suggested initial profile text for VVC operation range extension,” JVET-W0136

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix2 (HEVC text about HbrFactor and full Table A.10)

A.4.2 Profile-specific level limits for the video profiles

NOTE – The term "video profiles", as used in this clause, refers to those profiles that are not still picture profiles. The still picture profiles include the Main Still Picture, Main 10 Still Picture, Main 4:4:4 Still Picture, and Main 4:4:4 16 Still Picture profiles.

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream or sub-layer representation is indicated to conform to the Main profile or the Main 10 profile, HbrFactor is set equal to 1.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4, High Throughput 4:4:4 10, High Throughput 4:4:4 14, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, or Screen-Extended High Throughput 4:4:4 14  profile, HbrFactor is set equal to 6.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4 16 Intra profile, HbrFactor is set equal to 24 − ( 12 * general_lower_bit_rate_constraint_flag ) or 24 − ( 12 * sub_layer_lower_bit_rate_constraint_flag[ i ] ).

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag or 2 − sub_layer_lower_bit_rate_constraint_flag[ i ].

Table A.10 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Monochrome | 667 | 733 | 1.000 | 1.0 |
| Monochrome 10 | 833 | 917 | 1.250 | 1.0 |
| Monochrome 12 | 1 000 | 1 100 | 1.500 | 1.0 |
| Monochrome 16 | 1 333 | 1 467 | 2.000 | 1.0 |
| Main | 1 000 | 1 100 | 1.500 | 1.0 |
| Screen-Extended Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.0 |
| Screen-Extended Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1 500 | 1 650 | 2.250 | 1.0 |
| Main Still Picture | 1 000 | 1 100 | 1.500 | 1.0 |
| Main 10 Still Picture | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 4:2:2 10 | 1 667 | 1 833 | 2.500 | 0.5 |
| Main 4:2:2 12 | 2 000 | 2 200 | 3.000 | 0.5 |
| Main 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| High Throughput 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Screen-Extended Main 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Screen-Extended High Throughput 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.5 |
| High Throughput 4:4:4 10 | 2500 | 2750 | 3.750 | 0.5 |
| Screen-Extended Main 4:4:4 10 | 2500 | 2750 | 3.750 | 0.5 |
| Screen-Extended High Throughput 4:4:4 10 | 2500 | 2750 | 3.750 | 0.5 |
| Main 4:4:4 12 | 3 000 | 3 300 | 4.500 | 0.5 |
| High Throughput 4:4:4 14 | 3500 | 3850 | 5.250 | 0.5 |
| Screen-Extended High Throughput 4:4:4 14 | 3500 | 3850 | 5.250 | 0.5 |
| Main Intra | 1 000 | 1 100 | 1.500 | 1.0 |
| Main 10 Intra | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 12 Intra | 1 500 | 1 650 | 2.250 | 1.0 |
| Main 4:2:2 10 Intra | 1 667 | 1 833 | 2.500 | 0.5 |
| Main 4:2:2 12 Intra | 2 000 | 2 200 | 3.000 | 0.5 |
| Main 4:4:4 Intra | 2 000 | 2 200 | 3.000 | 0.5 |
| Main 4:4:4 10 Intra | 2 500 | 2 750 | 3.750 | 0.5 |
| Main 4:4:4 12 Intra | 3 000 | 3 300 | 4.500 | 0.5 |
| Main 4:4:4 16 Intra | 4 000 | 4 400 | 6.000 | 0.5 |
| Main 4:4:4 Still Picture | 2 000 | 2 200 | 3.000 | 0.5 |
| Main 4:4:4 16 Still Picture | 4 000 | 4 400 | 6.000 | 0.5 |
| High Throughput 4:4:4 16 Intra | 4 000 | 4 400 | 6.000 | 0.5 |