| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0086-v2 |
| --- | --- |

| Title: | EE2-related: Adaptive Filter Shape Selection for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 Nan Hu3, Vadim Seregin3, Marta Karczewicz3 Mohammed Golam Sarwer4, Ru-Ling Liao4, Jie Chen4, Yan Ye4, Xinwei Li4 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA 35775 Morehouse Drive, San Diego, CA 92121, USA 4525 Almanor Ave, Sunnyvale, CA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com nanh@qti.qualcomm.com m.sarwer@alibaba-inc.com |
| Source: | Bytedance Inc. Qualcomm Incorporated Alibaba Group. |  |  |

_____________________________

# Abstract

In the current ALF design, the online-trained filters only have one fixed shape. In this contribution, a CTB-level adaptive filter shape selection (AFSS) method for ALF is proposed. With AFSS, a filter shape index that is associated with luma filters is signalled for each APS. Each CTB could select the best luma filter shape from the available APSs. In EE2-4.8, BIF-Chroma, filter shape selection for CCALF and alternative classifier for ALF are combined. In this contribution, proposed AFSS is further combined with EE2-4.8 to seek an additive coding gain.

On top of ECM-2.0, simulation results (BD-rate changes, encoding time and decoding time) are reported as below:

Combining with EE2-Test4.8a:

AI: -0.18%, -2.55%, -3.31%, 105%, 101%.

RA: -0.47%, -2.12%, -2.26%, 105%, 102%.

LB: -0.71%, -3.58%, -3.76%, 107%, 102%.

Combining with EE2-Test4.8b:

AI: -0.23%, -2.53%, -3.29%, 104%, 100%.

RA: -0.50%, -2.13%, -2.20%, 105%, 102%.

LB: -0.82%, -3.54%, -3.90%, 106%, 100%.

Combining with EE2-Test4.8c:

AI: -0.16%, -2.66%, -3.47%, 105%, 102%.

RA: -0.48%, -2.02%, -2.19%, 105%, 102%.

LB: -0.78%, -3.25%, -3.48%, 106%, 101%.

Combining with EE2-Test4.8d:

AI: -0.22%, -2.64%, -3.45%, 105%, 101%.

RA: -0.52%, -2.00%, -2.18%, 105%, 101%.

LB: -0.82%, -3.60%, -3.41%, 107%, 100%.

# Introduction

In ALF of ECM 2.0, only a fixed diamond filter shape is used for the online-trained luma filters. The single shape design may not achieve the optimal compression performance. In this contribution, an adaptive filter shape selection (AFSS) method for ALF is proposed.

# Proposed methods

## AFSS

With AFSS, two candidate filter shapes: a diamond shape as shown in Figure 1(a), which is currently applied in ECM and a cross shape as shown in Figure 1(b), which is newly introduced, can be adaptively selected by the luma filters in ALF. The number of coefficients that need to be trained and signaled for a luma filter is 20 with both the filter shapes.

| (a) | (b) |
| --- | --- |

Figure 1 Filter shapes in AFSS.

In each APS, a shape index for the online-trained luma filters is signaled to decoder. Each APS contains the luma filters that are associated with the filter shape index.

For each CTB, an APS index is signaled to indicate which luma filter shape is used to filter the current CTB. When filtering a luma sample, the coefficients and clip indices are also rearranged according to the corresponding filter shape.

## Combination of Filter Shape Selection and EE2-Test4.8

EE2-4.8 contains a total package of all in-loop filtering improvements of EE2 in W-meeting: BIF-Chroma in JVET-W0098 [1], filter shape selection for CCALF in JVET-W0079 [2], and an alternative classifier in JVET-W0128 [3]. The tests of EE2-Test4.8 are listed as below:

| # | Test |
| --- | --- |
| 4.8a | Combination of BIF-Chroma, CTB level filter shape selection of CCALF and alternative 2x2 classifiers for ALF |
| 4.8b | Combination of BIF-Chroma, CTB level filter shape selection of CCALF and alternative sample-based classifier for ALF |
| 4.8c | Combination of BIF-Chroma, CCALF with larger filter size and alternative 2x2 classifier for ALF |
| 4.8d | Combination of BIF-Chroma, CCALF with larger filter size and alternative sample-based classifier for ALF |

To further improve the performance of ALF in ECM, AFSS is combined with tests of EE2-4.8.

# Simulation results

The proposed method is implemented on top of ECM-2.0. The simulation results are tested under ECM-2.0 test condition [5] and the anchor is ECM-2.0.

Table 3-1 shows the simulation results of combining filter shape selection for ALF with EE2-Test4.8a.

Table 3-2 shows the simulation results of combining filter shape selection for ALF with EE2-Test4.8b.

Table 3-3 shows the simulation results of combining filter shape selection for ALF with EE2-Test4.8c.

Table 3-4 shows the simulation results of combining filter shape selection for ALF with EE2-Test4.8d.

It is observed that for all the combination tests, the performance on both Luma and Chroma components are very attractive. Besides, all the combinations have a limited impact on encoding and decoding time.

TABLE 3-1 Performance of filter shape selection for ALF combined with EE2-Test4.8a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -1.70% | -4.25% | 111% | 102% | -0.35% | -1.00% | -2.15% | 107% | 102% |  |  |  |  |  |
| Class A2 | -0.07% | -3.41% | -4.77% | 106% | 105% | -0.39% | -2.54% | -3.85% | 107% | 102% |  |  |  |  |  |
| Class B | -0.10% | -4.14% | -3.36% | 104% | 101% | -0.60% | -3.03% | -2.28% | 105% | 103% | -1.04% | -4.23% | -3.40% | 111% | 103% |
| Class C | -0.30% | -1.44% | -1.46% | 101% | 99% | -0.44% | -1.49% | -1.12% | 101% | 100% | -0.48% | -3.01% | -2.49% | 99% | 101% |
| Class E | -0.20% | -1.39% | -3.26% | 103% | 101% |  |  |  |  |  | -0.45% | -3.27% | -6.03% | 114% | 100% |
| Overall | -0.18% | -2.55% | -3.31% | 105% | 101% | -0.47% | -2.12% | -2.26% | 105% | 102% | -0.71% | -3.58% | -3.76% | 107% | 102% |
| Class D | -0.15% | -1.54% | -1.89% | 99% | 105% | -0.44% | -1.70% | -2.00% | 97% | 102% | -0.38% | -4.08% | -3.06% | 93% | 100% |

TABLE 3-2 Performance of filter shape selection for ALF combined with EE2-Test4.8b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -1.69% | -4.24% | 109% | 100% | -0.38% | -0.99% | -2.11% | 106% | 102% |  |  |  |  |  |
| Class A2 | -0.14% | -3.40% | -4.75% | 107% | 103% | -0.45% | -2.59% | -3.84% | 108% | 102% |  |  |  |  |  |
| Class B | -0.17% | -4.11% | -3.33% | 104% | 100% | -0.64% | -2.99% | -2.13% | 105% | 102% | -1.17% | -4.31% | -3.79% | 106% | 102% |
| Class C | -0.30% | -1.43% | -1.46% | 100% | 99% | -0.45% | -1.56% | -1.12% | 101% | 100% | -0.51% | -3.04% | -2.56% | 99% | 99% |
| Class E | -0.26% | -1.35% | -3.21% | 104% | 102% |  |  |  |  |  | -0.65% | -2.93% | -5.89% | 114% | 100% |
| Overall | -0.23% | -2.53% | -3.29% | 104% | 100% | -0.50% | -2.13% | -2.20% | 105% | 102% | -0.82% | -3.54% | -3.90% | 106% | 100% |
| Class D | -0.17% | -1.52% | -1.88% | 98% | 101% | -0.31% | -1.67% | -1.91% | 97% | 101% | -0.41% | -4.23% | -3.24% | 93% | 100% |

TABLE 3-3 Performance of filter shape selection for ALF combined with EE2-Test4.8c

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -1.64% | -4.55% | 110% | 103% | -0.34% | -0.90% | -2.15% | 106% | 102% |  |  |  |  |  |
| Class A2 | -0.04% | -3.73% | -5.45% | 107% | 104% | -0.42% | -2.69% | -4.19% | 106% | 101% |  |  |  |  |  |
| Class B | -0.09% | -4.13% | -3.28% | 104% | 102% | -0.62% | -2.87% | -1.93% | 106% | 103% | -1.06% | -4.00% | -3.28% | 107% | 103% |
| Class C | -0.28% | -1.43% | -1.56% | 101% | 101% | -0.45% | -1.31% | -1.04% | 101% | 100% | -0.59% | -2.75% | -2.41% | 99% | 100% |
| Class E | -0.17% | -1.81% | -3.29% | 104% | 102% |  |  |  |  |  | -0.58% | -2.67% | -5.23% | 115% | 99% |
| Overall | -0.16% | -2.66% | -3.47% | 105% | 102% | -0.48% | -2.02% | -2.19% | 105% | 102% | -0.78% | -3.25% | -3.48% | 106% | 101% |
| Class D | -0.14% | -1.37% | -1.76% | 100% | 99% | -0.40% | -1.12% | -1.72% | 97% | 102% | -0.31% | -3.15% | -3.15% | 93% | 102% |

TABLE 3-4 Performance of filter shape selection for ALF combined with EE2-Test4.8d

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -1.64% | -4.54% | 111% | 102% | -0.41% | -0.94% | -2.20% | 107% | 101% |  |  |  |  |  |
| Class A2 | -0.11% | -3.72% | -5.44% | 108% | 102% | -0.48% | -2.70% | -4.22% | 108% | 102% |  |  |  |  |  |
| Class B | -0.16% | -4.10% | -3.25% | 105% | 100% | -0.67% | -2.73% | -1.82% | 106% | 102% | -1.14% | -4.22% | -3.09% | 107% | 102% |
| Class C | -0.29% | -1.43% | -1.55% | 101% | 101% | -0.44% | -1.35% | -1.10% | 101% | 99% | -0.55% | -3.07% | -2.39% | 100% | 100% |
| Class E | -0.23% | -1.76% | -3.25% | 104% | 100% |  |  |  |  |  | -0.65% | -3.27% | -5.30% | 116% | 99% |
| Overall | -0.22% | -2.64% | -3.45% | 105% | 101% | -0.52% | -2.00% | -2.18% | 105% | 101% | -0.82% | -3.60% | -3.41% | 107% | 100% |
| Class D | -0.16% | -1.35% | -1.74% | 99% | 104% | -0.32% | -1.11% | -1.62% | 97% | 101% | -0.47% | -2.64% | -2.62% | 94% | 100% |

# Conclusion

Total four combination tests of adaptive filter shape selection for ALF and EE2-4.8 are proposed in this contribution. All the combination tests can achieve promising coding performance with limited encoding time increase and negligible decoding time changes. It is recommended to adopt one of the combinations to the next version of ECM software.

# References

- W. Yin, K. Zhang and L. Zhang, “Non-EE2: Bilateral Inloop Filter on Chroma,” JVET-W0098, July 2021.

- N. Hu, V. Seregin and M. Karczewicz, “AHG12: Alternative classifiers for ALF,” JVET-W0128, July 2021.

- M. G. Sarwer, R. Liao J. Chen, Y. Yan and X. Li, “AHG12: CTB level filter shape selection of CCALF”, JVET-W0079, July 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2.

- M.Karczewicz, Y.Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).