| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0093-v1 |
| --- | --- |

| Title: | AHG2/AHG8: Comments on VVC operation range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The following modifications are proposed to improve the VVC operation range extension specifications:

Proposal-1: Add the constraint that vps_max_layers_minus1 shall be equal to 0 in the format range extension profiles.

Proposal-2: Update the value range of the syntax element sps_bitdepth_minus8 in VPS to cover the extended bitdepth range.

Proposal-3: Add the description that clarifies the meaning of “-“ in the Table A.1.

Proposal-4: Replace “active SPSs” with “referenced SPSs”.

# Proposal - 1

As long as the format range extension profiles do not support multi-layers, one of the following constraint options needs to be adopted:

Option - 1

Bitstreams conforming to the format range extensions profiles shall obey the following constraints: - Referenced VPSs shall have vps_max_layers_minus1 equal to 0.

or

Option - 2

Bitstreams conforming to the format range extensions profiles shall obey the following constraints: - Referenced SPSs shall have sps_video_parameter_set_id equal to 0.

Between options 1 and 2, the option 1 is preferred to enable the presence of VPS in the bitstreams that conforms to the format range extension profiles.

# Proposal - 2

It is proposed to update the value range of the syntax element sps_bitdepth_minus8 in VPS to cover the extended bitdepth range.

sps_bitdepth_minus8 specifies the bit depth of the samples of the luma and chroma arrays, BitDepth, and the value of the luma and chroma quantization parameter range offset, QpBdOffset, as follows: BitDepth = 8 + sps_bitdepth_minus8 (38) QpBdOffset = 6 * sps_bitdepth_minus8 (39) sps_bitdepth_minus8 shall be in the range of 0 to 28, inclusive, as specified in Annex A.3.

# Proposal - 3

It is proposed to add the following constraint so that the meaning of “-“ is clarified in the Table A.1.

Bitstreams conforming to the format range extensions profiles shall obey the following constraints: - The constraints specified in Table A.1 shall apply, in which entries marked with "–" indicate that the table entry does not impose a profile-specific constraint on the corresponding syntax element. NOTE – For some syntax elements with table entries marked with "–", a constraint may be imposed indirectly – e.g., by semantics constraints that are imposed elsewhere in this Specification when other specified constraints are fulfilled. Table A.1 – Allowed values for syntax elements in the format range extensions profiles Profile for which constraint is specified; sps_chroma_format_idc; sps_bitdepth_minus8; sps_palette_enabled_flag; sps_extended_precision_flag sps_ts_residual_coding_rice_present_in_sh_flag, sps_rrc_rice_extension_flag, sps_persistent_rice_adaptation_enabled_flag sps_reverse_last_sig_coeff_enabled_flag. Main 12; 0 or 1; 0..4; 0; 0. Main 12 4:4:4; –; 0..4; –; –. Main 16 4:4:4; –; –; –; –. Main 12 Intra; 0 or 1; 0..4; 0; 0. Main 12 4:4:4 Intra; –; 0..4; –; –. Main 16 4:4:4 Intra; –; –; –; –. Main 12 Still Picture; 0 or 1; 0..4; 0; 0. Main 12 4:4:4 Still Picture; –; 0..4; –; –. Main 16 4:4:4 Still Picture; –; –; –; –.

# Proposal - 4

It is proposed to replace “active SPSs” with “referenced SPSs” in the description of format range extensions profiles.

Bitstreams conforming to the format range extensions profiles shall obey the following constraints: - Referenced SPSs shall have ptl_multilayer_enabled_flag equal to 0. - In bitstreams conforming to the Main 12 Still Picture, Main 12 4:4:4 Still Picture and Main 16 4:4:4 Still Picture profiles, the bitstream shall contain only one picture. - In bitstreams conforming to the Main 12, Main 12 4:4:4, Main 16 4:4:4, Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, general_level_idc for all values of i in active referenced SPSs shall not be equal to 255 (which indicates level 15.5).

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).