| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0095-v2 |
| --- | --- |

| Title: | AHG8: General Constraints Information (GCI) flags for operation range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Five new GGI flags are proposed for new adopted tools in VVC version 2 operation range extensions:
1. general_no_extended_precision_constraint_flag

2. general_no_ts_residual_coding_rice_present_in_sh_constraint_flag

3. general_no_rrc_rice_extension_constraint_flag

4. general_no_persistent_rice_adaptation_constraint_flag

5. general_no_reverse_last_sig_coeff_constraint_flag

# Proposal

Since several coding tools or features have been adopted into VVC version 2 for format/operation range extensions, signaling new general constraints information (GCI) for those new tools should be beneficial to inform the availability of the range extension tools in bitstreams. Five new GGI flags are proposed as follows:

General constraints information syntax

| gci_no_virtual_boundaries_constraint_flag | u(1) |
| --- | --- |
| gci_num_additional_bits | u(8) |
| if( gci_num_additional_bits > 0 ) { |  |
| general_lower_bit_rate_constraint_flag | u(1) |
| general_no_extended_precision_constraint_flag | u(1) |
| general_no_ts_residual_coding_rice_present_in_sh_constraint_flag | u(1) |
| general_no_rrc_rice_extension_constraint_flag | u(1) |
| general_no_persistent_rice_adaptation_constraint_flag | u(1) |
| general_no_reverse_last_sig_coeff_constraint_flag | u(1) |
| numAdditionalBitsUsed = 16 |  |
| } else |  |
| numAdditionalBitsUsed = 0 |  |
| for( i = 0; i < gci_num_additional_bits − numAdditionalBitsUsed; i++ ) |  |
| gci_reserved_zero_bit[ i ] | u(1) |
| } |  |

general_no_extended_precision_constraint_flag equal to 1 specifies that sps_extended_precision_flag for all pictures in OlsInScope shall be equal to 0. general_no_extended_precision_constraint_flag equal to 0 does not impose such a constraint.

general_no_ts_residual_coding_rice_present_in_sh_constraint_flag equal to 1 specifies that sps_ts_residual_coding_rice_present_in_sh_flag for all pictures in OlsInScope shall be equal to 0. general_no_ts_residual_coding_rice_present_in_sh_constraint_flag equal to 0 does not impose such a constraint.

general_no_rrc_rice_extension_constraint_flag equal to 1 specifies that sps_rrc_rice_extension_flag for all pictures in OlsInScope shall be equal to 0. general_no_rrc_rice_extension_constraint_flag equal to 0 does not impose such a constraint.

general_no_persistent_rice_adaptation_constraint_flag equal to 1 specifies that sps_persistent_rice_adaptation_enabled_flag for all pictures in OlsInScope shall be equal to 0. general_no_persistent_rice_adaptation_constraint_flag equal to 0 does not impose such a constraint.

general_no_reverse_last_sig_coeff_constraint_flag equal to 1 specifies that sps_reverse_last_sig_coeff_enabled_flag for all pictures in OlsInScope shall be equal to 0. general_no_reverse_last_sig_coeff_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).