| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0097-v1 |
| --- | --- |

| Title: | AHG11: A CNN-based Super Resolution Method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Cheng Fang, Xue Zhang Dong Jiang, Jucai Lin | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based super-resolution used as upsampling filter for Y component. Before entering the network, the luma block is downsampled by RPR and then encoded.

In the proposed method, the channel attention mechanism is added into the residual block.

Compared with VTM-11.0-NNVC, the experimental results of the proposed method are as follow:

Test1: AI  8.26% BD-rate gain for Class A1&A2.

Test2: AI  9.03% BD-rate gain for Class A1&A2.

Introduction

The convolutional neural network (CNN) based video coding super-resolution method has significant coding gains compared with the traditional up-sampling methods. However, there is still room for improvement in coding performance. In addition, the attention module can be used to improve the performance of CNN, especially shallow neural networks.

Proposed method

In this contribution, several CNN models are trained for up-sampling under different combinations of QP. In the decoding process, the CNN model is selected according to the sequence-level QP. Details of the model architecture, training and inference process are described below.

Architecture

The architecture of the proposed CNN model is shown in Fig.1. The network contains 16 advanced residual blocks(ARB), several convolutional layers, a shuffle layer and a shortcut connection from conv1 to conv2.

The parameters of each convolution layer are [cin, k, k, cout], where cin, k and cout represent the number of input channels, the size of convolution kernel and the number of output channels respectively.

The input of the model is the low-resolution reconstruction YLR, which is downsampled by RPR mechanism of VTM-11.0-NNVC[1] before encoding. The output of the model is the super-resolution reconstruction YSR.

In addition, the architecture of the designed ARB is shown in Fig.2, where ReLU, GAP and Softmax are activation function, global average pooling layer and normalization function respectively. The channel attention extraction module is circled by a dotted line in Fig.2.

Architecture of the proposed CNN model

Architecture of the designed ARB

Training and Inference

The training stage use BVI_DVC 4K dataset[2]. The data is downsampled by RPRx2 and then compressed by VTM.

We designed two tests of different training strategies:

- Test1: We trained 5 models.

- For the QP 22 model, the training set consists of the low-resolution reconstruction images with QPs {17, 22, 27}.

- For the QP 27 model, the training set consists of the low-resolution reconstruction images with QPs {22, 27, 32}.

- For the QP 32 model, the training set consists of the low-resolution reconstruction images with QPs {27, 32, 37}.

- For the QP 37 model, the training set consists of the low-resolution reconstruction images with QPs {32, 37, 42}.

- For the QP 42 model, the training set consists of the low-resolution reconstruction images with QPs {37, 42, 47}.

- Test2: We trained 2 models.

- For the low QP (smaller than or equal to 32) model, the training set consists of the low-resolution reconstruction images with QPs {22, 27, 32}.

- For the high QP (lager than 32) model, the training set consists of the low-resolution reconstruction images with QPs {32, 37, 42}.

Other details of the training and inference stage are as shown in Table 1 and Table 2, respectively.

Network Information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type: | GeForce GTX 2080 Ti |
|  | Framework: | PyTorch v1.3.0 |
|  | Number of GPUs per Task: | 1 |
|  | Epoch: | Test1:~2000 Test2: ~1000 |
|  | Batch size: | 16 |
|  | Training time: | Test1:~520 h/model Test2:~250 h/model |
|  | Training data information: | BVI_DVC 4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 17, 22, 27, 32, 37, 42,47 |
| Optional | Learning rate: | 1.00E-04 |
|  | Patch size | 128128 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 loss |
|  | Preprocessing: | randomly cropped |
|  | Other information: |  |

Network Information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | GPU Type | Intel(R) Xeon(R) Gold 6242R CPU @ 3.10GHz |
|  | GPU Type | No GPU |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | Test1: 1481025 /model5=7405125 Test2:1481025 /model 2 = 2962050 |
|  | Parameter Precision (Bits) in Float | 32 (F) |
|  | Memory Parameter (MB) | Test1: 5.65 /model5=28.25 Test2:5.65 /model 2 = 11.3 |
|  | MAC (Giga) | 0.40 M/pixel |
| Optional | Total Conv. Layers | 68 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 512*512 |
|  | Padding size | 16 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based upsampling filter is tested on top of VTM-11.0-NNVC with the common test conditions[3].

For the anchor and the test, the QP sets are {27, 32, 37, 42, 47} and {22, 27, 32, 37, 42}, respectively.

Table 3 and Table 4 shows the test results of the Test1 under AI and RA configuration, respectively ; Table 5 and Table 6 shows the test results of the Test2 under AI and RA configuration, respectively.

Results of Test1 under all intra configuration

|  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC(QP 27,32,37,42,47) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -9.70% |  |  | 262.27% | 50110.04% |
|  | FoodMarket4 | -4.12% |  |  | 235.36% | 44360.78% |
|  | Campfire | -12.07% |  |  | 148.39% | 41614.18% |
| Class A2 | CatRobot | -8.57% |  |  | 171.09% | 39139.57% |
|  | DaylightRoad2 | -3.00% |  |  | 180.52% | 41166.35% |
|  | ParkRunning3 | -12.06% |  |  | 114.46% | 29581.84% |
| Overall |  | -8.25% |  |  | 185.35% | 40995.46% |

Results of Test1 under random access configuration

|  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC(QP 27,32,37,42,47) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 |  |  |  |  |  |
|  | FoodMarket4 |  |  |  |  |  |
|  | Campfire |  |  |  |  |  |
| Class A2 | CatRobot |  |  |  |  |  |
|  | DaylightRoad2 |  |  |  |  |  |
|  | ParkRunning3 |  |  |  |  |  |
| Overall |  |  |  |  |  |  |

Results of Test2 under all intra configuration

|  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC(QP 27,32,37,42,47) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -10.16% |  |  | 262% | 50057% |
|  | FoodMarket4 | -4.01% |  |  | 236% | 45397% |
|  | Campfire | -14.15% |  |  | 149% | 42188% |
| Class A2 | CatRobot | -9.02% |  |  | 173% | 39553% |
|  | DaylightRoad2 | -3.74% |  |  | 180% | 40832% |
|  | ParkRunning3 | -13.07% |  |  | 115% | 29420% |
| Overall |  | -9.03% |  |  | 186% | 41241% |

Results of Test2 under random access configuration

|  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC(QP 27,32,37,42,47) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -10.16% |  |  | 262% | 50057% |
|  | FoodMarket4 | -4.01% |  |  | 236% | 45397% |
|  | Campfire | -14.15% |  |  | 149% | 42188% |
| Class A2 | CatRobot | -9.02% |  |  | 173% | 39553% |
|  | DaylightRoad2 | -3.74% |  |  | 180% | 40832% |
|  | ParkRunning3 | -13.07% |  |  | 115% | 29420% |
| Overall |  | -9.03% |  |  | 186% | 41241% |

Conclusions

This contribution presents a CNN-based upsampling filter of RPR for VVC. We proposed two experiments of different strategies, the Test1 shows an average BDBR-Y gains of 8.26% for Class A1&A2 sequences under AI configuration , and the Test2 shows an average BDBR-Y gains of 9.03% for Class A1&A2 sequences under AI configuration.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

- https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn , Di Ma, Fan Zhang, and David R. Bull.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-W2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).