| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0098-v1 |
| --- | --- |

| Title: | EE2-3.1: Combination of CIIP and DIMD/TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the results of the combinations of CIIP and DIMD/TIMD, which are studied in EE2 Test 3.1. It is proposed to replace the planar mode in CIIP with intra mode implicitly derived by DIMD method in Test 3.1a or TIMD method in Test 3.1b. It is reported that on top of ECM-2.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is:

- Test 3.1a (CIIP + DIMD):

RA {-0.03%, -0.01%, -0.02%, 100%, 100%}, LDB {-0.14%, 0.09%, -0.17%, 100%, 100%}

- Test 3.1b (CIIP + TIMD):

RA {-0.07%, -0.11%, 0.00%, 100%, 100%}, LDB {-0.22%, -0.27%, -0.28%, 101%, 101%}

Introduction

In CIIP mode, the prediction samples are generated by weighting an inter prediction signal predicted using regular merge mode and an intra prediction signal predicted using planar mode. In ECM-2.0 [1], two decoder-side intra mode derivation methods are included, namely DIMD and TIMD. A method of combining CIIP and DIMD or TIMD was proposed in the last JVET meeting [2], and studied in EE2 Test 3.1 [3]. This contribution reports the simulation results of the Test 3.1.

Proposed method

In EE2 Test 3.1, it is proposed to use the DIMD method or TIMD method to derive an intra prediction mode and replace the planar mode to generate the intra prediction signal in CIIP mode without extra signaling. The proposed method is only applied to coding blocks with an area less than or equal to 1024. And the propagated intra prediction mode of a CIIP coding block is always set to planar mode.

Test 3.1a: Combination of CIIP and DIMD

The DIMD derivation method is used to derive the intra prediction mode in CIIP. Specifically, the intra prediction mode with the highest amplitude in the DIMD histogram is selected.

Test 3.1b: Combination of CIIP and TIMD

The TIMD derivation method is used to derive the intra prediction mode in CIIP. Specifically, the intra prediction mode with the smallest SATD values in the TIMD mode list is selected and mapped to one of the 67 regular intra prediction modes.

In addition, it is also proposed to modify the weights (wIntra, wInter) for the two tests if the derived intra prediction mode is an angular mode. For near-horizontal modes (2 <= angular mode index < 34), the current block is vertically divided as shown in Figure 1 (a); for near-vertical modes (34 <= angular mode index <= 66), the current block is horizontally divided as shown in Figure 1 (b). The (wIntra, wInter) for different sub-blocks are shown in Table 1.

Figure 1 The division method for angular modes.

Table 1. The modified weights used for angular modes.

| The sub-block index | (wIntra, wInter) |
| --- | --- |
| 0 | (6, 2) |
| 1 | (5, 3) |
| 2 | (3, 5) |
| 3 | (2, 6) |

Simulation results

The described methods are implemented on top ECM-2.0 software [4] and are tested under CTC for enhanced compression tool testing [5]. It is observed that both methods can improve the coding performance with negligible encoding time and decoding time increase.

Table 2. Simulation results of Test 3.1a (CIIP + DIMD)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.08% | -0.05% | 100% | 100% |
| Class A2 | -0.03% | -0.06% | 0.02% | 100% | 100% |
| Class B | -0.02% | 0.04% | -0.13% | 100% | 100% |
| Class C | -0.04% | 0.03% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.02% | -0.20% | 0.16% | 100% | 100% |
| Class F | -0.03% | 0.05% | -0.06% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | 0.30% | -0.14% | 100% | 99% |
| Class C | -0.25% | -0.37% | -0.04% | 100% | 100% |
| Class E | -0.01% | 0.34% | -0.40% | 101% | 100% |
| Overall | -0.14% | 0.09% | -0.17% | 100% | 100% |
| Class D | -0.10% | -0.22% | 0.00% | 101% | 100% |
| Class F | -0.20% | -0.14% | 0.12% | 100% | 99% |

Table 3. Simulation results of Test 3.1b (CIIP + TIMD)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.20% | -0.07% | 100% | 100% |
| Class A2 | -0.06% | -0.13% | 0.01% | 100% | 100% |
| Class B | -0.06% | -0.10% | 0.02% | 101% | 100% |
| Class C | -0.09% | -0.04% | 0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.11% | 0.00% | 100% | 100% |
| Class D | -0.03% | -0.17% | 0.07% | 101% | 99% |
| Class F | -0.03% | -0.03% | -0.11% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | 0.00% | -0.21% | 101% | 100% |
| Class C | -0.29% | -0.22% | 0.02% | 102% | 101% |
| Class E | -0.15% | -0.76% | -0.80% | 100% | 101% |
| Overall | -0.22% | -0.27% | -0.28% | 101% | 101% |
| Class D | -0.11% | -0.27% | 0.00% | 102% | 101% |
| Class F | -0.12% | 0.13% | 0.35% | 101% | 102% |

References

- M. Coban, et. al., “Algorithm description of Enhanced Compression Model 2 (ECM 2),” JVET-W2025, September 2021.

- X. Li, et. al., “EE2-related: A combination of CIIP and DIMD/TIMD,” JVET-W0068, July 2021.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-W2024, July 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-W2017, July 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).