| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0120-v2 |
| --- | --- |

| Title: | AHG12: On sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye Jie Chen Ru-Ling Liao 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

# Abstract

This contribution proposes a sorting-based approach to identify the signs to be predicted. In the proposed method, after decoding absolute values of the levels, the transform block (TB) is sorted based on the qIdx values and the first maxNumPredSigns signs of the sorted TB are predicted and rest of the signs are EP coded. This contribution also extended the sign prediction area of a TB from 4x4 block to maximum 32x32 block.

Following results are reported:

- AI: -0.11% (Y), -0.10%(U), -0.08%(V), 105%(EncTime), 105% (DecTime)

- RA: -0.14% (Y), -0.05 %(U), -0.05 %(V), 102%(EncTime), 107% (DecTime)

- LB: -0.16% (Y), 0.16 %(U), 0.09 %(V), 102%(EncTime), 106% (DecTime)

# Introduction

In ECM-2.0, signs of few transform coefficient levels are encoded using a system of prediction.  A sign that has been predicted is no longer equiprobable (EP) signaled in the bitstream, but is replaced by a “residue” (signaled using an associated CABAC context) which indicates if the prediction was correct or not. The existing sign prediction method has following two constraints.

- For a TB, there is a limit of number of signs to be predicted. In ECM-2.0, the maximum number of signs ( maxNumPredSigns) to be predicted is signaled to the decoder through SPS. The allowable value of maxNumPredSigns is 0 to 8, inclusive. In CTC, the value of maxNumPredSigns is set to 8.

- Only signs of the top-left 4x4 block of a TB are predicted. The rest of the signs are EP coded.

If the number of signs of the top left 4x4 block of a TB is larger than maximum limit, first maxNumPredSigns signs (in raster scan order) are predicted.

# Proposed methods

## Sorting based signs selection

In ECM-2.0, if the number of signs (in the top-left 4x4 block) is larger than the allowable limit, first maxNumPredSigns signs in raster scan order are predicted. However, the raster scan order may not optimal for all TB. Usually, signs of large transform coefficient are relatively easy to predict since the impact of sign error of large transform coefficients on the reconstructed block are relatively higher. Based on this observation, a sorting based approached is proposed. One way to sort a TB is based on the absolute values of the level. However, sorting by level does not give the optimal performance because, in ECM-2.0 (also in VVC), the levels do not accurately reflect the quantization due to use of two quantizers.

Figure 1: ECM quantization (horizontal axis: dequantized coefficients, vertical axis: levels)

For same level values, the dequantized transform coefficient may be different because of two quantizers (Q0, and Q1 shown in Figure 1 [1]). Assume following two cases,

- Case 1: level = 2, quantizer Q0, the dequantize transform coefficient = 4∆k

- Case 2: level = 2, quantizer Q1, the dequantize transform coefficient = 3∆k

In above two cases, levels are equal to 2 for both cases, however, the dequantized transform coefficient of case1 (which is 4∆k) is larger than case2 (which is 3∆k).  Based on this observation, in the proposed method sorting is performed based on the qIdx value (dequantized transform coefficient = qIdx ×∆k).

The qIdx of the level depends of the DQ state and can be computed as follows:

qIdx = (abs(level) << 1) - (state & 1);

In the proposed method after decoding the absolute value of the levels, the TB is sorted based on the qIdx values. The coefficient which has highest value of qIdx is placed at the beginning of the sorted TB. The sign of the first maxNumPredSigns signs in the sorted TB is predicted and rest of the signs are EP coded.

## Extension of sign prediction area

This contribution also extends sign prediction area of a TB to maximum 32x32 block. In the proposed method, signs of top-left MxN block are predicted. The value of M and N is computed as follows:

  - M=minwidth, 32

  - N=min⁡(height, 32)

Where, width and height are the width and height of the transform block.

# Simulation results

The proposed methods are implemented on top of ECM-2.0. The performance is evaluated based on EE2 test conditions suggested in JVET-V2017 [2]. The simulation results are shown in Table 1.

Table 1: Simulation results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.21% | 103% | 103% |
| Class A2 | -0.12% | -0.12% | -0.10% | 104% | 104% |
| Class B | -0.11% | -0.11% | -0.09% | 105% | 105% |
| Class C | -0.09% | -0.10% | -0.03% | 107% | 108% |
| Class E | -0.15% | -0.16% | 0.03% | 106% | 105% |
| Overall | -0.11% | -0.10% | -0.08% | 105% | 105% |
| Class D | -0.10% | -0.15% | -0.22% | 108% | 117% |
| Class F | -0.12% | -0.16% | -0.19% | 106% | 104% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.07% | -0.25% | 101% | 104% |
| Class A2 | -0.15% | -0.14% | 0.00% | 101% | 102% |
| Class B | -0.14% | -0.01% | -0.06% | 101% | 109% |
| Class C | -0.11% | -0.13% | 0.05% | 104% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.05% | -0.05% | 102% | 107% |
| Class D | -0.05% | -0.38% | -0.08% | 104% | 115% |
| Class F | -0.08% | -0.08% | -0.35% | 103% | 117% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | 0.08% | 0.24% | 101% | 105% |
| Class C | -0.23% | 0.08% | 0.12% | 104% | 108% |
| Class E | -0.06% | 0.41% | -0.18% | 103% | 106% |
| Overall | -0.16% | 0.16% | 0.09% | 102% | 106% |
| Class D | -0.03% | 0.07% | 0.76% | 103% | 114% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Conclusion

A sorting-based sign prediction method is proposed, where, after decoding absolute values of the levels, the TB is sorted based on the qIdx values. The first maxNumPredSigns signs of the sorted TB are predicted and rest of the signs are EP coded. On top of the sorting based approached, sign prediction area of a TB is extended to maximum of 32x32 block. As compared to ECM-2.0, the proposed methods can bring -0.11% (AI), -0.14(RA), -0.16(LB) luma coding gain.

# References

H. Schwarz, M. Coban, M. Karczewicz, T. D. Chuang, F. Bossen, A. Alshin, J. Lainema, C. R. Helmrich, “Quantization and Entropy Coding in the Versatile Video Coding (VVC) Standard”, IEEE-TCSVT, August 2020.

V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-W2024, July 2021.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).