| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: by teleconference, 6 – 15 Oct. 2021 | Document: JVET-X0125-v2 |
| --- | --- |

| Title: | [AHG11] Autoencoder-based intra prediction with auxiliary feature |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Luhang Xu Yue Yu Haoping Yu Kazushi Sato Zhenyu Dai Zhihuang Xie Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This document describes an autoencoder-based intra prediction mode that consists of a pair of jointly trained encoder network and decoder network. For a given block, the encoder network extracts feature information from the original samples and sends the feature to the video encoder, and the decoder network inside a video decoder generates a prediction block by using the decoded feature and neighboring reference samples. This autoencoder is integrated into VTM-11.0 as an additional intra prediction mode (also called “autoencoder mode” in this contribution). For the all-intra configuration, the test results showed -1.11%, -0.89 %, and -0.91 % of Y BD-rate gain, U BD-rate gain, and V BD-rate gains, respectively. With this configuration, the relative encoding and decoding runtimes of VTM-11.0 with the proposed neural network-based intra prediction mode vs. VTM-11.0 are 338% and 559%, respectively. For the random-access configuration, the test results showed, in average, -0.53 %, -0.49 %, and -0.50 % of Y BD-rate gain, U BD-rate gain, and V BD-rate gain, respectively. With this configuration, the relative encoding and decoding runtimes of VTM-11.0 with the proposed neural network-based intra prediction mode vs. VTM-11.0 are 138% and 253%, respectively.

Proposed Autoencoder

Description of Autoencoder

The autoencoder-based intra prediction mode contains nine neural networks that are trained for luma blocks with nine different sizes in 4×4, 8×4, 8×8, 16×4,16×8, 16×16, 32×4, 32×16, 32×32. Each neural network consists of a pair of encoder network and decoder network. Let encoder network trained for a luma block of size w×h denote as ga . , ϕw,h, and decoder network trained for a luma block of size w×h denote as gs . , θw,h. As illustrated in Figure 1-1, ga takes pre-processed X of original block X as input and outputs a continuous feature. Then the feature is quantized, yielding a discrete-value feature q, which is encoded in the bitstream if the autoencoder mode is selected. As illustrated in Figure 1-2, gs takes q and preprocessed version R of neighboring reference samples R as input and produces Y. Y is then post-processed, yielding the prediction Y.

Figure 1-1. Encoder network extracts feature vector q of size 2 from a 4×4 original block X

Figure 1-2. Decoder network predicts 4x4 block Y from reference samples R and feature q, k is number of reference lines and set to 4 for block size min(w, h) < 8, 8 for other block size.

Architecture of encoder network and decoder network

Both the encoder network and the decoder network in an autoencoder consist of fully connected layers and non-linear activations in the following tables.

Table 1-1. Architecture of the encoder network

| layer index | input | layer type | number of neurons | activation |
| --- | --- | --- | --- | --- |
| 0 | X | Fully connection | 512 | Leaky ReLU |
| 1 | 0 | Fully connection | 512 | Leaky ReLU |
| 2 | 1 | Fully connection | 2 | Sigmoid |

X k×(k+2h+2w)

Table 1-2. Architecture of the decoder network

| layer index | input | layer type | number of neurons | activation |
| --- | --- | --- | --- | --- |
| 0 | R , q | Fully connection | 512 | Leaky ReLU |
| 1 | 0 | Fully connection | 512 | Leaky ReLU |
| 2 | 1 | Fully connection | w×h | None |

In column “input” of Table 1-1 and Table 1-2, a number refers to the index of the network layer whose output is the input to the current layer. In Table 1-1, X is a vector of size wk×h(k+2h+2w). In Table 1-2, the input for layer index 0 is the concatenation of 
R  and q, and the number of neurons for layer index 2 is w×h which is then reshaped to a one-dimensional array Y.

Preprocessing and postprocessing

Preprocessing in the encoder network

The preprocessing includes the following tasks:

- Mean value subtraction for the current original block X.

- The zero-mean original block are divided by 2b, b being the internal bit-depth, i.e. 10 in VVC.

- Then normalized block is flattened into vector X.

Preprocessing in the decoder network

The following preprocessing tasks are performed:

- The reference samples R of the current block X are divided by 2b, b being the internal bit-depth, i.e. 10 in VVC.

- Mean value μ is subtracted for available reference samples in R, same padding strategy for unavailable reference samples in VVC.

Postprocessing in the decoder network

The postprocessing in the decoder network de-normalizes Y back to internal bit-depth. The whole process can be described in following equation:

Y=minmax2b×reshapeY+μ, 0, 2b

Joint optimization of encoder network and decoder network

The gradient descent algorithm in training requires the network parameters and feature maps in floating data type. However, in inference, floating-point features should be quantized into integer vector q. The quantized feature makes the network training non-differentiable, and therefore it is not practical to be implemented. Hence, a workaround is adopted in the design in this contribution.

Let x denote the floating-point output of the encoder network, the range of x is normalized to [0, 1] for the sigmoid activation in Table 1-1. In the forward propagation, the quantization process binarizes continuous vector x into discrete vector q , as defined below, where xi and qi are elements in x and q.

qi=1,   xi≥0.50,   xi<0.5

In the backward propagation, the gradient of quantization process is set to 1.

Integration into VTM

Signaling of the autoencoder-based prediction mode

Each feature is represented by two bins (i.e., size of q is 2, and each element qi in feature q can be 0 or 1), and they are directly coded in the coding unit syntax when a CU-level flag for the proposed autoencoder mode, intra_ae_flag, is true. The following table is an example to show how the feature information is coded in the bitstream.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else{ |  |
| if (sps_autoencoder_enabled_flag && cbWidth <= 32 && cbHeight <= 32 && !(cbWidth == 32 && cbHeight == 8) && !(cbWidth == 8 && cbHeight == 32)){ |  |
| intra_ae_flag | ae(v) |
| if( intra_ae_flag ){ |  |
| feature[0] | u(1) |
| feature[1] | u(1) |
| } |  |
| else { |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag | ae(v) |
| if( intra_mip_flag ) { |  |
| intra_mip_transposed_flag[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx | ae(v) |
| if( sps_isp_enabled_flag && intra_luma_ref_idx = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) && !cu_act_enabled_flag ) |  |
| intra_subpartitions_mode_flag | ae(v) |
| if( intra_subpartitions_mode_flag = = 1 ) |  |
| intra_subpartitions_split_flag | ae(v) |
| if( intra_luma_ref_idx = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … … |  |
| } |  |
| } |  |

Currently, the autoencoder-based prediction mode is applied only to luma blocks in intra slice. There is no change to the chroma intra prediction part, except, when a luma block uses the autoencoder mode, the corresponding intra DM for chroma prediction is set to the planar mode.

Decoding process in VTM

At the parsing stage, two new steps are proposed to be carried out by a decoder. First, for each luma CU with an allowed size for the autoencoder mode (e.g., 64x64, 32x8, 8x32 are disallowed), a flag is parsed that signals whether the prediction mode for the current CU is the autoencoder mode. Second, if the current mode is indeed the autoencoder mode, the two feature bins are then parsed.

At the prediction stage, if the mode for the current CU is the autoencoder mode, three steps are performed in the order as follows.

Step-1.	preprocessing the neighboring reference samples (see 1.3.2) to obtain R .

Step-2.	feeding R  and feature q into the decoder network.

Step-3.	postprocessing the output of decoder network.

At the reconstruction stage, if LFNST is to be applied to the current luma transform unit and the intra prediction mode for the current CU is the autoencoder mode, the LFNST kernel for the planar and DC mode will be used.

Selection of Prediction Mode in VTM

In the encoder, all the possible combinations of feature q and their corresponding autoencoder modes are evaluated and compared, using rate distortion (RD) full search, with other conventional intra prediction modes (DC, planar, angular or MIP modes). The mode with the least RD cost is selected for the current CU.

Training and inference information

Table 3-1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel(R) Xeon(R) Gold 6142 CPU @ 2.60GHz GPU:Tesla V100 (32GB) |
|  | SW environment: | OS: Ubuntu 16.04.4 |
|  | Framework: | Pytorch v1.6.0 |
|  | Epoch: | 300 |
|  | Batch size: | 20Kx100 |
|  | Training time: |  |
|  | Training data information: | Training images that converted into YCbCr in DIV2K First frame of each sequence in BVI-DVC |
|  | Configurations for generating compressed training data (if different to VTM CTC): | VTM-13.0, all intra configuration and with qp {22, 27, 32, 37, 42} |
|  | Patch size | ga . , ϕw,h and gs . , θw,h are trained on pairs of a w×h block and its reference samples. |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | Adam |
|  | Loss function: | L1 |
|  | Preprocessing: | See 1.3.1 and 1.3.2 |
|  | Other information: |  |

Table 3-2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | SW environment: | CPU: Intel(R) Xeon(R) Gold 6148 CPU @ 2.40GHz GPU: None |
|  | Framework: | OS: Ubuntu 16.04.4 |
|  |  | Libtorch v1.9.0 |
|  | Total Conv. Layers | None |
|  | Total FC Layers | 3 for encoder network, 3 for decoder network. |
|  | Total Parameter Number | Decoder network { θ4,4: 313360, θ8,4: 337952, θ8,8: 460864, θ16,4: 387136, θ16,8: 559232, θ16,16: 690432, θ32,4: 485514, θ32,16: 952832, θ32,32: 1346560 } Encoder network (encoder network does not perform at VTM decoder side) { ϕ4,4: 272386, ϕ8,4: 280578, ϕ8,8: 296962, ϕ16,4: 296962, ϕ16,8: 329730, ϕ16,16: 395266, ϕ32,4: 329730, ϕ32,16: 526388, ϕ32,32: 788482 } |
|  | Parameter Precision | float32 |
|  | Memory Parameter (MB) | 34.5MB |
|  | Memory Temp (MB) |  |
|  | Kmac /pxl | Decoder network { θ4,4: 19.38Kmac, θ8,4: 10.63Kmac, θ8,8: 7.19Kmac, θ16,4: 6.09Kmac, θ16,8: 4.38Kmac, θ16,16: 2.70Kmac, θ32,4: 3.83Kmac, θ32,16: 1.86Kmac, θ32,32: 1.32Kmac } Encoder network (encoder network does not perform at VTM decoder side) { ϕ4,4: 16.88Kmac, ϕ8,4: 8.75Kmac, ϕ8,8: 4.69Kmac, ϕ16,4: 4.69Kmac, ϕ16,8: 2.58Kamc, ϕ16,16: 1.56Kmac, ϕ32,4: 2.58Kmac, ϕ32,16: 1.04Kmac, ϕ32,32: 0.77Kmac } |
| Optional | Preprocessing: | See 1.3.2 |
|  | Other information: |  |

Experimental results

Table 4-1. Simulated results for All Intra configuration and qp {22, 27, 32, 37, 42}

Table 4-2. Simulated results for Random Access configuration and qp {22, 27, 32, 37, 42}

Table 4-3. Simulated results for All Intra configuration and qp {22, 27, 32, 37}

Table 4-4. Simulated results for Random Access configuration and qp {22, 27, 32, 37}

Table 4-5. Simulated results for All Intra configuration and qp {27, 32, 37, 42}

Table 4-6. Simulated results for Random Access configuration and qp {27, 32, 37, 42}

Conclusions

This contribution presents an autoencoder-based intra prediction mode. The proposed intra prediction mode reportedly shows coding gains over VTM-11.0. It is proposed to further study in the Ad-Hoc group.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).