| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0148 (v1) |
| --- | --- |

| Title: | AHG12: On the PDPC handling in DIMD and TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Wei Chen, Che-Wei Kuo, Ning Yan, Xianglin Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, chenwei06, cheweikuo, yanning03, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In ECM-2.0, two decoder-side intra coding techniques, namely, decoder-side intra mode derivation (DIMD) and template-based intra mode derivation (TIMD), are applied to reduce the overhead of intra mode signaling. In either DIMD or TIMD, prediction samples generated from multiple intra modes could be combined together to from the final prediction samples of one intra CU where position dependent intra prediction combination (PDPC) may be applied.

It is reported that in the current design, the handling of the PDPC in the DIMD and the TIMD is not unified. Specifically, in the TIMD, the PDPC can be performed either before or after the fusion process according to the derived intra mode. On the other hand, in the DIMD, the PDPC is always performed before the fusion. It is asserted that such non-unified design not only complicates software/hardware design logics but also causes latency issue in the decoding pipeline of intra blocks.

In this proposal, one unification scheme is proposed to resolve the latency issue by performing all the PDPC operations before the fusion process of the DIMD and the TIMD. The proposed method was implemented on top of ECM-2.0, and it reports the average Y BD-rate changes of -0.02% and 0.00% for AI and RA configurations, respectively, without encoding/decoding impacts.

# Introduction

In ECM-2.0 [1], two decoder-side coding techniques, namely, decoder-side intra mode derivation (DIMD) and template-based intra mode derivation (TIMD), are applied to improve the efficiency of intra coding. Additionally, when either the DIMD or the TIMD is applied to one intra coding block (CU), multiple position dependent intra prediction combination (PDPC) processes could be invoked before the generation of the final prediction samples of the CU. It is reported that in ECM-2.0 there are different ways to handle the PDPC by the DIMD and the TIMD, as described as follows:

- The handling of the PDPC in the TIMD: when the TIMD is applied, depending on the derived intra mode, the PDPC can be conducted before or after the fusion process., As depicted in Figure 1, if the first intra mode is angular mode, the PDPC process is applied before the fusion process, if the first intra mode is DC or Planar mode, the PDPC is applied after the fusion process. Due to the non-trivial computational complexity of the PDPC process, sequentially performing the PDPC on multiple locations within the TIMD could cause latency issue in the pipeline design. This is because the TIMD predictor cannot be fully reconstructed until the PDPC for DC/planar is finished.

Figure 1 Illustration of the TIMD modes with fusion.

- The handling of the PDPC in the DIMD: As shown in Figure 2, when the DIMD is applied, though the PDPC is always conducted before the fusion, its application is determined based on the derived intra mode. Specifically, different from the existing PDPC enabling/disabling condition, the PDPC is always skipped when the derived intra mode is Planar.

Figure 2 Illustration of the DIMD modes with fusion.

# Proposal

To resolve the latency issue and achieve one more unified design, it is proposed in this contribution to apply all PDPC processes before the fusion process of the DIMD and the TIMD, as depicted in Figure 3 and Figure 4. As a result with the proposed scheme, the PDPC of each derived intra mode can be performed independently in parallel before the fusion process. Additionally, as shown Figure 4, the PDPC is also enabled for the third intra mode (i.e., Planar mode) of the DIMD to achieve a more unified design.

Figure 3 Proposed illustration of the TIMD modes with fusion.

Figure 4 Proposed illustration of the DIMD modes with fusion.

# Simulation Results

The proposed method was implemented based on the ECM-2.0 reference software. The performance was evaluated using the common test conditions as defined in [2].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.13% | -0.10% | 100% | 99% |
| Class A2 | -0.03% | -0.10% | -0.04% | 100% | 99% |
| Class B | -0.02% | -0.02% | -0.12% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.00% | 100% | 101% |
| Class E | -0.02% | -0.06% | 0.09% | 100% | 100% |
| Overall | -0.02% | 0.00% | -0.04% | 100% | 100% |
| Class D | 0.01% | 0.05% | -0.05% | 101% | 101% |
| Class F | -0.08% | -0.08% | -0.18% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.09% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.01% | 0.01% | 0.06% | 101% | 101% |
| Class C | -0.01% | -0.01% | -0.17% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | -0.42% | 0.08% | 101% | 101% |
| Class F | 0.01% | -0.11% | 0.06% | 101% | 101% |

# Conclusions

In this proposal, a method is proposed to apply all PDPC process before fusion process of the DIMD and the TIMD to remove the latency due to the sequential PDPC operations in the ECM-2.0. The results show that the proposed changes result in negligible BD-rate and encoding/decoding changes. It is recommended to adopt the proposed changes into ECM.

# References

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/tags/ECM-2.0.

- M. Karczewicz and Y. Ye, “ Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, 22th meeting, by teleconference, 20–28 Apr. 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).