| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0152-v1 |
| --- | --- |

| Title: | AHG12: CCSAO classification with edge information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C.-W. Kuo, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, W. Chen, Ning Yan, Xianglin Wang | Email: | {cheweikuo, xiaoyuxiu, yiwenchen, jhuhong-jheng, chenwei06, yanning03, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, one classifier considering both edge and band information is proposed to improve the CCSAO in ECM-2.0, which only uses band information from three component samples to classify the current component sample. Specifically, for a given luma/chroma sample, luma EO and BO are combined to classify the given sample into different categories. The original three component band classifier and the proposed classifier are switched at CTB level.

Compared to ECM-2.0 anchors, the BD-rate impact is summarized as below.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Combined EO/BO classifier | RA |  |  |  |  |  |
|  | LB |  |  |  |  |  |

Proposal

Cross-component Sample Adaptive Offset (CCSAO) is proposed in [1] and is adopted into ECM-2.0 [2]. In ECM-2.0, the CCSAO classifier utilizes only band information of all three components to classify the current sample. In this contribution, one new classifier considering both edge and band information is proposed. Specifically, for a given luma/chroma sample, luma EO and BO are combined to classify the given sample into different categories. For the EO, similar to SAO EO, one of four directions (horizontal, vertical, 135° diagonal and 45° diagonal) is firstly selected, and the luma sample is classified using the relative comparison with the two neighbouring samples. Secondly, different form the SAO EO, one threshold T is signaled to further divide the difference d of the luma sample and its neighbour into four classes, and the edge indices of the two neighbours iEa, iEb are then combined with the luma band index iB , which can be formulated as:

iE=0, if d<-TiE=1, if-T≤d<0iE=2, if 0≤d<TiE=3, o.w.                                 (1)

iB=Ycol∙NY≫BDi=iB∙16+iEa∙4+iEbC'rec=Clip1Crec+σCCSAOi                            (2)

Similar to the original CCSAO BO classifier, the parameters for the proposed classifier (i.e., the position of Ycol, NY, T and offsets) are signaled in frame level, and the original three component band classifier and the proposed classifier are switched at CTB level. The threshold T only contains power-of-2 values to ease hardware comparison complexity.

Figure 1 Illustration of the alternative CCSAO classifier.

Simulation results

The proposed methods were implemented on top of the reference software ECM-2.0 [2] and tested using the common test conditions as defined in [3].

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class A2 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class C |  | 0.00% |  | 0.14% |  | 0.18% |  | 105% |  | 104% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class D |  | -0.01% |  | -0.06% |  | -0.21% |  | 102% |  | 103% |  |
| Class F |  | -0.13% |  | -0.25% |  | -0.10% |  | 111% |  | 104% |  |
| Class TGM |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  |  | Over ECM-1.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class C |  | -0.11% |  | 0.11% |  | -0.08% |  | 106% |  | 104% |  |
| Class E |  | -0.28% |  | -0.83% |  | -0.14% |  | 116% |  | 106% |  |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class D |  | 0.05% |  | -0.15% |  | -0.43% |  | 102% |  | 104% |  |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class TGM |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |

References

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “AHG12: Cross-component Sample Adaptive Offset”, JVET-V0153, May 2021, Teleconference.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July, 2021.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).