| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0154-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-X0103 ([CE] Evaluation of VVC DCT-2 transform for Film Grain Synthesis (test CE1.1)) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangjun Pu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | +1 408 330 3281 fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for contribution JVET-X0103 on Evaluation of VVC DCT-2 transform for Film Grain Synthesis (test CE1.1) [1]. The MD5 of bitstreams used in CE1.1 test were confirmed. JVET SDR-CTC [2] Class A (4K) content was used in complexity tests. Both anchor and CE1.1 decoder were applied with SIMD ON and SIMD OFF options. From the results, film grain synthesis time difference between anchor and CE1.1 test is shown to be negligible. Six 4K/2K test sequences selected from JVET site were used in subjective tests. Both anchor and CE1.1 decoder were applied. Decoded YUV with and without FGS were viewed on a BenQ pro-level SW271C 27” 4K monitor. Similar visual quality was observed for CE1.1 compared with the anchor. The cross-checking results match with the results reported in the contribution.

# Introduction

The software for cross-checking test is downloaded from the Git repo:

https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS

The following configurations is set for cross-checking:

CE anchor:		FILM_GRAIN_DCT = 0 and FILM_GRAIN_BLK_SIZE = 0

CE1.1 test:		FILM_GRAIN_DCT = 1 and FILM_GRAIN_BLK_SIZE = 0

Both cases are tested with SIMD option enabled and disabled (CE_FGS_SIMD_OPT set to 0 and to 1).

There are two tests performed: complexity test and subjective test. Complexity test aims to analyze implementation complexity of the proposed VVC DCT-2 in the context of the film grain when compared to RDD 5 transform. Subjective test provides quality assessment of the synthesis process by using VVC DCT-2 compared to RDD 5 transform.

JVET SDR-CTC Class A content and three film grain configurations (see Table 1) are used in complexity tests. Six 4K/2K test sequences selected from JVET site are used in subjective tests (see Table 2). Encoder configuration is random access, and QP is 22,27,32,37. For subjective test, only QP 32 and 37 are used.

Table 1 Film grain configuration settings for complexity test

| Film grain configuration | # of intensity intervals | Components |
| --- | --- | --- |
| FGC_SEI_I10C3 | 10 | Luma & Chroma |
| FGC_SEI_I10C1 | 10 | Luma only |
| FGC_SEI_I1C1 | 1 | Luma only |

Table 2 Test sequences (4K and 1080p) used in subjective test

| SeqName | Category | Resolution | FPS | Frames | BitDepth | MD5 of Source YUV |
| --- | --- | --- | --- | --- | --- | --- |
| MountainBay2 | Clean | 3840x2160 | 30 | 300 | 10 | f27b6b70244fb083baac546958fcf696 |
| OberbaumSpree | Clean | 3840x2160 | 60 | 600 | 10 | 6975b81c9e63c92b3bf4223796102da1 |
| Tango2 | Clean | 3840x2160 | 60 | 294 | 10 | 0471a59c423b7059c5c6c8b395e864a9 |
| RaceNight | Noisy | 3840x2160 | 50 | 600 | 10 | c06ed896df6e7dd90b4164df39652d50 |
| TrafficFlow | Noisy | 3840x2160 | 30 | 300 | 10 | 63135fc6fd7c73675f4ec1211b11dc93 |
| Crowdrun | Noisy | 1920x1080 | 50 | 500 | 8 | da34812b5b2c316d40481c7b6c841e41 |

# Experimental Results

For complexity test, the runtime of film grain synthesis process of CE1.1 and the relative time over anchor are reported in Table 3. From the results, film grain synthesis time difference between anchor and CE1.1 test is shown to be negligible.

Table 3 complexity test results of cross-checking for CE1.1 test

|  | SIMD_OFF |  |  | SIMD_ON |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Configure | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) |
| FGC_SEI_I10C3 | 9.52 | 9.48 | -0.39% | 3.24 | 3.26 | 0.59% |
| FGC_SEI_I10C1 | 6.12 | 6.09 | -0.49% | 2.04 | 2.07 | 1.41% |
| FGC_SEI_I1C1 | 6.52 | 6.52 | 0.01% | 2.15 | 2.18 | 1.28% |
| Average | 7.24 | 7.22 | -0.29% | 2.43 | 2.45 | 1.09% |

For subjective test, both decoded YUV with and without FGS were viewed on a BenQ pro-level SW271C 27” 4K monitor. Similar visual quality was observed for CE1.1 compared with the anchor.

# Reference

- M. Radosavljevic, E. François, “[CE] Evaluation of VVC DCT-2 transform for Film Grain Synthesis (test CE1.1)”, JVET-X0103, Teleconference, 6–15 October 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Teleconference, 7–16 Oct. 2020.