| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0165-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-X0122 (Non-EE2: Unification of negative modes processing in TIMD) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kenji Kondo Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-70-7598-7407 Kenji.Kondo@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results for JVET-X0122 (Non-EE2: Unification of negative modes processing in TIMD). The crosscheck results match the coding performance reported in the contribution JVET-X0122.

Experiment

We tested a software that is provided by a proponent of [1]. For the crosscheck simulations, we used the ECM-2.0 software as an anchor and followed a common test condition described in [2].

Simulation results

The summary of test results is shown in the below table.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.19% | 99% | 101% |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.10% | -0.09% | 102% | 102% |
| Class C | -0.10% | -0.07% | -0.15% | 100% | 103% |
| Class E | -0.18% | -0.28% | -0.23% | 101% | 102% |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.03% | -0.09% | 99% | 101% |
| Class F | -0.08% | -0.26% | -0.20% | 100% | 103% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.21% | 0.08% | 100% | 100% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

(Currently, the simulations are still running. The results will be updated.)

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Alexey Filippov, Vasily Rufitskiy, Kalyan Goswami, Damian Ruiz Coll, Young-Yoon Lee, Tae Meon Bae “Non-EE2: Unification of negative modes processing in TIMD”, ISO/IEC JTC1/SC29/WG11 JVET-X0122, October 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-W2017, July 2021.