| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0173 |
| --- | --- |

| Title: | Cross-check of JVET-X0136: AHG8: On significance, GT1, and GT2 flag coding for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu | Email: | xiezhihuang@oppo.com yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results for JVET-X0136 which shifts the significance, GT1 and GT2 tests left by a number of bits signalled from the encoder. Tests were completed using the high bit depth / high bit rate CTC in both the low and standard QP ranges and also for lossless coding. The simulation results match those supplied by the proponents.

# Introduction

JVET-V0122 [1] proposed a bypass mode for coefficient coding which removed the context coded bin allowance for coefficient coding. For SVT-16 content at low QPs, and both HLG and SVT content in the lossless CTC, gains were reported. Based upon this observation, JVET-W0093 [2] provided an analysis of the usage of significance, GT1 and GT2 flags at low QPs and introduced a technique for regular residual coding which introduced a left shift of these flags to reduce the fractions that are set and thus increase the number of coefficients coded using significance, GT1 and GT2. JVET-X0136 reports further analysis and updates to the technique reported in JVET-W0093.

# Procedure

One code base was provided by the proponent. The code base consists of VTM-14.0 and the proponent’s modification. The simulations may run at different cluster, therefore, the timing reported here may not be accurate.

# Simulation results

All results match the proponents’s supplied results. The detailed results could be seen in the attached excel files.

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | -0.01% | 0.02% | 0.01% | 100% | 96% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | -0.01% | 0.02% | 0.02% | 102% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.01% | 0.02% | 0.02% | 101% | 96% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.01% | 96% | 91% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.03% | 0.03% | 0.05% | 97% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.01% | 0.01% | 0.03% | 97% | 91% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.01% | 0.02% | 0.01% | 0.01% | 0.03% | 92% | 87% |
| PQ422 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 93% | 91% |
| Overall | 0.02% | 0.02% | 0.03% | 0.02% | 0.02% | 0.03% | 92% | 89% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.07% | -0.07% | 103% | 93% |
| HLG422 | -0.08% | -0.10% | -0.10% | 104% | 95% |
| Overall | -0.07% | -0.08% | -0.08% | 104% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 100% | 107% |
| HLG422 | 0.02% | 0.03% | 0.01% | 100% | 104% |
| Overall | 0.02% | 0.02% | 0.02% | 100% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.03% | 0.03% | 95% | 92% |
| HLG422 | 0.02% | 0.04% | 0.04% | 98% | 94% |
| Overall | 0.02% | 0.03% | 0.04% | 96% | 93% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.75% | -0.77% | -0.78% | 96% | 85% |
| SVT12 | -0.20% | -0.19% | -0.21% | 95% | 83% |
| Overall | -0.47% | -0.48% | -0.50% | 95% | 84% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.76% | -0.73% | -0.73% | 102% | 101% |
| SVT12 | -0.05% | -0.05% | -0.05% | 101% | 99% |
| Overall | -0.41% | -0.39% | -0.39% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.71% | -0.67% | -0.67% | 98% | 94% |
| SVT12 | -0.04% | -0.02% | -0.02% | 95% | 93% |
| Overall | -0.38% | -0.34% | -0.34% | 97% | 94% |

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  |
| PQ444 | 2.6 | 2.6 | -0.17% | 3.1 | 3.1 | -0.21% | 3.1 | 3.1 | -0.20% |
| PQ422 | 2.4 | 2.4 | -0.23% | 2.9 | 2.9 | -0.26% | 2.9 | 2.9 | -0.28% |
| Overall | 2.5 | 2.5 | -0.20% | 3.0 | 3.0 | -0.24% | 3.0 | 3.0 | -0.24% |
| Enc Time[%] | 103% |  |  | 100% |  |  | 98% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  | 104% |  |  |

| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  |
| HLG444 | 1.8 | 1.8 | -0.80% | 2.0 | 2.0 | -0.86% | 2.0 | 2.0 | -0.86% |
| HLG422 | 1.7 | 1.7 | -0.85% | 1.9 | 2.0 | -0.99% | 1.9 | 2.0 | -0.98% |
| Overall | 1.7 | 1.8 | -0.83% | 2.0 | 2.0 | -0.92% | 2.0 | 2.0 | -0.92% |
| Enc Time[%] | 105% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 88% |  |  | 97% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  | VTM-14.0 | JVET-X0136 |  |
| SVT16 | 1.2 | 1.2 | -1.17% | 1.2 | 1.2 | -1.35% | 1.2 | 1.2 | -1.37% |
| SVT12 | 1.3 | 1.3 | -1.18% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.53% |
| Overall | 1.2 | 1.3 | -1.17% | 1.3 | 1.3 | -1.43% | 1.3 | 1.3 | -1.45% |
| Enc Time[%] | 105% |  |  | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 92% |  |  | 96% |  |  |

Normal QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.05% | 0.04% | 0.04% | 0.04% | 0.05% | 0.04% | 0.04% | 0.05% | 102% | 103% |
| Class H2 |  |  |  |  |  | 0.05% | 0.06% | 0.06% | 101% | 101% |
| Overall | 0.05% | 0.04% | 0.04% | 0.04% | 0.05% | 0.05% | 0.05% | 0.05% | 102% | 102% |

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 98% | 91% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 98% | 91% |
| Overall | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 98% | 91% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-X0136.

# References

[1] “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, JVET-V0122, F, Wang, Z. Xie, Y. Yu, H. Yu, D. Wang

[2] “AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, JVET-W-0093, A. Browne, S. Keating, K. Sharman