| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0187 |
| --- | --- |

| Title: | Inference rule on general_lower_bit_rate_constraint_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to add the following inference rule to resolve the issue that the value in some profiles shall be equal to 1 but signaling is necessary.

general_lower_bit_rate_constraint_flag equal to 1 specifies that a lower maximum bit rate constraint may be imposed in a profile, tier and level specification in Annex A than would be imposed when general_lower_bit_rate_constraint_flag is equal to 0. general_lower_bit_rate_constraint_flag equal to 0 specifies that such a corresponding lower maximum bit rate constraint is not imposed in Annex A. When not present, the value of general_lower_bit_rate_constraint_flag is inferred to be equal to 1.

Appendix

Table A.2 – Bitstream indications for conformance to format range extensions profiles

| Profile for which the bitstream indicates conformance | general_lower_bit_rate_constraint_flag |
| --- | --- |
| Main 12 | 1 |
| Main 12 4:4:4 | 1 |
| Main 16 4:4:4 | 1 |
| Main 12 Intra | 0 or 1 |
| Main 12 4:4:4 Intra | 0 or 1 |
| Main 16 4:4:4 Intra | 0 or 1 |
| Main 12 Still Picture | 0 or 1 |
| Main 12 4:4:4 Still Picture | 0 or 1 |
| Main 16 4:4:4 Still Picture | 0 or 1 |

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).