| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0005-v2 |
| --- | --- |

| Title: | JVET AHG report: Conformance testing (AHG5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Iole Moccagatta Frank Bossen Kei Kawamura T. Hashimoto Hong-Jheng Jhu Karsten Suehring Yue Yu | Email: | dmytror@qti.qualcomm.com iole.moccagatta@intel.com fbossen@sharpsec.com ki-kawamura@kddi.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com karsten.suehring@hhi.fraunhofer.de yue.yu@oppo.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG5: “Conformance testing” between the 24th Meeting (teleconference, 6–15 October 2021) and the 25th Meeting (teleconference, 12–21 January 2022).

# Mandates

At the 24th JVET meeting, the AHG on Conformance testing was established with the following mandates:

- Study the JVET-X2026 draft conformance testing for operation rage extensions and investigate the need for improvements.

- Study the JVET-X2008 draft conformance testing specification and investigate the need for extensions.

- Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability.

- Maintain and update the conformance bitstream database.

- Study additional testing methodologies to fulfil the needs for VVC conformance testing.

# Timeline

The progress on the Conformance testing specification is consistent with the preliminary timeline agreed at the 24th JVET meeting, as follows:

- 24th meeting Oct 2021: CDAM v2 conformance

- 25th meeting Jan 2022:

  - FDIS v1 conformance (plus ITU consent)

  - no action pending CDAM v2 ballot in ISO/IEC

- 27th meeting July 2022: FDAM v2

- CDAM v2 ITU consent in October

# Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

- conformance bitstreams for VVC operation range extensions:

  - 51 bitstream categories have been identified

  - Volunteers have been identified to generate bitstreams in all categories

  - Volunteers have been identified to cross-check bitstreams in 47 categories

  - 73 bitstream descriptions have been provided

  - 102 bitstreams of 47 identified categories have been cross-checked and uploaded

  - 20 bitstreams of 4 identified categories have been generated and are waiting to be cross-checked

# Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

- Three bitstreams (CROP_A_Panasonic, TEMPSCAL_B_Panasonic, and TEMPSCAL_C_Panasonic) have been re-generated with the sequence BQMall because of potential licensing issue of the sequence RitualDance.

There are not currently any known issues with the other provided VVC conformance bitstream packages. A VTM15 directory of bitstreams was made available. All provided bitstreams can be decoded using VTM15.

VVC  operation range extensions activities:

- Volunteers to generate and cross-check the bitstreams have been solicited and identified.

- Volunteers and cross-checkers have exchanged bitstreams using their own means, and bitstreams have been uploaded when cross-check has been confirmed.

- All cross-checked bitstreams can be decoded using VTM14.

- Most of the cross-checked bitstreams cannot be decoded using VTM15 because of this error

ERROR: In function "parseConstraintInfo" in VVCSoftware_VTM/source/Lib/DecoderLib/VLCReader.cpp:4638: gci_reserved_zero_bit not equal to zero

This problem seems related to the bug reported in JVET-Y0190:

JVET-Y0190 abstract: It is asserted that the current text for general constraints information (GCI) in VVC version 2 contains two bugs. For the first bug, it is asserted that incorrect semantics and handling of gci_num_additional_bits lead to GCI decoding behaviour in VVC version 2 incompatible with VVC version 1 decoders. For the second bug, it is asserted that setting gci_present_flag to 1 and gci_num_additional_bits to 0 leads to undefined constraint behaviour for the 6 coding tools which are governed by the 6 additional GCI flags introduced in VVC version 2.

- Some packages have minor issues (i.e., file names do not match package name, incorrect format of MD5, etc.). Plan is to fix them when re-generating the streams following resolution of VTM15.0 error.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

# Contributions

Two input contributions:

JVET-Y0127 AHG5: Editors' update on conformance testing for VVC operation range extensions [D. Rusanovskyy, T. Hashimoto, H.-J. Jhu, I. Moccagatta, M. G. Sarwer, Y. Yu]

JVET-Y0190 AHG2/AHG8: Suggestions for the operation range extensions GCI [J. Gan, Y. Yu, H. Yu]

# Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

# Recommendations

The AHG recommends the following:

- Enable VVC operation range extensions streams decoding by VTM15.0

- Solicit and collect missing VVC operation range extensions streams bitstream descriptions

- Solicit and identify volunteers to cross-check 20 bitstreams for the remaining 4 VVC  operation range extensions categories

- Review the list of conformance bitstreams for VVC operation range extensions and the conformance specification for VVC operation range extensions Draft 2