| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabricel@xiaomi.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 24rd and 25th JVET meetings.

# Introduction

The mandates for the AHG are as follows.

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-3.0 software version and the reference configuration encodings according to JVET-X2017 common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

The following adopted aspects were integrated into ECM-3.0:

JVET-X0083 (test 3.3a): AMVP-merge mode

JVET-X0049 (test 3.4a): Lossless software optimization for BDMVR

JVET-X0071 (EE combination 4.1/4.3a/4.4): Chroma bilateral filter, longer filter for CCALF, and alternative band classifier for ALF

JVET-X0056 test 1: Early termination for DMVR and TM

JVET-X0141: CIIP with TIMD and TM merge

JVET-X0090 solution 3: Combination of CIIP, OBMC and LMCS

JVET-X0124: Cleanup signalling of intra template matching

JVET-X0148 (only the TIMD pipeline modification): PDPC handling for TIMD

JVET-X0149: LUT-based derivation of DIMD and TIMD

JVET-X0144 trade-off 2: Max MTT hierarchy depth set by temporal ID

VET-X0121: Fixes for enabling RPR

JVET-X0122 / merge request #16: Fix the shift number of TIMD absInvAngle

JVET-X0156: Fix for histogram of gradients derivation in DIMD mode

Software improvements and bug fixes:

Fix SSE for CTU 256 (MR 18)

Fix issue #1: compiling error when BASE_ENCODER=1, BASE_NORMATIVE=0, and TOOLS=0 (MR 33)

Fix mismatching when TM_MRG=1 with BSE_NORMATIVE=0 and TOOLS=0, and fix compilation error when MULTI_PASS_DMVR=0 (MR 25)

Fix intra cost overflow issue (solves the mismatch with different compilers) (MR 31 + MR 50)

Fix CABAC initial values for CcSaoControlIdc and MultiHypothesisFlag (MR 26)

Fix intra span issue (MR 45)

Fix ALF encoder bug for HDR PQ optimization (MR 46)

Some other lossless code cleanup

The following fixes were integrated on top of ECM-3.0 and released as ECM-3.1:

Fix for LFNST table size (solves the mismatch with different compilers) (MR 52)

ECM-3.0 was tagged on November 15, 2021 and ECM-3.1was tagged on November 21, 2021.

## CTC Performance

In this section, ECM-3.0 and ECM-3.1 test results following ECM CTC configuration descried in JVET-X2017 are summarized.

The below tables show ECM-3.0 performance over ECM-2.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.75% | -4.82% | 102% | 103% |
| Class A2 | 0.00% | -3.85% | -5.60% | 102% | 105% |
| Class B | -0.14% | -4.23% | -3.41% | 101% | 105% |
| Class C | -0.35% | -1.58% | -1.64% | 98% | 106% |
| Class E | -0.37% | -2.03% | -3.62% | 98% | 105% |
| Overall | -0.22% | -2.80% | -3.65% | 100% | 105% |
| Class D | -0.16% | -1.51% | -1.71% | 96% | 106% |
| Class F | -0.33% | -2.64% | -1.84% | 100% | 105% |
| Class TGM | -0.15% | -1.49% | -1.13% | 101% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.65% | -0.71% | -2.12% | 108% | 92% |
| Class A2 | -1.71% | -3.84% | -4.81% | 112% | 86% |
| Class B | -1.23% | -3.09% | -2.32% | 109% | 90% |
| Class C | -1.13% | -1.05% | -0.67% | 113% | 90% |
| Class E |  |  |  |  |  |
| Overall | -1.38% | -2.22% | -2.34% | 111% | 90% |
| Class D | -0.88% | -1.65% | -1.48% | 110% | 89% |
| Class F | -0.39% | -1.54% | -1.03% | 110% | 94% |
| Class TGM | -0.28% | -1.19% | -0.97% | 109% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.17% | -5.17% | -4.97% | 102% | 102% |
| Class C | -0.80% | -3.35% | -2.86% | 102% | 105% |
| Class E | -1.74% | -4.05% | -5.82% | 116% | 101% |
| Overall | -1.60% | -4.28% | -4.48% | 106% | 103% |
| Class D | -0.15% | -3.39% | -2.95% | 99% | 107% |
| Class F | -0.56% | -2.63% | -2.22% | 104% | 106% |
| Class TGM | -0.35% | -1.72% | -1.30% | 107% | 106% |

The next tables show ECM-3.1 performance over ECM-2.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.75% | -4.82% | 102% | 103% |
| Class A2 | 0.00% | -3.85% | -5.60% | 101% | 103% |
| Class B | -0.14% | -4.23% | -3.41% | 100% | 104% |
| Class C | -0.35% | -1.58% | -1.64% | 98% | 104% |
| Class E | -0.37% | -2.03% | -3.62% | 98% | 103% |
| Overall | -0.22% | -2.80% | -3.65% | 100% | 103% |
| Class D | -0.16% | -1.51% | -1.71% | 95% | 104% |
| Class F | -0.33% | -2.64% | -1.84% | 100% | 104% |
| Class TGM | -0.15% | -1.49% | -1.13% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.65% | -0.71% | -2.11% | 107% | 93% |
| Class A2 | -1.70% | -3.79% | -4.82% | 113% | 88% |
| Class B | -1.23% | -3.10% | -2.36% | 108% | 90% |
| Class C | -1.13% | -1.05% | -0.67% | 110% | 88% |
| Class E |  |  |  |  |  |
| Overall | -1.38% | -2.21% | -2.35% | 109% | 89% |
| Class D | -0.88% | -1.65% | -1.48% | 106% | 87% |
| Class F | -0.39% | -1.54% | -1.03% | 108% | 93% |
| Class TGM | -0.28% | -1.19% | -0.97% | 107% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.17% | -5.17% | -4.97% | 100% | 102% |
| Class C | -0.80% | -3.35% | -2.86% | 97% | 103% |
| Class E | -1.74% | -4.05% | -5.82% | 112% | 100% |
| Overall | -1.60% | -4.28% | -4.48% | 102% | 102% |
| Class D | -0.15% | -3.39% | -2.95% | 95% | 100% |
| Class F | -0.56% | -2.63% | -2.22% | 101% | 104% |
| Class TGM | -0.35% | -1.72% | -1.30% | 103% | 105% |

Next tables show ECM-3.1 performance over VTM-11.0 + JVET-V0056 (https://vcgit.hhi.fraunhofer.de/chollmann/VVCSoftware_VTM/-/tree/MCTF_VTM11) anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.98% | -12.44% | -16.63% | 311% | 241% |
| Class A2 | -6.44% | -13.13% | -11.99% | 298% | 232% |
| Class B | -6.05% | -13.75% | -14.21% | 338% | 257% |
| Class C | -7.06% | -10.21% | -10.67% | 322% | 252% |
| Class E | -7.58% | -11.55% | -12.41% | 322% | 294% |
| Overall | -6.75% | -12.28% | -13.16% | 320% | 255% |
| Class D | -5.86% | -8.45% | -8.23% | 316% | 265% |
| Class F | -10.80% | -15.69% | -15.66% | 244% | 298% |
| Class TGM | -15.62% | -18.73% | -18.25% | 232% | 293% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.95% | -16.42% | -21.85% | 367% | 466% |
| Class A2 | -15.84% | -20.38% | -20.42% | 364% | 511% |
| Class B | -13.57% | -19.94% | -19.32% | 384% | 493% |
| Class C | -15.35% | -17.27% | -16.98% | 387% | 427% |
| Class E |  |  |  |  |  |
| Overall | -14.77% | -18.61% | -19.42% | 377% | 473% |
| Class D | -16.10% | -17.65% | -16.99% | 378% | 459% |
| Class F | -13.54% | -18.02% | -17.76% | 345% | 407% |
| Class TGM | -14.65% | -18.96% | -19.20% | 348% | 304% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.28% | -24.89% | -24.11% | 302% | 331% |
| Class C | -12.51% | -18.81% | -18.94% | 315% | 294% |
| Class E | -12.11% | -17.24% | -19.15% | 298% | 296% |
| Overall | -12.31% | -20.95% | -21.14% | 305% | 310% |
| Class D | -14.09% | -20.17% | -19.36% | 303% | 296% |
| Class F | -12.15% | -18.99% | -18.67% | 295% | 324% |
| Class TGM | -14.02% | -21.79% | -22.13% | 269% | 255% |

# AHG related contributions

JVET-Y0102: On the balance of ECM coding gains between luma and chroma

JVET-Y0113: Adjusting luma/chroma BD-rate balance in ECM

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.