| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0024-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Ling Li Karam Naser Jacob Ström Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com aurali@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as partitioning, inter prediction, and in-loop filtering tests.

The software basis for this EE is ECM-3.1, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-3.1. ECM-3.1 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-x-ee2/ECM/-/branches.

Test and cross-check results can be found in the input JVET contributions and are located at https://vcgit.hhi.fraunhofer.de/ecm/jvet-x-ee2/simulation-results/.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1a | ABT | ByteDance Kai Zhang InterDigital Fabrice Urban JVET-Y0150 | Xiaomi Fabrice Le Léannec JVET-Y0180 |
| 1.1b | UQT | ByteDance Kai Zhang InterDigital Fabrice Urban JVET-Y0150 | Xiaomi Fabrice Le Léannec JVET-Y0180 |
| 1.1c | ABT + UQT | ByteDance Kai Zhang InterDigital Fabrice Urban JVET-Y0150 | Huawei Johannes Sauer |
| 2 Intra prediction |  |  |  |
| 2.1a | Extended MRL candidate list | Qualcomm Keming Cao JVET-Y0116 | OPPO Kazushi Sato JVET-Y0171 Ofinno Vasily Rufitskiy JVET-Y0204 |
| 2.1b 2.1c | Extended MRL with the reduced number candidates | Qualcomm Keming Cao JVET-Y0116 | Ofinno Vasily Rufitskiy JVET-Y0204 |
| 3 Inter prediction |  |  |  |
| 3.1a | GPM with intra and inter prediction | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng JVET-Y0065 | OPPO Kazushi Sato JVET-Y0182 Alibaba Ru-Ling Liao |
| 3.1b | 3.1a + DIMD and neighboring mode based IPM derivation + combination of GPM-intra and GPM-MMVD | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng JVET-Y0065 | Alibaba Ru-Ling Liao |
| 3.1c | 3.1b + TIMD-based IPM derivation | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng JVET-Y0065 | Alibaba Ru-Ling Liao |
| 3.1d | 3.1c + combination of GPM-intra and GPM-TM | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng JVET-Y0065 | Alibaba Ru-Ling Liao |
| 3.2 | Modified pairwise candidates | Canon Guillaume Laroche JVET-Y0100 | Bytedance Lei Zhao |
| 3.3a | TMVP improvement method 1 | Alibaba Ru-Ling Liao JVET-Y0093 | OPPO Zhihuang Xie JVET-Y0183 |
| 3.3b | TMVP improvement method 1 + method 2 | Alibaba Ru-Ling Liao JVET-Y0093 | OPPO Zhihuang Xie JVET-Y0183 |
| 3.4 | MV candidate type-based ARMC | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao JVET-Y0134 | Kwai Xiaoyu Xiu |
| 3.5a | Template matching based merge candidate list construction | Bytedance Lei Zhao JVET-Y0134 | Canon Guillaume Laroche |
| 3.5b | Template matching based merge candidate list construction (Test 3.5a) + Modified pairwise candidates (Test 3.2) | Bytedance Lei Zhao Canon Guillaume Laroche JVET-Y0134 |  |
| 3.6a | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao JVET-Y0134 | Kwai Xiaoyu Xiu |
| 3.6b | MV candidate type-based ARMC (Test 3.4) + modified pairwise candidates (Test 3.2) | Qualcomm Yao-Jen Chang Canon Guillaume Laroche | withdrawn |
| 3.6c | MV candidate type-based ARMC (Test 3.4) + MV candidates from Test 3.5 | Qualcomm Yao-Jen Chang Bytedance Lei Zhao | withdrawn |
| 3.6d | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 + Modified pairwise candidates from Test 3.2 + MV candidates from Test 3.5 Test 3.2 part is withdrawn from the combination | Qualcomm Yao-Jen Chang Canon Guillaume Laroche Bytedance Lei Zhao Alibaba Ru-Ling Liao JVET-Y0134 | InterDigital Fabrice Urban |
| 3.7 | Increased number of TM merge candidates | Qualcomm Yao-Jen Chang JVET-Y0132 | Alibaba Ru-Ling Liao |
| 3.8 | Alternative template matching | Qualcomm Han Huang | withdrawn |
| 3.9a | TM based reordering for MMVD | Bytedance Mehdi Salehifar JVET-Y0067 |  |
| 3.9b | TM based reordering for MMVD and affine MMVD | Bytedance Mehdi Salehifar JVET-Y0067 | Kwai Xiaoyu Xiu |
| 3.9c | TM based reordering for MMVD (Test 3.9a) + MVD sign prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang JVET-Y0067 |  |
| 3.9d | TM based reordering for MMVD and affine MMVD (Test 3.9b) + MVD sign prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang JVET-Y0067 | Kwai Xiaoyu Xiu |
| 3.10 | MVD sign prediction | Qualcomm Yan Zhang JVET-Y0067 | Kwai Han Gao Ofinno Vasily Rufitskiy |
| 3.11 | Non-adjacent spatial neighbor for affine merge mode | Kwai Wei Chen JVET-Y0153 | Qualcomm Yan Zhang Bytedance Kai Zhang JVET-Y0207 |
| 3.12a | History-parameter-based affine model inheritance | Bytedance Kai Zhang JVET-Y0145 | Alibaba Jie Chen |
| 3.12b | History-parameter-based affine model inheritance (Test 3.12a) + non-adjacent spatial neighbors for affine merge mode (Test 3.11) | Bytedance Kai Zhang Kwai Wei Chen JVET-Y0146 | Alibaba Jie Chen |
| 3.12c | 3.12b with TM disabled | Bytedance Kai Zhang Kwai Wei Chen JVET-Y0146 |  |
| 3.13 | Modifications of IBC merge/AMVP list construction | Bytedance Na Zhang JVET-Y0058 | Ofinno Damian Ruiz Coll JVET-Y0206 |
| 4 Transform and coefficients coding |  |  |  |
| 4.1a | QIdx based signs selection | Alibaba Jie Chen JVET-Y0094 | OPPO Luhang Xu JVET-Y0185 |
| 4.1b | Extension of sign prediction area | Alibaba Jie Chen | withdrawn |
| 4.1c | Qidx based signs selection + extension of sign prediction area | Alibaba Jie Chen JVET-Y0094 | OPPO Luhang Xu JVET-Y0185 |
| 4.2a | Sign prediction with adaptive coefficient selection | Kwai Xiaoyu Xiu JVET-Y0137 | OPPO Luhang Xu JVET-Y0184 |
| 4.2b | Sign prediction with adaptive coefficient selection + extension of sign prediction to LFNST | Kwai Xiaoyu Xiu JVET-Y0137 | OPPO Luhang Xu JVET-Y0184 |
| 4.3a | Test 4.1c + extension of sign prediction to LFNST (Test 4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu JVET-Y0138 | Bytedance Yang Wang |
| 4.3b | Extension of sign prediction area (4.1b) + Sign prediction with adaptive coefficient selection with extension of sign prediction to LFNST (4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu JVET-Y0138 | Tencent L.-F. Chen |
| 4.4a | Adaptive intra MTS | Qualcomm Bappaditya Ray JVET-Y0142 | Sharp Tomonori Hashimoto |
| 4.4b | Adaptive intra MTS – encoder only (non-normative) | Qualcomm Bappaditya Ray JVET-Y0142 | Sharp Tomonori Hashimoto |
| 4.4c | Adaptive intra MTS – with parameter signalling | Qualcomm Bappaditya Ray JVET-Y0142 | Sharp Tomonori Hashimoto |
| 4.4d | Intra MTS without adaptivity (fixed number of candidates) | Qualcomm Bappaditya Ray JVET-Y0142 | Sharp Tomonori Hashimoto |
| 5 In-loop filtering |  |  |  |
| 5.1 | Edge-based classifier for CCSAO | Qualcomm Anand Meher Kotra Kwai Che-Wei Kuo JVET-Y0106 | Alibaba Xinwei Li JVET-Y0196 |
| 5.2a | Adaptive filter shape selection for ALF | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer JVET-Y0147 | Ericsson Jacob Ström |
| 5.2b | Adaptive filter shape selection for ALF with reduced filter length | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer JVET-Y0147 | Ericsson Jacob Ström |

# Description of tests

## Partitioning

### Test 1.1a: Asymmetric binary tree (ABT)

Four new asymmetric binary tree splitting modes are added to the multi-type tree structure of VVC shown on Figure 1.

| ABT-H1 | ABT-H2 |
| --- | --- |
| ABT-V1 | ABT-V2 |

Figure 1. ABT partitions

CU can be divided into 2 sub-CUs with 1/4 and 3/4 sizes. 6-point, 12-point, 24-point, and 48-point transforms/inverse-transforms are introduced. Intra prediction is also modified to accommodate non-power-2 block sizes.

### Test 1.1b: Unsymmetric quad tree (UQT)

Four types of unsymmetric quad tree partitions with 1/8, 1/2, 1/4, and 1/8 sizes as shown on Figure 2 are tested.

| UQT-H1 | UQT-H2 |
| --- | --- |
| UQT-V1 | UQT-V2 |

Figure 2. UQT partitions

### Test 1.1c: Asymmetric binary tree (ABT) and Unsymmetric quad tree (UQT)

This test is a combination of the tests 1.1a and 1.1b.

## Intra prediction

### Test 2.1: Extended MRL candidate list

MRL candidate list is extended from lines {1, 2} to {1, 3, 5, 7, 12} in test 2.1a, to {1, 3, 5, 7} in test 2.1b or {1, 3, 5} in test 2.1c. For template-based intra mode derivation (TIMD), lines {1, 3} are used.

Figure 3. Extended MRL candidate list

Table 1 Usage frequency of each line in MRL candidate list {1, 3, 5, 7, 12}

|  | line 1 | line 3 | line 5 | line 7 | line 12 |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 36% | 15% | 15% | 13% | 21% |
| Class A2 | 54% | 19% | 11% | 7% | 9% |
| Class B | 43% | 21% | 16% | 10% | 11% |
| Class C | 53% | 19% | 11% | 8% | 9% |
| Class E | 46% | 21% | 12% | 8% | 12% |
| Overall | 46% | 19% | 14% | 9% | 12% |
| Class D | 45% | 19% | 13% | 9% | 13% |
| Class F | 57% | 18% | 10% | 7% | 9% |
| Class TGM | 38% | 17% | 20% | 10% | 15% |

## Inter prediction

### Test 3.1: GPM with intra and inter prediction

In the tests, the prediction is generated by weighting inter predicted samples and intra predicted samples for each GPM-separated region. Inter part is derived by using existed prediction in GPM, while intra part is derived by intra prediction mode indicated by an index into the intra modes candidate list.

The intra candidate list has size of 3, which has the parallel to GPM edge angular mode, the perpendicular angular mode, and planar mode as shown Figure 4.

Figure 4 GPM with intra and inter prediction.

This base mode is tested in Test 3.1a.

In the Test 3.1b, the following two modifications are introduced on top of Test 3.1a:

- DIMD and neighboring mode based IPM derivation are added after parallel mode in the intra mode list. Neighbor intra modes are derived from at most 5 locations but they are restricted based on the GPM angle as detailed in Table 2

- Combination of GPM-intra and GPM-MMVD

Table 2 : The positions of available neighboring blocks for intra candidate derivation based on the angle of GPM block boundary. A and L denotes the above and left side of the prediction block.

| Angle of GPM | 0 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |
| Partition angle | 16 | 18 | 19 | 20 | 21 | 24 | 27 | 28 | 29 | 30 |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |

In the Test 3.1c, on top of Test 3.1b template-based intra mode derivation (TIMD) is additionally utilized for intra mode candidates, the order of the modes is parallel mode, TIMD candidates, DIMD candidates, and neighboring intra modes. The list size is kept as 3.

In the Test 3.1d, template matching for GPM is added on top of Test 3.1c.

### Test 3.2: Modified pairwise candidates

In the test, the pairwise candidate is removed from the regular merge and BM merge candidate derivation. It is added, when it is not a duplicate, during the adaptive reordering of merge candidates with template matching (ARMC-TM) as depicted in Figure 5. The pairwise is built with the two first reordered candidates.

In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate if the created pairwise is not a duplicate.

Figure 5. Pairwise in the merge candidate reordering ARMC-TM and additional pairwise candidates after reordering.

### Test 3.3: TMVP improvement

In Test 3.3a, TMVP candidate is conditionally added to the list according to the motion information of spatial neighboring blocks, which are bottom left, above right, and above left.

TMVP is derived from the bottom right position, if it is inter coded and if none of the spatial neighboring blocks has similar motion or, at least one of the spatial neighboring blocks has similar motion and the spatial neighboring blocks use TMVP.

If TMVP is not derived, the same derivation process is repeated for the center position.

In checking the similarity of the temporal motion, if the inter prediction direction of the two temporal motion are the same and the motion vector differences in horizontal and vertical direction are both smaller than 5, the two temporal motion are treated as similar to each other.

The target reference picture of the TMVP candidate can be selected from any one of reference picture in the list according to scaling factor. The scaling factor of each reference picture in the reference picture list is calculated, and the one whose scaling factor is the closest to 1 is used as the target reference picture of the TMVP.

Test 3.3b is implemented on top of Test 3.3a. When a CU is coded using template matching, AMVP-merge and regular merge modes and the current picture is a non-low delay picture, the TMVP candidate is refined using the template matching method. The template is used to decide whether the TMVP is L0-predicted, L1-predicted or bi-predicted, and is used to decide which one of the three scaling factors {tbtd, tbtd×18, tbtd×98} is used to perform temporal motion vector scaling. The tb is defined to be the POC difference between the reference picture of the current picture and the current picture and td is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture.

### Tests 3.4: MV candidate type-based ARMC

In this test, merge candidates of one single candidate type, e.g., TMVP or non-adjacent MVP (NA-MVP), are reordered based on the ARMC TM cost values. The reordered candidates are then added into the merge candidate list. The TMVP candidate type adds more TMVP candidates with more temporal positions and different inter prediction directions to perform the reordering and the selection. Moreover, NA-MVP candidate type is further extended with more spatially non-adjacent positions.

### Test 3.5: Template matching based merge candidate list construction

In the test, NA-MVP, HMVP, and PAMVP candidates are put into the merge candidate list in an ascending order of template matching costs. Moreover, non-adjacent MVPs is further extended with more spatial and non-adjacent temporal positions.

### Test 3.6: Combination of the ARMC related methods

The combination test results of Test 3.2, Test 3.3a, Test 3.4 and Test 3.5a are as follows:

Test 3.5b (Combination of Test 3.5a and Test 3.2): On top of 3.5a software, the PAMVP is removed from the merge list and derived during the reordering process of merge candidate list (ARMC). In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate.

Test 3.6a (Combination of Test 3.4 and Test 3.3a): On top of 3.4 software, it adds the following method from Test 3.3a:

- The reference picture of the TMVP candidate is selected from any one of reference pictures in the reference picture list according to a pre-defined rule. The selected reference picture is the one whose scaling factor is the closest to 1.

Test 3.6d (Test 3.3a + Test 3.4 + Test 3.5a): On top of 3.5a software, it adds the following methods from Test 3.4 and Test 3.3a:

- Test 3.4: TMVP candidates reordering based on TM cost values.

- Test 3.3a: The reference picture of the TMVP candidate is selected from any one of reference pictures in the reference picture list according to a pre-defined rule. The selected reference picture is the one whose scaling factor is the closest to 1.

### Test 3.7: Increased number of TM merge candidates

The number of TM merge candidates is increased from 4 to 7 by modifying the parameter TM_MRG_MAX_NUM_CANDS in ECM-3.1 software.

### Test 3.9: MMVD extension based on template matching cost

In Test 3.9a, MMVD offsets are extended for MMVD and affine MMVD modes. Additional refinement positions along k×π/8 diagonal angles are added shown on Figure 6, thus increasing the number of directions from 4 to 16. Second, based on the SAD cost between the template (one row above and one column left to the current block) and its reference for each refinement position, all the possible MMVD refinement positions (16×6) for each base candidate are reordered. Finally, the top 1/8 refinement positions with the smallest template SAD costs are kept as available positions, consequently for MMVD index coding. The MMVD index is binarized by the rice code with the parameter equal to 2.

Figure 6. Additional directions along k×π/8 diagonal angles (red positions are used in the anchor).

In Test 3.9b, on top of the 3.9a, affine MMVD reordering is extended, in which additional refinement positions along k×π/4 diagonal angles are added. After reordering top 1/2 refinement positions with the smallest template SAD costs are kept.

In Test 3.9c (Test 3.9a + Test 3.10), where MMVD MVD sign prediction part of EE2 3.10 is replaced with MMVD extension of Test 3.9a.

In Test 3.9d (Test 3.9b + Test 3.10), where MMVD and affine MVD sign prediction parts of EE2 3.10 are replaced with MMVD and affine MMVD extensions of Test 3.9a.

### Test 3.10: MVD sign prediction

In this method, possible MVD sign combinations are sorted according to the template matching cost and index corresponding to the true MVD sign is derived and context coded. At decoder side, the MVD signs are derived as following:

- Parse the magnitude of MVD components

- Parse context-coded MVD sign prediction index

- Build MV candidates by creating combination between possible signs and absolute MVD value and add it to the MV predictor

- Derive MVD sign prediction cost for each derived MV based on template matching cost and sort

- Use MVD sign prediction index to pick the true MVD sign

MVD sign prediction is applied to inter AMVP, affine AMVP, MMVD and affine MMVD modes.

### Test 3.11: Non-adjacent spatial neighbour for affine merge mode

Non-adjacent spatial neighbor positions used in the regular merge mode are added for affine merge and affine AMVP modes.

The motion information of the non-adjacent spatial neighbors is utilized to generate additional inherited and constructed affine merge candidates.

For inherited candidates, the same derivation process of the inherited affine merge candidates in the VVC is kept unchanged except that the CPMVs are inherited from non-adjacent spatial neighbors. The non-adjacent spatial neighbors are checked based on their distances to the current block, i.e., from near to far. At a specific distance, only the first available neighbor (that is coded with the affine mode) from each side (e.g., the left and above) of the current block is included for inherited candidate derivation. As indicated by the red dash arrows in Figure 7 (a), the checking orders of the neighbors on the left and above sides are bottom-to-up and right-to-left, respectively.

Figure 7. Spatial neighbors for deriving affine merge candidates: (a) for deriving inherited affine merge candidates (b) for deriving constructed affine merge candidates

For constructed candidates, as shown in the Figure 7 (b), the positions of one left and above non-adjacent spatial neighbors are firstly determined independently. After that, the location of the top-left neighbor can be determined accordingly which can enclose a rectangular virtual block together with the left and above non-adjacent neighbors. Then, as shown in the Figure 7, the motion information of the three non-adjacent neighbors is used to form the CPMVs at the top-left (A), top-right (B) and bottom-left (C) of the virtual block, which is finally projected to the current CU to generate the corresponding constructed candidates.

Figure 8. From non-adjacent neighbors to constructed affine merge candidates

One second type of constructed affine candidates that are derived from non-adjacent neighbors are introduced for affine merge mode. Those new constructed affine candidates are derived based on the same affine candidate construction scheme as in Test 3.12. However, instead of using history-based look-up table, the non-translational affine parameters are inherited from the non-adjacent spatial neighbors. Specifically, the second type of affine constructed candidates are generated from the combination of 1) the translational affine parameters of adjacent neighboring 4x4 blocks; and 2) the non-translational affine parameters {a, b, c, d} inherited from the non-adjacent spatial neighbors as defined in Figure 7 (a).

Due to the inclusion of the additional non-adjacent candidates, the size of the affine merge candidate list is increased from 5 to 15. The subgroup size of the affine merge mode for the adaptive reordering method is increased from 3 to 15.

### Test 3.12: History-parameter-based affine model inheritance

In the test, affine model can be inherited from a previously affine-coded block which may not be neighboring to the current block. A history-parameter table is established, each entry stores a set of affine parameters: a, b, c and d, each of which is represented by a 16-bit signed integer. Entries are categorized by reference list and reference index, and five reference indices are supported for each reference list in the history table.

A history-affine-parameter-based candidate is derived from a neighbouring 4×4 block denoted as A0, A1, B0, B1 or B2 in Figure 8 and a set of affine parameters stored in a corresponding entry in the history table. The MV of a neighbouring 4×4 block served as the base MV. The MV of the current block at position (x, y) is calculated as:

,

where (mvhbase, mvvbase) represents the MV of the neighbouring 4×4 block, (xbase, ybase) represents the center position of the neighbouring 4×4 block. (x, y) can be the top-left, top-right and bottom-left corner of the current block to obtain the corner-position MVs (CPMVs) for the current block, or it can be the center of the current block to obtain a regular MV for the current block.

Figure 9 shows an example of history based affine candidate derivation from block A0. The affine parameters {a0, b0, c0, d0} are directly fetched from one entry of category HPTIdx(RefListA0, refIdx0A0) in the history table. The affine parameters from the entry are used to derive CPMV with the center position of A0 as the base position and the MV of block A0 as the base MV.

Such candidates are added into the sub-block-based merge candidate list and affine AMVP candidate. The size of sub-block-based merge candidate list is increased from 5 to 10 in RA and 12 in LB, respectively.

Besides, such candidates are used to derive MVs located at the center of the current block, as regular merge candidates. The size of regular merge candidate list is increased from 10 to 11 for random access configurations to accommodate the newly added regular merge candidates.

Figure 9. Affine candidate derived from the history table

In Test 3.12b, non-adjacent affine candidates (Test 3.11) and history-based affine candidates (Test 3.12a) are all put into the subblock-based merge candidate list.

### Test 3.13: Modifications of IBC merge/AMVP list construction

The IBC merge/AMVP list construction is modified as follows:

- Only if an IBC merge/AMVP candidate is valid, it can be inserted into the IBC merge/AMVP candidate list.

- Above-right, bottom-left, and above-left spatial candidates and one pairwise average candidate can be added into the IBC merge/AMVP candidate list.

- Template based adaptive reordering (ARMC-TM) is applied to IBC merge list

## Transform and coefficients coding

### Test 4.1: Sign prediction

In test 4.1.a, instead of always selecting the first K transform coefficients in raster-scan order of a top-left 4x4 area for sign prediction, the transform coefficients with the largest K qIdx value of the top-left 4x4 area were selected. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level. qIdx is derived as follows

qIdx = (abs(level) << 1) - (state & 1);

where level is the transform coefficient level parsed from the bitstream and state is a variable maintained by the encoder and decoder in DQ.

On top of test 4.1.a, the sign prediction area was extended to maximum 32x32 in test 4.1.c. In this test, signs of top-left MxN block are predicted. The value of M and N is computed as follows:

  - M=minw, 32

  - N=min⁡(h, 32)

where, w and h are the width and height of the transform block.

The maximum number of predicted signs is kept unchanged.

### Test 4.2: Enhanced sign prediction

Two enhancements to sign prediction are tested:

- Instead of selecting the first M coefficients in raster-scan order, the non-zero transform coefficients are selected based on their impacts on the quality of the reconstructed border samples of one TB, by maximizing L1-norm cost of the variations caused by one transform coefficient on the reconstructed border samples:

cost=Ci,j∙k=0N-1Ti,jk

where Ci,j represents the transform coefficient at coordinate i,j in the TB; Ti,j represents the template that corresponds to the transform coefficient, which consists of N reconstructed samples at the top and left border of the TB.

- Sign prediction is extended to LFNST, and only up to 4 coefficients in the top-left CG are allowed to be predicted for one LFNST TB.

### Test 4.3: Combination of sign prediction methods

Test 4.3.a is on top of test 4.1.c and additionally applies sign prediction to LFNST blocks as follows:

- The transform coefficients with the largest K qIdx value are selected for sign prediction. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level.

- The sign prediction area is extended. The signs of top-left MxN block are predicted. The value of M and N is computed as M=minw, 32, N=min⁡(h, 32), where w and h are the width and height of the transform block

- The sign prediction is also applied to LFNST block. And for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

Test 4.3.b is on top of test 4.2.b and additionally extends the sign prediction area to maximum 32x32:

- Adaptively select the signs to be predicted based on their impacts on the reconstructed border samples of one TB.

- The sign prediction is applied to LFNST blocks and for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

- The maximum area for sign prediction is extended. Encoder selects the maximum area size from four allowed values 4, 8, 16 and 32 based on configuration, sequence class and QP, and signaled it in SPS.

### Test 4.4: Adaptive intra MTS

Number of intra MTS candidates are adaptively selected (between 1, 4 and 6 MTS candidates) depending on the sum of absolute value of transform coefficients. The sum is compared against the two fixed thresholds to determine the total number of allowed MTS candidates:

1 candidate: sum <= th0

4 candidates: th0 < sum <= th1

6 candidates: sum > th1

The existed the non-DCT2 transform kernels (DST7, DCT8, DCT5, DST4, DST1, and identity transform) are used to derive additional transform pairs.

The encoder speedup is done as follows: for a given intra mode, the corresponding sum of absolute coefficients from the DCT2 pass (sumDCT2) is stored. For the MTS pass, the number of MTS candidates for RD cost evaluation is determined as follows:

1 candidate: sumDCT2 <= th0

4 candidates: th0 < sumDCT2 <= th1

6 candidates: sumDCT2 > th1

Additionally, for inter-picture, if the SATD cost, i.e., difference between the original and predicted signals, of all intra modes are higher than a scaled version of the best inter mode SATD cost then MTS RD cost evaluation is skipped.

The following tests have been carried:

- Test 4.4a: adaptive intra MTS.

- Test 4.4b: only encoder modification.

- Test 4.4c: adaptive intra MTS with thresholds signaled in SPS.

- Test 4.4d: fixed 6 MTS candidates (without adaptivity).

## In-loop filtering

### Test 5.1: Edge-based classifier for CCSAO

Currently, CCSAO uses only band classifier. In this test, edge-based classifier is added to CCSAO.

Similar to SAO, edge-based classifier uses the four 1-D directional patterns for sample classification: horizontal, vertical, 135° diagonal and 45° diagonal, as shown in the figure below, where each sample is classified based on the sample difference between the luma sample value labeled as “c” and its two neighbor luma samples labeled as “a” and “b” along the selected 1-D pattern.

Figure 10. Four 1-D directional patterns for CCSAO EO sample classification: horizontal (EO class = 0), vertical (EO class =1), 135° diagonal and 45° diagonal

Both the sample differences “a-c” and “b-c” are compared against a pre-defined threshold value (Th) to derive the final “class_idx” information. The encoder selects the best “Th” value from an array of pre-defined threshold values based on RDO and the index into the “Th” array is signalled.

The Edge-based classifier process is formulated as follows:

Ea=(a-c<0)? (a-c<(-Th)? 0:1) : (a-c<(Th)? 2:3)

Eb=(b-c<0)? (b-c<(-Th)? 0:1) : (b-c<(Th)? 2:3)

class_idx = iB * 16 + Ea * 4 + Eb

C'rec=Clip1Crec+σCCSAOclass_idx

variable “iB” in equation (3) is derived as follows.

iB=cur ∙Ncur≫BD  (or) iB=col1 ∙Ncol1≫BD (or) iB=col2 ∙Ncol2≫BD,

wherein, sample “cur” is the current sample being processed, col1 and col2 are the collocated samples. When luma samples are processed, col1 and col2 are the collocated Cb and Cr samples, respectively. When chroma Cb samples are processed, col1 and col2 are the collocated Y and Cr samples, respectively. When chroma Cr samples are processed, col1 and col2 are the collocated Y and Cb samples, respectively.

Based on RDO, encoder signals one among the samples cur, col1, or col2 used in deriving the band information.

### Test 5.2: Adaptive filter shape selection for ALF

In the test, two candidate filter shapes: a diamond shape candidate as shown in Figure 11(a), which is currently applied in ECM, and a cross shape candidate as shown in Figure 11(b)/(c), which is newly introduced, can be adaptively selected by the luma filters in ALF. The number of coefficients that need to be trained and signaled for a luma filter is 20 with all the filter shapes.

In Test 5.2a, filter shape selection is performed between shapes (a) and (b), while in Test 5.2b shapes (a) and (c) are used as the shape candidates. The only difference between the two shapes (b) and (c) is the maximum filter length.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 11. Adaptive ALF shapes.

In each APS, a shape index for the online-trained luma filters is signaled to decoder. Each APS contains the luma filters that are associated with the filter shape index.

For each CTB, an APS index is signaled to indicate which luma filter shape is used to filter the current CTB. When filtering a luma sample, the coefficients and clip indices are also rearranged according to the corresponding filter shape.

# Test results

See the accompanied Excel table.

# EE related documents

| JVET-Y0062 | Non-EE2: Cross-component palette coding |
| --- | --- |
| JVET-Y0076 | Non-EE2: Template Matching-based OBMC Design |
| JVET-Y0088 | EE2-related: IBC with Template Matching |
| JVET-Y0089 | Non-EE2: DMVR with BCW enabled |
| JVET-Y0091 | Non-EE2: MVP refinement for regular AMVP mode |
| JVET-Y0092 | Non-EE2: On chroma intra prediction mode |
| JVET-Y0095 | Non-EE2: RPR with luma-only re-scaling |
| JVET-Y0114 | Non-EE2: Dependent quantization with 4 states for chroma components |
| JVET-Y0119 | EE2-related: On Extended MRL Intra Prediction |
| JVET-Y0120 | EE2-related: Non-adjacent temporal MVP |
| JVET-Y0121 | EE2-4.2-related: On adaptive sign prediction position selection |
| JVET-Y0124 | Non-EE2: Intra Block Copy with An Extended Reference Area |
| JVET-Y0128 | Non-EE2: fixing issues for RPR enabling and non-CTC configuration in ECM |
| JVET-Y0129 | Non-EE2: MVD and merge index signaling of AMVP-merge mode |
| JVET-Y0130 | EE2-related: Unification of availability check for intra mode coding |
| JVET-Y0131 | EE2-related: Clean-up on DIMD |
| JVET-Y0133 | EE2-related: BVP candidate adjustment based on IBC reference region implemented on top of test EE2-3.13 |
| JVET-Y0135 | Non-EE2: Template matching based reordering for GPM split modes |
| JVET-Y0139 | Non-EE2: On the extended number of active reference pictures and reference picture reordering |
| JVET-Y0140 | EE2-related: On the LCU boundary processing by intra-prediction tools |
| JVET-Y0141 | EE2-4.3 related: More combined test results for sign prediction |
| JVET-Y0144 | Non-EE2: DIMD Flag Signalling Clean-up |
| JVET-Y0148 | Non-EE2: Spatial-Temporal Adaptive Loop Filter |
| JVET-Y0149 | EE2-related: Modifications of the extended MRL candidate list |
| JVET-Y0151 | Non-EE2: Adaptive re-ordering of merge candidates with refined motion |
| JVET-Y0159 | EE2-related: inter MTS refinement on adaptive intra MTS (EE2-4.4) |
| JVET-Y0160 | EE2-3.13-related: Enlarged HMVP table for IBC |
| JVET-Y0161 | EE2-3.12-related: Extensions of history-parameter-based affine model inheritance |
| JVET-Y0172 | Non-EE2: Long tap interpolation filtering on chroma components |
| JVET-Y0174 | EE2-1.1–related: additional tests on partitioning flexibility |
| JVET-Y0203 | EE2-related: a combination of the extended MRL candidate list (JVET-Y0149) modifications with the extended LCU boundary processing area (JVET-Y0140) |

# AHG12 related documents

| JVET-Y0012 | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |
| --- | --- |
| JVET-Y0055 | AHG12: Slope adjustment for CCLM |
| JVET-Y0097 | AhG12: Removed DIMD from MPM list of TIMD |
| JVET-Y0108 | AhG3/AhG12 Modification of JVET CTC for environmental considerations |
| JVET-Y0109 | AhG12: Neural Network-based intra prediction |
| JVET-Y0117 | AhG12: ECM coding performance for HDR/WCG content and suggested common test conditions |
| JVET-Y0125 | AHG12: Enhanced bi-directional motion compensation |
| JVET-Y0154 | AHG12: Bilinear Interpolation Filtering for ARMC |
| JVET-Y0157 | AHG12: Improved probability estimation for CABAC |
| JVET-Y0181 | AHG12: CABAC initialization from previous inter slice |